3GPP TSG-RAN WG2 #127bis	R2-2408614

Hefei, China, Oct 14th – 18th, 2024

Source:		NEC

Agenda item:		8.6.4	Conditional intra-CU LTM

Title:	Discussion on conditional intra-CU LTM

Document for:		Discussion and decision

- Introduction

At RAN #105 meeting, the objective of supporting conditional intra-CU LTM was confirmed for Rel-19 mobility enhancement with the following update in the WID [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this paper, we provide our initial consideration on conditional intra-CU LTM.

- Discussion

Currently for CHO, the execution condition consists of one or two Layer 3 events. The UE starts evaluating the execution condition(s) upon receiving the CHO configuration, and performs CHO execution by applying the RRCReconfiguration associated with the target candidate cell upon the execution condition is fulfilled. For CLTM, the very first question is what execution condition(s) should be used for CLTM, Layer 1 event, Layer 3 event, or both can be supported.

In our understanding, if Layer 3 event based execution condition is used for CLTM, it will be no big difference from CHO, and we will lose the latency benefit of “L1/L2 triggered” mobility. Since we are now working on L1 LTM event to trigger Layer measurement reporting, it is straightforward to reuse the L1 LTM event to trigger execution of CLTM, just like what we did for CHO, i.e., to reuse the events triggering Layer 3 measurement reporting for CHO.

Proposal 1. Use L1 LTM event as execution condition for CLTM.

Now there are four L1 LTM events agreed. Each of them corresponds to one L3 event, Event LTM2 corresponds to Event A2, Event LTM3 corresponds to Event A3, and etc.:

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

For CHO, the execution conditions can be Event A3 (CondEventA3) and Event A5 (CondEventA5). Therefore, Even LTM3 and Event LTM5 should be supported as execution conditions for CLTM. Event A4 is supported for CPA or MN-initiated CPC in DC scenario. In Rel-19, there is no sufficient motivation to support and MN-initiated intra-SCG LTM. Additionally, as per the WID objective, we limit specifying the intra-CU conditional LTM to the scenario where the UE is in non-DC, and there is a checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenario. Therefore, we don’t need to support Event LTM4 as execution conditions for CLTM for now.

Proposal 2. Support Event LTM3 and Event LTM5 as execution conditions for CLTM.

Now at most two trigger conditions as part of an execution condition can be configured for CHO, and CHO is triggered when both of the trigger conditions are fulfilled for the target cell. This makes the selection of the target cell more reliable. For CLTM, RAN2 needs further discussion whether there is a need to support more than one L1 events as execution conditions for CLTM.

Proposal 3. Discuss whether to support more than one L1 events as trigger conditions of an execution condition for CLTM.

In Rel-18, LTM can be performed by skipping the Random Access procedure, which contributes a lot to the mobility latency and interruption. And to enable this, RAN2 spent a lot of time and efforts on specifying the RACH-less LTM and early sync procedure in Re-18. Actually, the UP part of discussion is almost mainly about it. If we cannot support RACH-less for CLTM, we will loss one important function of LTM procedure and cannot obtain the extreme mobility performance which is the main motivation why we introduce CLTM. Therefore, we should support RACH-less CLTM in Rel-19.

Proposal 4. Support RACH-less CLTM in Rel-19.

In Rel-18 RACH-less LTM, the TA can be obtained by the UE by two means, i.e., provided in LTM cell switch command MAC CE, or UE-based TA measurement. For CLTM, there is no LTM cell switch command MAC CE, but the UE-based TA measurement can be reused for RACH-less CLTM directly.

Proposal 5. UE-based TA measurement can be supported for RACH-less CLTM.

In Rel-18 RACH-less LTM, the source cell can transmit PDCCH order to the UE for early UL sync of one LTM candidate cell. And the UE would transmit preamble to the candidate cell without RAR reception. DU of the source cell may receive TA command from the DU of the candidate cell via CU, and include the TA command in the LTM cell switch command MAC CE. For CLTM, the early UL sync procedure can be reused to obtain TA of the UE by the source cell. But since there is no LTM cell switch command MAC CE for CLTM, RAN2 needs to discuss whether to support early UL sync by reusing one in Rel-18. For example, how to indicate the TA of the candidate cells to the UE, and how UE maintains the TA and determines TA validity needs further discussion. It should be noted that RAN2 had decided not to provide RAR for early UL sync from source cell nor candidate cell. This decision should not be revisited just to support this functionality for CLTM.

Proposal 6. Specify details on how to indicate and maintain TA for candidate cells, and how to determine TA validity for target cell of RACH-less CLTM.

In Rel-18 RACH-less LTM, both CG-based and DG-based RACH-less LTM can be supported. In case of DG-based option, upon initiation of RACH-less LTM, the UE would monitor PDCCH for dynamic grant based on the TCI state information indicated in the LTM cell switch command. And the network determine the TCI state information based on the L1 measurement report from the UE. However, for CLTM, the network may not have the latest L1 measurement report from the UE, as the fulfilment of L1 LTM event which is set as execution condition will not trigger a measurement report. This makes that the network cannot determine the beam information for PDCCH transmission, which may result in big resource waste in case DG-based LTM, as it may needs to schedule UL grant in all beams. In case of CG-based option, since the CG resources are reserved for RACH-less LTM, it can be reused for CLTM directly.

Proposal 7. Support at least CG-based RACH-less CLTM. And FFS whether to support DG-based CLTM.

# 3. Conclusion

In this paper, we provide our initial consideration for the support of CLTM, and have the following proposals:

Proposal 1. Use L1 LTM event as execution condition for CLTM.

Proposal 2. Support Event LTM3 and Event LTM5 as execution conditions for CLTM.

Proposal 3. Discuss whether to support more than one L1 events as trigger conditions of an execution condition for CLTM.

Proposal 4. Support RACH-less CLTM in Rel-19.

Proposal 5. UE-based TA measurement can be supported for RACH-less CLTM.

Proposal 6. Specify details on how to indicate and maintain TA for candidate cells, and how to determine TA validity for target cell of RACH-less CLTM.

Proposal 7. Support at least CG-based RACH-less CLTM. And FFS whether to support DG-based CLTM.

# Reference

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4