3GPP TSG-RAN WG2 Meeting #127bis	R2-2408615

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:	8.1.3

Source:	Samsung

Title:	Discussion on NW side data collection

Document for:	Discussion & Decision

Introduction

This document discusses on NW side data collection for AI/ML air interface WI.

Discussion

Memory capability

The following agreement was made in RAN2#127, in order for UE to log training data for NW side data collection:

<Agreements (RAN2#127)> - UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

As legacy, there are existing memory requirements for UE to log measurement results. Firstly, for logged MDT, the fixed size (i.e., 64KB) of minimum memory requirement is defined. This is mandatory for UEs who support logged MDT, and no separate capability is used for this memory requirement.

<TS 38.306> loggedMeasurements-r16 Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE state. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The minimum memory size of MDT logged measurements is 64KB. For eRedCap UE supporting this feature, the minimum memory size of MDT logged measurements is 16KB.; UE; No; No; No.

Meanwhile, there are two different and separate memory requirements for QoE measurement. One is the requirement for UE to log QoE reports while QoE reporting is temporarily paused by NW. The other is for UE to log QoE reports in RRC_IDLE and RRC_INACTIVE. Besides, one separate capability is defined for QoE reports in RRC_IDLE and RRC_INACTIVE.

<TS 38.306> qoe-AdditionalMemoryMeasReport-r18 Indicates the minimum AS layer memory size the UE supports for QoE measurement in RRC_IDLE and RRC_INACTIVE in addition to the "AS layer memory size for QoE paused measurement reports". Value kB128 means the UE supports at least 128 kilobytes for this purpose, and so on. A UE supporting this feature shall also support qoe-IdleInactiveMeasReport-r18.; UE; No; No; No. <SKIP> # 6 Conditionally mandatory features without UE radio access capability parameters Features; Condition. Acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList; It is mandatory to support acquisition of positioning SI messages with 80 milliseconds offset position compared to SI messages in schedulingInfoList for UEs which support the acquisition of the posSIB types in posSchedulingInfoList as specified in TS 38.331 [9].. Acquisition of SI messages with explicit SI window positions; It is mandatory to support acquisition of SI messages with explicit SI window positions for UEs which support the SIB types in schedulingInfoList2 as specified in TS 38.331 [9].. AS layer memory size for QoE paused measurement reports; It is mandatory to support the minimum AS layer memory size of 64KB for QoE paused measurement reports for UEs which support qoe-Streaming-MeasReport-r17, qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17.. AS layer memory size for QoE measurement reports in RRC_IDLE and RRC_INACTIVE; It is mandatory to support the minimum AS layer memory size of 64KB for QoE measurement reports stored in RRC_IDLE/RRC_INACTIVE for UEs which support qoe-IdleInactiveMeasReport-r18 and any of qoe-Streaming-MeasReport-r17 or qoe-MTSI-MeasReport-r17 or qoe-VR-MeasReport-r17. This memory size is additional to "AS layer memory size for QoE paused measurement reports". <TS 38.331> #### – AppLayerMeasParameters The IE AppLayerMeasParameters is used to convey the capabilities supported by the UE for application layer measurements. AppLayerMeasParameters information element -- ASN1START -- TAG-APPLAYERMEASPARAMETERS-START AppLayerMeasParameters-r17 ::= SEQUENCE { qoe-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-MTSI-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ul-MeasurementReportAppLayer-Seg-r17 ENUMERATED {supported} OPTIONAL, ..., [[ qoe-IdleInactiveMeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-NRDC-MeasReport-r18 ENUMERATED {supported} OPTIONAL, qoe-AdditionalMemoryMeasReport-r18 ENUMERATED {kB128, kB256, kB512, kB1024} OPTIONAL, qoe-PriorityBasedDiscarding-r18 ENUMERATED {supported} OPTIONAL, srb5-r18 ENUMERATED {supported} OPTIONAL ]] } -- TAG-APPLAYERMEASPARAMETERS-STOP -- ASN1STOP

If UE supports QoE measurement for at least one service type, UE mandatorily supports minimum AS layer memory size of 64KB for QoE paused measurement reports (i.e., without separate capability signalling). Meanwhile, if UE supports QoE measurement in RRC_IDLE and RRC_INACTIVE, UE mandatorily supports another 64KB memory without a separate capability. These two 64 KB memory requirements are similar to that of logged MDT. However, in addition to two 64 KB memory requirements, a separate capability (i.e., qoe-AdditionalMemoryMeasReport) was introduced for QoE measurement in RRC_IDLE and RRC_INACTIVE. With this capability, UE can indicate more than 64 KB (i.e., 128, 256, 512, and 1024 KB). This is optional capability, so if UE does not include the capability, it means the UE only supports the mandatory 64 KB memory for QoE measurement in RRC_IDLE and RRC_INACTIVE. The motivation for this optional capability is UEs may have different memory capabilities and NW may want to select the UE who has big memory capability for more and consistent data collection.

We assume the same benefit can be expected, if we have similar capability for NW side data collection. i.e., UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability alarger minimum AS layer memory size.

Proposal 1. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size.

How to report data

From the last meetings, RAN2 agreed two different reporting methods for NW side data collection:

- Method 1. Periodic reporting

- Method 2. On-demand request based reporting

<Agreements (RAN2#126)> - Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting <Agreements (RAN2#127)> - On-demand request from the network is supported. FFS details on signalling

Method 1 works based on immediate MDT framework, so signalling procedure for L3 measurement reporting should be the baseline. So, we assume the following procedure for method 1:

- Step 1. NW provides the measurement configuration (including reporting periodicity) for NW side data collection to UE via RRCReconfiguration message.

- Step 2. UE logs the measurement results until the next reporting occasion.

- Step 3. Every reporting periodicity, UE reports the logged measurement results via MeasurementReport message.

For Method 2, the following procedure can be considered:

- Step 1. NW provides the measurement configuration for NW side data collection to UE via RRCReconfiguration message.

- Step 2. UE logs the measurement results until an on-demand request from NW.

- Step 3. NW sends an on-demand request to retrieve logged data.

  - Option 1. via UEInformationRequest message

  - Option 2. via RRCReconfiguration message

- Step 4. UE reports logged data upon reception of the on-demand request from NW.

  - Option 1. via UEInformationResponse message

  - Option 2. via MeasurementReport message

In our understanding, aforementioned two options in Method 2 are feasible, but we prefer Option 2 as Method 1 uses the same reporting message (i.e., MeasurementReport message). We do not see the need to define two separate reporting messages for the same feature (i.e., NW side data collection).

The next issue is how UE can differentiate Method 1 and Method 2. Obviously, UE can do based on Step 1 of both methods. In other words, NW configures either periodic reporting (i.e., Method 1) or On-demand request based reporting (i.e., Method 2) in Step 1. Based on this configuration in Step 1, UE can choose to whether to perform Step 2 in Method 1 or Method 2.

Proposal 2. NW configures either 1) Periodic reporting or 2) On-demand request based reporting, when configuring NW side data collection.

Proposal 3. When configured with periodic reporting, UE logs the measurement results until the next reporting occasion. Then, UE reports the logged data via MeasurementReport message every reporting periodicity configured.

Proposal 4. When configured with on-demand request based reporting, UE logs the measurement results until an on-demand request from NW. Then, UE reports the logged data via MeasurementReport message, upon reception of request from NW via RRCReconfiguration message.

Availability for logging

RAN2 agreed on-demand request for logged data (i.e., Method 2). So, in our understanding, there is no need for UE’s autonomous reporting (ex. AS buffer event based reporting). Given multiple UEs may be configured in a cell to collect sufficient data for completion of NW side model training, UE’s autonomous reporting may cause severe network congestion. It is safer for UE to send reports based on periodicity configured by NW (i.e., Method 1) or NW’s on-demand request (Method 2).

However, the issue for Method 2 is, NW may not know whether UE stores logged data or not (i.e., availability) or how much logged data UE stores. Without this UE-side logging information, NW cannot send an on-demand request timely to retrieve the logged data. As this information (ex. availability or logged data size) is UE side assistance information which may be helpful to NW, we assume UAI can be used to deliver this information to NW.

Proposal 5. UE reports logging information (ex. availability or logged data size) via UAI.

Meanwhile, RAN2 agreed not to support RRC segmentation for NW side data collection.

<Agreements (RAN2#127)> - The UE implementation can determine how many entries to include in the list radio measurements information, such that the maximum PDCP SDU size is not exceeded. No standardized RRC segmentation procedure is needed (as for the logged MDT measurements)

So, when UE stores a big size of logged data, all the logged data may not be accommodated in a single MeasurementReport message (as proposed in Proposal 4). In that case, UE needs to indicate the availability for the residual logged data when sending the MeasurementReport message. Based on this information, NW can request further UE’s reports. This is similar to logMeasAvailable in UEInformationResponse message in logged MDT.

Proposal 6. Availability indication is defined in MeasurementReport message to indicate the residual logged data.

Next, we consider the scenario UE handovers to other NW while logging training data. During handover, if NW data collection configuration is kept in UE, UE needs to inform the new gNB of its availability. Then, the new gNB can request retrieval of the logged data of UE.

Proposal 7. UE can indicate availability for logged data in RRCReconfigurationComplete message with reconfigurationWithSync.

Data type of UE/NW-side data collection

For UE-side data collection, there are 4 options (i.e., option 1a, 1b, 2 and 3) on the table. However, RAN2 could not agree any option and now RAN2 is waiting for response from SA WGs. Based on Option 1a, there is no need to discuss type of data for UE side data collection. For the other options, it is also pending whether standardized data is needed. Even if needed, we understand data type should be discussed in RAN1 first. So, we do not think RAN2 needs to discuss type of data for UE-side data collection at least this meeting.

For NW-side inference, RAN1 agreed UE reports L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set. We assume the similar information can be collected for NW-side data collection for training. However, we think this issue is not urgent and RAN1 can discuss first like the UE-side data collection, so we propose to wait RAN1 input for data type of NW-side data collection.

Proposal 8. RAN2 to postpone the data type for UE-side data collection.

Proposal 9. RAN2 to wait RAN1 input for the data type for NW-side data collection.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. UE mandatorily supports the fixed size of minimum AS layer memory without UE capability signalling, and optionally reports via UE capability a larger minimum AS layer memory size.

Proposal 2. NW configures either 1) Periodic reporting or 2) On-demand request based reporting, when configuring NW side data collection.

Proposal 3. When configured with periodic reporting, UE logs the measurement results until the next reporting occasion. Then, UE reports the logged data via MeasurementReport message every reporting periodicity configured.

Proposal 4. When configured with on-demand request based reporting, UE logs the measurement results until an on-demand request from NW. Then, UE reports the logged data via MeasurementReport message, upon reception of request from NW via RRCReconfiguration message.

Proposal 5. UE reports logging information (ex. availability or logged data size) via UAI.

Proposal 6. Availability indication is defined in MeasurementReport message to indicate the residual logged data.

Proposal 7. UE can indicate availability for logged data in RRCReconfigurationComplete message with reconfigurationWithSync.

References