3GPP TSG-RAN WG2 Meeting #127bis	R2-2408616

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:	8.3.3

Source:	Samsung

Title:	Discussion on measurement event prediction

Document for:	Discussion & Decision

Introduction

This document discusses on measurement event predictions for AI/ML mobility SI.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

Discussion

The output of direct/indirect prediction

For measurement event prediction, the following agreement was made in RAN2#125bis:

Agreement (RAN2#125bis) - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed. - Clarifications on what is being as input should be provided with results - Start with A3 as a baseline. - Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

Based on the agreement, RAN2 studies two different approaches for measurement event prediction i.e., 1) indirect prediction and 2) direct prediction.

Let us first discuss on the indirect measurement event prediction (see Figure 1).

Figure 1. Indirect measurement event prediction

In the indirect event prediction, RRM prediction model is re-used. The inputs for the RRM prediction model are cell-level or beam-level measurement results (depending on Case 1, 2, and 3 in RRM prediction) for the serving cell and neighboring cells. The output for the RRM prediction model (i.e., intermediate output) are the predicted cell level measurement results (e.g., RSRP/RSRQ/SINR) for serving cell and neighboring cells. As shown in the Figure 1, using this RRM prediction result, there could be a rule (i.e., non-AI) to derive measurement event (ex. event A3) prediction results. For example, if the predicted RSRP for a neighboring cell is better than that for a serving cell during TTT (i.e., to emulate event A3), we can assume event A3 is predicted to trigger. Although any further details for the rule are FFS, we assume this simple example as the baseline rule. Then, for indirect event prediction, we can predict the time instance of event occurrence (i.e., the earliest time instance before which the predicted RSRP for a neighboring cell has been better than the serving cell during TTT).

Proposal 1. For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP/RSRQ/SINR of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3).

In direct measurement event prediction (see Figure 2), actual RRM measurement results (e.g., RSRP/RSRQ/SINR) could be one of input parameters. One of differences between direct and indirect prediction is event specific inputs for the model inference can be considered in direct event prediction. For example, for event A3 prediction, A3-offset, hysteresis, and TTT can be used. Besides, associated MO related information can be also considered as input of the model. Similar to RLF prediction, we assume the probability of the event occurrence within a prediction window as output of direct event prediction. The prediction window could be pre-configured or the input of the model.

Figure 2. Direct measurement event prediction

Proposal 2. For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

Performance metric/KPI

For evaluation of accuracy of measurement event prediction, we can consider the same metrics agreed for RLF prediction. i.e., missed detection, false detection, F1 score. These metrics can be commonly used for both direct and indirect prediction.

For indirect RLF prediction, RAN2 agreed to use time difference between true RLF and predicted RLF, given the final outcome of indirect RLF prediction is the expected time of RLF. We assume the same for indirect event prediction (i.e., time difference between true event and predicted event), and would like to clarify the time difference is calculated only if the event is predicted and actually occurs. Meanwhile, although SINR difference was agreed for indirect RLF prediction, we do not think RSRP/RSRQ/SINR difference for KPI of indirect event prediction. RLF prediction is performed based on SINR only for serving cell, but event A3 prediction is performed based on comparison of RRM prediction results (RSRP/RSRQ/SINR) between the serving cell and a neighboring cell. So, in our understanding, RRM prediction error cannot be assumed as a direct metric reflecting event prediction accuracy. For example, if RSRP prediction error is +5dB for serving cell and +4dB for a neighboring cell, their difference is only 1dB so there would not be much performance degradation in event A3 prediction itself.

Proposal 3. Missed detection, false detection, and F1 score are considered as KPI for direct measurement event prediction.

Proposal 4. Missed detection, false detection, F1 score, and time difference are considered as KPI for indirect measurement event prediction.

Simulation assumption

For simulation of measurement event prediction, we first want to clarify the goal of this use case. During the discussion for RRM measurement prediction, RAN2 agree to consider two target study goals (1st goal: Reduce measurement effort, 2nd goal: HO KPI improvement). In measurement event prediction, it is assumed that the UE predicts some measurement event (e.g., A3) and reports the predicted event to gNB to help HO decision. Here, we can expect the gNB to use the prediction results for HO timing/target optimization, but there would not be measurement reduction at the UE-side. Therefore, we think the measurement event prediction is for the 2nd goal.

Proposal 5. Measurement event prediction use case is for 2nd study goal (i.e., HO KPI improvement).

Assuming that there is not much room to improve HO KPI in FR1, RAN2 already agreed to prioritize the evaluation scenario in FR2 for the use case targeting 2nd study goal (e.g., temporal domain Case A prediction). With the same logic, we can prioritize the evaluation of measurement event prediction in FR2 rather than in FR1.

Proposal 6. For evaluation of measurement event prediction, RAN2 focus on FR2 to FR2 intra-frequency HO scenario.

At the previous RAN2#125bis, RAN2 agreed to start with A3 event as a baseline for evaluation of measurement event prediction and thus we need to determine reference parameter values for A3. For A3 event, we can think of two related parameters, timeToTrigger(TTT) and a3-Offset. In practice, network operators can use different TTT & A3 offset value sets at different areas with different cell planning. Since it is uncertain which value set can most efficiently enhance handover KPI performance with measurement event prediction, we suggest to use the 5parameter sets ({TTT, A3 offset} = {480ms, 3 dB}, {160ms, 3 dB}, {160ms, 2 dB}, {80ms, 1 dB}, {40ms, -1dB}) in TR 36.839 as a baseline. However, we have one issue for the parameter set {40ms, -1dB} since the sampling period of RRM measurement in FR2 is agreed as 80ms. In our view, the TTT value (i.e., 40ms) smaller than the sampling period (i.e., 80ms) does not make sense for simulation. Thus, we don’t need to consider 40ms as candidate value for TTT.

Proposal 7. For evaluation of A3 event prediction, the A3 offset/TTT parameter sets in TR 36.839 can be a baseline, but the value (i.e., 40ms) smaller than the sampling period (i.e., 80ms) is not considered for TTT.

Regarding relation to HO procedure, the network usually triggers HO procedure based on the A3 event report from the UE. Therefore, we can assume that A3 measurement event always occurs before HO procedure and thus A3 event prediction is independent of HO procedure. Thus, to obtain the dataset for A3 event prediction, we can exclude HO modelling in the simulation and let UE’s trajectory terminate upon A3 event occurrence.

Proposal 8. To obtain the dataset for A3 event prediction, no HO execution is expected and UE’s trajectory terminates upon A3 event occurrence in simulation.

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP/RSRQ/SINR of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3).

Proposal 2. For direct measurement event prediction, the model output is the probability of event occurrence within a time window.

Proposal 3. Missed detection, false detection, and F1 score are considered as KPI for direct measurement event prediction.

Proposal 4. Missed detection, false detection, F1 score, and time difference are considered as KPI for indirect measurement event prediction.

Proposal 5. Measurement event prediction use case is for 2nd study goal (i.e., HO KPI improvement).

Proposal 6. For evaluation of measurement event prediction, RAN2 focus on FR2 to FR2 intra-frequency HO scenario.

Proposal 7. For evaluation of A3 event prediction, the A3 offset/TTT parameter sets in TR 36.839 can be a baseline, but the value (i.e., 40ms) smaller than the sampling period (i.e., 80ms) is not considered for TTT.

Proposal 8. To obtain the dataset for A3 event prediction, no HO execution is expected and UE’s trajectory terminates upon A3 event occurrence in simulation.

References