3GPP TSG-RAN WG2 Meeting #127bis											R2-2408617

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.7.3

Source: 	Sharp

Title:  	Measurement Gap Enhancements for XR

Document for: 	Discussion

# Introduction

The WID on XR Phase 3 has added an objective on enhancements to enable transmission/reception in gaps, as follows:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

This document discusses RAN2 aspects of the measurement gap enhancements:

# Discussion

In RAN1#118, RAN1 made a working assumption that DCI is used for an explicit indication to skip a particular gap as follows:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s).

RAN2 needs to discuss whether and what RAN2 spec impacts are expected. TS 38.321 specifies the MAC behaviour during the activated measurement gap. The legacy measurement gap is configured by RRC. An assumption is that the MAC entity is aware of the time during which the measurement gap is activated. Thus, the MAC specification does not specify how the MAC entity is aware of it.

For DCI indication, RRC signaling does not provide any information on the actual activated time of measurement gap. Thus, MAC entity needs to refer to somewhere in the specification. A simplest way is that the MAC specification specifies that MAC entity considers the corresponding first measurement gap offset after the DCI as not activated. Then, we can just reuse other existing texts without any change.

TS 38.321 specifies: if the explicit indication by DCI to skip a particular gap, the MAC entity considers the corresponding first measurement gap as not activated.

In RAN1#117, RAN1 reached the conclusion on UE assistance information without consensus as follows:

Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.

Instead, RAN1 sent an LS to RAN4 (R1-2405736) to ask both feasibility and usefulness of the UAI on measurement occasion. Triggered by the LS, RAN4 started the relevant discussion but they did not reach the conclusion yet. In this situation, RAN2 discussion on UAI may be too early. We see that RAN2 starts the discussion on UAI after RAN4 reaches a conclusion that it is at least feasible.

RAN2 discusses UAI on measurement occasion after RAN4 reaches a conclusion.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- TS 38.321 specifies: if the explicit indication by DCI to skip a particular gap, the MAC entity considers the corresponding first measurement gap as not activated.

RAN2 discusses UAI on measurement occasion after RAN4 reaches a conclusion.