3GPP TSG-RAN WG2 Meeting #127bis      											                     R2-2408624

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.9.4

Source:	MediaTek Inc.

Title:	Discussion on supporting PWS for NB-IoT

Document for:	Discussion and Decision

# Introduction

New objective was added to Rel-19 IoT NTN work item in RAN#105 [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

# Discussion

To support the PWS message for NB-IoT, the LTE mechanism can be reused as indicated in the WID.

The PWS message contains the ETWS and CMAS. ETWS is a public warning system developed to meet the regulatory requirements for warning notifications related to earthquake and/or tsunami events. ETWS warning notifications can either be a primary notification or secondary notification (providing detailed information). CMAS is a public warning system developed for the delivery of multiple, concurrent warning notifications. The CMAS warning notifications are short text messages.

Similar to LTE, the new SIB10-NB, SIB11-NB and SIB12-NB shall be introduced for ETWS primary notification, ETWS secondary notification, and CMAS notification. The content of these SIBs should be exactly the same as that in LTE.

Proposal 1: Introduce new SIB10-NB, SIB11-NB, and SIB12-NB for ETWS primary notification, ETWS secondary notification, and CMAS notification respectively. The content of these SIBs is exactly the same as that in LTE.

Similar to LTE, new bits in paging message should be introduced to send ETWS indication and CMAS indication to the UE in RRC Idle mode.

Proposal 2: Introduce new ETWS indication and CMAS indication in Paging-NB message.

In Idle mode, the Direct Indication Information using P-RNTI only includes system information modification indication as show in 36.331 Table 6.7.5-1.

Table 6.7.5-1: Direct Indication information

| Bit | Field in Direct Indication information |
| --- | --- |
| 1 | systemInfoModification |
| 2 | systemInfoModification-eDRX |
| 3, 4, 5, 6, 7, 8 | Not used, and shall be ignored by UE if received |

To support PWS on IoT NTN, new bits in Direct Indication Information using P-RNTI should be introduced to send ETWS indication and CMAS indication to the UE in RRC Idle mode. But this should also be defined in RAN1, so an LS to RAN1 is needed.

Proposal 3: Send an LS to RAN1 for new ETWS indication and CMAS indication in Direct Indication Information using P-RNTI.

For eMTC, receiving Direct Indication Information using SI-RNTI in connected mode is supported as in 36.331 Table 6.6-2.

Table 6.6-2: Direct Indication information using SI-RNTI

| Bit | Direct Indication information |
| --- | --- |
| 1 | etws-Indication |
| 2 | cmas-Indication |
| 3, 4, 5, 6, 7, 8 | Not used, and shall be ignored by UE if received. |

NB-IoT does not monitor paging message in RRC connected mode. To acquire ETWS indication and CMAS indication in RRC connected mode, a new Direct Indication Information in RRC connected mode similar to that in eMTC should be introduced. This should also be defined in RAN1, and an LS for this is needed.

Proposal 4: Send an LS to RAN1 for new Direct Indication Information containing ETWS indication and CMAS indication in RRC Connected mode.

Monitoring the Direct Indication Information in RRC connected mode will result in increased power consumption. Hence, whether a UE needs to monitor the Direct Indication Information in RRC connected mode should be configured by the network. Similar to eMTC, a new dedicated RRC configuration should be introduced.

Proposal 5: Introduce a new dedicated RRC configuration to receive new Direct Indication Information in RRC Connected mode.

The network needs to know if the UE capable of monitoring new Direct Indication Information in RRC Connected mode, so that it can determine whether to send the configuration. Similar to eMTC, a new UE capability is needed.

Proposal 6: Introduce a new UE capability to receive new Direct Indication Information in RRC Connected mode.

There is a concern regarding NB-IoT’s ability to supporting PWS message. According to TS 22.168, the ETWS Primary Notification shall be delivered within 4 seconds to the UE in the Notification Area where the Warning Notification is expected to be distributed even under congestion situation. However, NB-IoT is primarily designed to provide delay-tolerant services, and a few configurations can affect the monitoring of the Paging/Direct Indication Information messages in RRC idle mode and RRC connected mode. Under some configurations, ETWS Primary Notification cannot be delivered to the UE within 4 seconds.

- Default paging cycle in RRC idle mode

defaultPagingCycle-r13; ENUMERATED {rf128, rf256, rf512, rf1024}.

- DRX cycle in RRC connected mode

drx-Cycle-r13 ENUMERATED { sf256, sf512, sf1024, sf1536, sf2048, sf3072, sf4096, sf4608, sf6144, sf7680, sf8192, sf9216, spare4, spare3, spare2, spare1},

- eDRX

- PSM

When the network needs to send ETWS, it can adjust the configuration to ensure that the ETWS primary notification is delivered to the UE within 4 seconds.

Proposal 7: It is up to network configuration to ensure that the ETWS primary notification can be delivered to the UE within 4 seconds.

# Conclusion

Proposal 1: Introduce new SIB10-NB, SIB11-NB, and SIB12-NB for ETWS primary notification, ETWS secondary notification, and CMAS notification respectively. The content of these SIBs is exactly the same as that in LTE.

Proposal 2: Introduce new ETWS indication and CMAS indication in Paging-NB message.

Proposal 3: Send an LS to RAN1 for new ETWS indication and CMAS indication in Direct Indication Information using P-RNTI.

Proposal 4: Send an LS to RAN1 for new Direct Indication Information containing ETWS indication and CMAS indication in RRC Connected mode.

Proposal 5: Introduce a new dedicated RRC configuration to receive new Direct Indication Information in RRC Connected mode.

Proposal 6: Introduce a new UE capability to receive new Direct Indication Information in RRC Connected mode.

Proposal 7: It is up to network configuration to ensure that the ETWS primary notification can be delivered to the UE within 4 seconds.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)