3GPP TSG-RAN WG2 Meeting  #127bis                                                                  R2-2408630

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.7.5

Source: 	Meta

Title:  	Discussion on RLC AM Enhancements for XR

Document for:	Discussion and Decision

# Introduction

In the Revised WID on XR (eXtended Reality) for NR Phase 3, one of the objectives is to specify RLC AM enhancements [1]:

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In RAN2 #126, the following agreements were made:

Agreement - To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism - Impact on capacity should be considered - RAN2 focuses on the enhancements for UL traffic

In this paper, we discuss our views on the RLC AM enhancements for XR.

# Discussion

The RLC layer offers two operating modes: Acknowledged Mode (AM) and Unacknowledged Mode (UM). RLC AM mode employs a reliable data transmission method by using ARQ, where the receiver sends acknowledgments for correctly received data blocks and requests retransmission for those with errors. Conversely, RLC UM mode is a less reliable, efficient mode that does not employ ARQ or error correction mechanisms. Data is transmitted without any acknowledgment or retransmission requests, making it suitable for delay-sensitive applications where a small amount of data loss can be tolerated.

As identified and acknowledged in [1], RLC AM mode is useful to limit data loss. However, its feedback or retransmission triggering mechanisms are not well adapted for short packet delay budgets applicable to XR traffic. Also, for RLC AM, considerable amounts of data may be in-flight, i.e. in the window, and there is no current way to avoid retransmitting this data, even if the data is old.

For delay-sensitive and reliability-demanding traffic, an approach to improve the delay associated with RLC retransmissions is to enable early retransmissions without dependence on polling or STATUS reports. This can be achieved by autonomously performing RLC SDU retransmissions at a shortened, fixed time interval, ensuring all retransmissions complete within the Packet Delay Budget (PDB).

Proposal 1: Support RLC retransmissions at a shortened time interval within PDB.

Alternatively, for delay-sensitive and less reliability-focused traffic, RLC retransmissions could be terminated once the PDB is exceeded.

Proposal 2: Support termination of RLC retransmissions beyond PDB.

# Conclusion

In this paper, we discuss two approaches to improve RLC retransmissions:

Proposal 1: Support RLC retransmissions at a shortened time interval within PDB.

Proposal 2: Support termination of RLC retransmissions beyond PDB.

# References

[1] RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3” Melbourne, Australia, September 9-12, 2024