3GPP TSG-RAN WG2 Meeting #127bis			           	R2-2408640

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.4

Source: 	Lenovo

Title: 	Conditional LTM basics

Document for:	Discussion and decision

# Introduction

In RAN plenary in Melbourne instructed RAN2 to work on the Conditional LTM topic. This document attempts to develop a RAN2 understanding of “conditional LTM” and then further proposes some first scenarios for RAN2 consideration.

# Discussion

Configuration of Conditional LTM

The Intra-CU LTM feature has been specified in Rel. 18 and some further enhancements (Inter-CU) is in progress in Rel. 19. Here UE receives and RRC configuration message allowing the UE to receive RRC Configuration of one or more candidate cells. In the due course of the procedure, UE initiates an Early TA for one or more candidate cell based on receiving PDCCH order. An Early TA may be determined and sent to the UE in LTM MAC CE, if the UE has already measured and reported at least one target cell.

Now in Rel. 19, as part of this work, an alternative to the above LTM procedure is to be specified, where a conditional handover reconfiguration message can be used by a gNB to transmit a message (e.g., RRCReconfiguration) to the UE including both LTM candidate cell configurations as well as their corresponding Execution Condition(s).

As “configuration” has been in RRC domain and worked well in the field, we see no need to change this and therefore we propose:

Proposal 1: To configure conditional LTM, a Serving gNB transmits a RRCReconfiguration to the UE including LTM candidate cell configurations as well as their corresponding Execution Condition(s).

Evaluation of Conditional LTM

Rel. 18 LTM procedure is designed to reduce HO latency by acquiring the Early TA but since the L1 measurement reports are still used, network could do "some filtering" on its side, up to network implementation. And therefore, the LTM decisions for finding a target cell can be good enough.

Observation 2a: Rel. 18 LTM decisions for finding a robust target cell can be good enough since it is still in network control and network can perform some “averaging” based on L1 reports received from the UE.

In Conditional LTM, we should be able to benefit from Early TA but at the same time the mobility decision should also be robust enough. Unlike Inter-CU case (where RAN2 agreed to use MAC for event evaluation and measurement report triggering), in Conditional LTM there's no network filtering possible as there's no reporting. So, it might be beneficial to have still L3 based filtering for Conditional LTM

Observation 2b: Depending only on L1 measurements (without L3 filtering) and no network-side averaging possible, finding a robust LTM target based purely on single L1 execution condition event can be challenging.

In view of the above observations, we propose:

Proposal 2: L3 filtering is used for evaluation of a configured Conditional LTM candidate cell.

Timeline confusion

In case of conditional handover procedure, a handover may only be executed by the UE when included condition(s) for at least one candidate cell are fulfilled. The fulfilment of radio condition(s) is not dependent on acquisition of an Early TA – UE receives it from the source in the LTM MAC CE.

One question therefore arises: What if at the point of fulfilment of included condition(s) for a first candidate cell, the early TA acquisition of that candidate cell has not been received by the UE? In other words, if an Early TA value is not yet available with the UE but radio condition(s) for a potential candidate is fulfilled, should it initiate afresh RACH procedure towards the candidate (target) cell?

Figure 1: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., before receiving a PDCCH Order for C1

As another question, if an Early TA value is available for a first candidate cell with the UE but included condition(s) for a first candidate cell are not yet fulfilled, how long shall the UE keep evaluating the said candidate cell while assuming the obtained TA is still valid?

Figure 2: Condition(s) for a candidate cell C1 is fulfilled at point A, i.e., after receiving an Early TA for C1

There will be other “timeline confusion” scenarios but first it is better to understand if RAN2 sees these as something to work and resolve.

Proposal 3: RAN2 should discuss if the “timeline confusion” scenarios need to be resolved to implement Conditional LTM or treat solutions where such confusion will not even take place.

Other details

As part of a possible solution, it can be argued that UE should obtain TA of a candidate cell (for which it received PDCCH order and made the Msg1 transmission) directly from that candidate cell, instead of Rel. 18 way of receiving it from source cell. This will be a change compared with Rel. 18 baseline and this also does not resolve the “time confusion”. Therefore, we propose:

Proposal 4: Like in Rel. 18 LTM Baseline, the Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE).

When the serving gNB(CU) decides to prepare the LTM candidate cell, the serving gNB may request its DU to prepare the condition for a LTM candidate cell. Serving gNB (DU) generates the condition for LTM candidate cell and transmits the condition to serving gNB(CU). Then, serving gNB (CU) transmits the condition via RRC reconfiguration message to UE.

Proposal 5: Serving gNB (DU) generates the condition for LTM candidate cell based on the request from serving gNB(CU). And transmits the condition to serving gNB(CU).

# Conclusion

This document attempted to develop a basic understanding of “conditional LTM” and proposed some first scenarios for RAN2 consideration. In summary, we have following observations and proposals:

Configuration of Conditional LTM

Proposal 1: To configure conditional LTM, a Serving gNB transmits a RRCReconfiguration to the UE including LTM candidate cell configurations as well as their corresponding Execution Condition(s).

Evaluation of Conditional LTM

Observation 2a: Rel. 18 LTM decisions for finding a robust target cell can be good enough since it is still in network control and network can perform some “averaging” based on L1 reports received from the UE.

Observation 2b: Depending only on L1 measurements (without L3 filtering) and no network-side averaging possible, finding a robust LTM target based purely on single L1 execution condition event can be challenging.

Proposal 2: L3 filtering is used for evaluation of a configured Conditional LTM candidate cell.

Timeline confusion

Proposal 3: RAN2 should discuss if the “timeline confusion” scenarios need to be resolved to implement Conditional LTM or treat solutions where such confusion will not even take place.

Other details

Proposal 4: Like in Rel. 18 LTM Baseline, the Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE).

Proposal 5: Serving gNB (DU) generates the condition for LTM candidate cell based on the request from serving gNB(CU). And transmits the condition to serving gNB(CU).

# References

- Chair notes for RAN2#127