3GPP TSG-RAN WG2 Meeting #127bis	R2-2408647

Hefei, China, 14 – 18 October 2024

Agenda item:	8.11.2

Source:	Nokia

Title:	Random Access Operation of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved. SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where a separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. The intention is to allow more UL transmission opportunities compared to the typical DL-heavy TDD configurations used in today’s TDD deployments. This bring benefits in terms of UL coverage and UL latency, especially in wide-area macro networks where many UEs are in power-limited conditions in the UL direction. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in  RP-241614.

RAN2 start discussing on SBFD random access procedures in RAN2#127, and the following agreemenst are made:

Agreements Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. FFS whether/how early indication is used during a SBFD RA procedure.

In this contribution, we further discuss different aspects of the SBFD random access operation.

# SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. As mentioned in section 1, WI was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

In section 3, we present the various aspects of random access in RRC Connected mode, and in RRC Idle/Inactive mode, and the conclusion is provided in section 4.

# Various Aspects of SBFD random access

In this section, we discuss the RAN2 aspects of the SBFD random access procedure for RRC_CONNECTED and IDLE/INACTIVE operations.  RAN1 agreements on SBFD random access is listed in Appendix 1.

## PRACH resource configuration for RRC_CONNECTED UEs

To enable random access for SBFD symbols in SBFD-aware UEs, the gNB must set up a new set of PRACH resources which includes RO configuration, SSB-RO mapping, the random access configuration table, and PRACH repetition.

In RAN1#117, the following agreements are made on the PRACH resource configuration:

Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options.

For option 1, it seems straightforward to reinterpret that for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs which fall outside the UL subband after applying offset are considered as invalid ROs for SBFD UEs.

Proposal 1: For option 1,  for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs that fall outside the UL subband after applying offset are considered invalid ROs for SBFD UEs.

On the FFS part of option 2, since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough.

Proposal 2: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters, only the legacy RACH configuration will be configured with this parameter.

For Option 2, according to the current RA procedure, the following methods were defined in order to configure separated ROs

- Method 1: define a new IE for the separated RACH configuration

- Method 2: define a feature for RACH partitioning, and use RACH configuration within AdditionalRACH-Config.

Since there are already different features such as SDT, Redcap, Slicing, etc which partition the preambles, further partition of the RACH preambles will reduce the number of preambles available for the CBRA, and hence increase collision. Hence we propose:

Proposal 3: Define a new IE for the separated RACH configuration for SBFD UEs

## PRACH power control for RRC_CONNECTED UEs

A UE calculates the PRACH preamble transmit power through an open-loop power control method, which relies on a downlink path loss measurement. The MAC layer performs the initial part of this calculation using the following expression:

PREAMBLE_RECEIVED TARRGET_POWER = preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAM PING COUNTER -1) x preamblePowerRampingStep

preambleReceivedTargetPower is provided to the UE within the RACH-ConfigCommon parameter structure, DEL TA_PREAMBLE represents a power offset which adjusts the PRACH Preamble received power requirement according to the PRACH sequence duration (Its value is extracted from a look-up table standardized by 3GPP within TS 38.321). A UE maintains two counters when transmitting a series of PRACH preambles: PREAMBLE_ TRANSMISSION_ COUNTER and PREAMBLE_POWER_RAMPING_COUNTER. The former is used to limit the total number of PRACH preamble transmissions within a single Random Access procedure, whereas the latter is used to control the ramping of the PRACH Preamble transmit power (visible within the equation shown above). preamblePowerRampingStep can be configured with either a 'normal' value or a 'high priority' value. The 'normal' value is provided to the UE within the RACH-ConfigCommon parameter structure. The 'high priority' value can be used to override the 'normal' value when it is necessary to minimize latency, i.e. the 'high priority' step size can be configured with a higher value to allow more rapid

power ramping.  Once, the MAC layer has calculated PREAMBLE RECEIVED TARGET POWER, the result is passed to the PHY which then calculates the PRACH preamble transmit power.

Considering that SBFD slots can be significantly different than UL slots in terms of frequency resource availability, number of available time slots, SINR, CLI conditions, etc., for RACH Configuration option 2, it is reasonable to specify separate transmit power and power ramp-up parameters for Msg1 transmissions in SBFD-ROs compared to legacy ROs.

Proposal 4: For RACH Configuration option 2,  specify separate transmit power and power ramp-up parameters for Msg1 transmissions in SBFD-ROs compared to legacy ROs.

## RACH Occasion selection and prioritization

For both RACH configuration Option 1 and Option 2 agreed in RAN1, SBFD-aware UEs will have the possibility of transmitting RACH on legacy ROs and SBFD ROs, while legacy UEs would only be capable of using legacy ROs. For load balancing purposes, it is beneficial that SBFD-aware UEs prioritize as much as possible RACH transmissions on SBFD ROs and avoid consuming the quite-limited capacity of legacy RACH occasions. Considering that UE-to-UE CLI may only be a problem in SBFD ROs but not in legacy ROs, and may be especially critical in cell-edge areas (where the UL transmission power is high, while the DL received signal power is low), the selection between legacy ROs and SBFD ROs may also be based UE’s coverage conditions. For instance, a cell-centre UEs transmit RACH on SBFD ROs, while a cell-edge SBFD-aware UE uses ROs belonging to the legacy RACH configuration.

Proposal 5: From the SBFD-aware UE perspective and to have better load balancing across different RACH resources, (new) SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions, e.g. SBFD ROs are only used by UEs in cell-centre conditions.

## RA-RNTI collision in SBFD

For the additional RACH configuration in option 2, designed for SBFD-aware UEs, a challenge is avoiding RA-RNTI confusion caused by having two sets of RACH resources. The RA-RNTI is used by a UE to receive the Random Access Response (RAR) once the PRACH is transmitted. Different RACH occasions within a system frame are assigned unique RA-RNTI values to ensure that the RAR for a given PRACH transmission is directed only to the UE that used that specific RACH occasion. If two RACH occasions share the same RA-RNTI value, there is a risk that two UEs using different RACH occasions could incorrectly act on the same RAR message.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

Proposal 6: RAN2 to discuss methods to avoid RA-RNTI collision in SBFD UEs.

## Early Indication for SBFD RA procedure

From the RAN2 perspective,  no early indication is needed for the SBFD RA procedure. This is because, for Option 1 with Alt 1-1, since both SBFD-aware and legacy UEs will use the same RACH configuration for the RA procedure, there will be no significant impact on the current MAC procedure. This means that the MAC procedure for selecting the RACH partition, Preamble Group, and/or SSB will rely on the same RA parameters for both SBFD-aware and legacy UEs.  The differentiation comes at the RO validation stage, which is handled by the RAN1. For Option 2, the network can differentiate between the SBFD ROs and legacy ROs based on the RACH configuration used.

Proposal 7: From the RAN2 perspective,  no early indication is needed for the SBFD RA procedure.

# Conclusion

Proposal 1: For option 1,  for ROs within SBFD symbols, msg1-FrequencyStart indicates the offset of the first RO with respect to the start of the UL subband, and FDMed ROs which falls outside the UL subband after applying offset  is considered as invalids ROs for SBFD UEs.

Proposal 2 : If same parameters /values are used in RO validation rules, SSB-RO mapping rules for SBFD UEs and legacy UEs, then those parameters are the included only in legacy RACH configuration.

Proposal 3: Define a new IE for the separated RACH configuration

Proposal 4: For RACH Configuration option 2,  specify separate transmit power and power ramp up parameters for Msg1 transmissions in SBFD-ROs compared to legacy ROs.

Proposal 5: From the SBFD-aware UE perspective and in order to have better load balancing across different RACH resources, (new) SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions, e.g. SBFD ROs are only used by UEs in cell-centre conditions.

Proposal 6: RAN2 to discuss methods to avoid RA-RNTI collision in SBFD UEs.

Proposal 7: From the RAN2 perpective,  no early indication is needed for the SBFD RA procedure.