3GPP TSG-RAN WG2 #127bis	R2-2408659

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.6.2	Inter-CU LTM

Title:		Discussion on inter-CU LTM

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The Objective 1 of the WID [1] for NR mobility enhancements Phase 4 is formulated as follows:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

During the previous meeting, RAN2 discussed several aspects to support inter-CU LTM, including scenarios, RRC configuration and signalling procedures at different LTM stages, some agreements for inter-CU LTM have been reached [2][3][4].

In this contribution, we further discuss the signalling flow of inter-CU LTM procedure and other details of inter-CU LTM.

# 2. Discussion

Based on the previous RAN2 agreements, several cases for inter-CU LTM are listed as follows:

Case 1: DC is not configured, i.e., CU is acting as MN

Case 2: inter-CU SCG LTM in DC, only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19

Case 3: inter-CU MCG LTM in DC, SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g., release or reconfiguration).

In the following part, we will analyze the procedure of different cases.

Inter-CU LTM without DC

For the case CU is acting as MN when DC is not configured, when an inter-CU LTM is applied, the UE is switched to a cell belongs to another gNB. Taking intra-CU LTM procedure and Intra-AMF/UPF handover procedure defined in TS 38.300 as reference, we depict the inter-CU LTM without DC in the following Figure 1.

Figure 1. inter-CU LTM without DC

During the RAN2#127 meeting, RAN2 agreed that the inter-CU LTM re-uses the reference configuration from Rel-18 LTM, no additional reference configurations are supported. Regarding the issue of which node to decide a reference configuration, it is similar to SCPAC, we think that the source gNB can decide it. If the source gNB decides to prepare the reference configuration by itself, then the source CU can request the source DU to provide it. If the source gNB decides that the candidate gNB should provide the reference configuration, the source CU may include the reference configuration request indication in handover request message, then the candidate CU can obtain the reference configuration from the candidate DU and provide it to source CU in handover request acknowledge message.

Proposal 1: For inter-CU LTM, either the source gNB or the candidate gNB can prepare the reference configuration, and the source gNB can decide which node should prepare it.

When the LTM cell switch is triggered by the source gNB, the target gNB needs to know about the initiation of the cell switch command to the UE. Especially for DG based RACH-less LTM, the target gNB starts the dynamic scheduling based on the notification. In addition, the selected TCI state also need to be transferred from source gNB to target gNB. In Rel-18 intra-CU LTM, only the CU-DU cell switch notification in F1 interface is needed because the CU is not changed. However, for Rel-19 inter-CU LTM, the CU is changed, cell change notification from source CU to target CU is also needed in Xn interface. On the detailed Xn signaling for cell switch notification, whether it can extend the current F1 message or introduce a new message can up to RAN3.

Proposal 2: The source CU should inform the target CU about the cell switch. The detailed signaling up to RAN3.

In Rel-18 intra-CU LTM, both CBRA-based and CFRA-based procedures can be supported. And for CFRA-based intra-CU LTM, the CFRA resource may be provided in RRC configuration, or CFRA resource related information may be included in LTM MAC CE, which similar to those in the legacy PDCCH order triggered RACH. We think that the intra-CU LTM mechanism can be reused for inter-CU LTM.

Proposal 3: CFRA-based LTM can be supported for inter-CU LTM.

In addition, the target gNB needs to send the LTM success indication to the source gNB to inform that the UE has successfully accessed the target cell. The detailed signalling, e.g., whether LTM success indication can be included in HANDOVER SUCCESS message can up to RAN3.

Proposal 4: The target gNB sends the LTM success indication to the source gNB to inform that the UE has successfully accessed the target cell.

Inter-CU SCG LTM in DC

For inter-CU SCG LTM in DC scenario, we agreed that only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. On top of inter-CU LTM without DC, we depict the SN-initiated inter-CU SN LTM in DC in the following Figure 2.

Figure 2. SN-initiated inter-CU SN LTM in DC

Since the LTM configuration, early DL and UL synchronization and LTM execution are initiated by SN, the UE should report the L1 measurement result to SN directly, so that the SN can trigger the inter-CU SCG LTM or send LTM cell switch command to the UE.

Proposal 5: For inter-CU SN LTM, the UE should report the L1 measurement report to SN directly.

Proposal 6: RAN2 take the Figure 2 as the baseline procedure for SN-initiated inter-CU SN LTM in DC.

Inter-CU MCG LTM in DC

For inter-CU MN LTM in DC, SCG configuration can be changed and leave how to handle SCG part up to NW implementation (e.g., release or reconfiguration). For inter-CU MN LTM with SCG change case, SN should be added in flowchart based on Figure 1, and the candidate MN to perform the SN addition procedure during the LTM preparation phase. For inter-CU MN LTM with SCG release, it is up to network (e.g., MN) to decide when to release the SN. We depict the inter-CU MN LTM with SCG configuration change in DC in the following Figure 2.

Figure 3. inter-CU MN LTM in DC

Proposal 7: RAN2 take the Figure 3 as the baseline procedure for inter-CU MN LTM in DC.

# 3. Conclusion

In this contribution we discussed some issues related to inter-CU LTM and made the following proposals.

Proposal 1: For inter-CU LTM, either the source gNB or the candidate gNB can prepare the reference configuration, and the source gNB can decide which node should prepare it.

Proposal 2: The source CU should inform the target CU about the cell switch. The detailed signaling up to RAN3.

Proposal 3: CFRA-based LTM can be supported for inter-CU LTM.

Proposal 4: The target gNB sends the LTM success indication to the source gNB to inform that the UE has successfully accessed the target cell.

Proposal 5: For inter-CU SN LTM, the UE should report the L1 measurement report to SN directly.

Proposal 6: RAN2 take the Figure 2 as the baseline procedure for SN-initiated inter-CU SN LTM in DC.

Proposal 7: RAN2 take the Figure 3 as the baseline procedure for inter-CU MN LTM in DC.

# 4. References

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4

[2] R2-2402102, Report of 3GPP TSG RAN WG2 meeting #125bis

[3] Draft Report of 3GPP TSG RAN WG2 meeting #126

[4] Report of 3GPP TSG RAN WG2 meeting #127