3GPP TSG-RAN WG2 #127bis	R2-2408660

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.16	BDS B2b in A-GNSS

Title:		Discussion on BDS B2b in A-GNSS

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

The Objective of the WID [1] for A-BDS is formulated as follows:

Specify A-BDS enhancements in Rel-19, including: - Supporting new signal of BDS B2b in A-GNSS for both E-UTRA and NR [RAN2]; - Specify corresponding assistant information in dedicated signaling [RAN2].

In this contribution, we provide our initial consideration on BDS B2b signal in A-GNSS.

# 2. Discussion

For BDS system in A-GNSS, the B1C, B2a and B3I signals are supported in TS 37.355. The current working procedures of TS 37.355 can be reused for B2b signal in BDS, but some BDS B2b specific information (e.g., new BDS ID, clock model for BDS B2b, Almanac for BDS B2b, etc.) and related description should be introduced. In the following part, we discuss the potential impact on top of BDS B2b signal.

For GNSS Assistance Data and GNSS Assistance Data elements, the potential impact analysis is as follows:

- GNSS Assistance Data (section 6.5.2.1 of TS 37.355)

| Data Elements and field | Impact description |
| --- | --- |
| A-GNSS-ProvideAssistanceData | No impact |
| GNSS-CommonAssistData | No impact |
| GNSS-GenericAssistData | No impact |
| GNSS-PeriodicAssistData | No impact |

- GNSS Assistance Data elements (section 6.5.2.2 of TS 37.355)

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTime | No impact |
| GNSS-SystemTime | No impact |
| GPS-TOW-Assist | No impact. |
| NetworkTime | No impact |
| GNSS-ReferenceLocation | No impact |
| GNSS-IonosphericModel > klobucharModel2-r16 | The IonosphericModel of BDS B2b is similar to existed KlobucharModel2Parameter, so KlobucharModel2Parameter can be reused for B2b, but the description/reference should be updated/added. Of course, the new model for B2b also can be considered. |
| NeQuickModelParameter | No impact |
| GNSS-EarthOrientationParameters | GNSS-EarthOrientationParameters can be reused for B2b BDS. The reference ICD file of BDS B2b shall be added in description. |
| GNSS-RTK-ReferenceStationInfo | No impact |
| GNSS-RTK-CommonObservationInfo | No impact. |
| GNSS-RTK-AuxiliaryStationData | No impact |
| GNSS-TimeModelList | No impact |
| GNSS-DifferentialCorrections | No impact. |
| GNSS-NavigationModel > NavModelSatelliteList >>GNSS- NavModelSatelliteElement >>>GNSS-ClockModel >>>>BDS-ClockModel2-r16 >>>GNSS-OrbitModel >>>>NavModel-BDS-KeplerianSet2-r16 | - BDS-ClockModel2 can be reused for B2b and new B2b specific fields need be added. Or new BDS-ClockModel3 is introduce for B2b - NavModel-BDS-KeplerianSet2 may be reused for B2b, or new NavModel-BDS-KeplerianSet3 may be introduced for B2b. - B2b health state is added in ‘GNSS to svHealth Bit String(8) relation’ table. - IOD of B2b is updated in ‘GNSS to iod Bit String(11) relation’ table. |
| StandardClockModelList | No impact |
| NAV-ClockModel | No impact |
| CNAV-ClockModel | No impact |
| GLONASS-ClockModel | No impact |
| SBAS-ClockModel | No impact |
| BDS-ClockModel | No impact |
| BDS-ClockModel2 | BDS-ClockModel2 can be reused for B2b and new B2b specific fields need be added. Or new BDS-ClockModel3 is introduce for B2b |
| NavModelKeplerianSet | No impact |
| NavModelNAV-KeplerianSet | No impact |
| NavModelCNAV-KeplerianSet | No impact |
| NavModel-GLONASS-ECEF | No impact |
| NavModel-SBAS-ECEF | No impact |
| NavModel-BDS-KeplerianSet | No impact |
| NavModel-BDS-KeplerianSet2 | NavModel-BDS-KeplerianSet2 may be reused for B2b, or new NavModel-BDS-KeplerianSet3 may be introduced for B2b. |
| GNSS-RealTimeIntegrity | No impact |
| GNSS-DataBitAssistance | The data bits for BDS B2b description and reference should be added. |
| GNSS-AcquisitionAssistance | No impact |
| GNSS-Almanac >GNSS-AlmanacList >>GNSS-AlmanacElement >>>AlmanacBDS-ReducedKeplerianSet-r16 >>>AlmanacBDS-MidiKeplerianSet-r16 | Some updated is need for support BDS B2b. |
| AlmanacKeplerianSet | No impact |
| AlmanacNAV-KeplerianSet | No impact |
| AlmanacReducedKeplerianSet | No impact |
| AlmanacMidiAlmanacSet | No impact |
| AlmanacGLONASS-AlmanacSet | No impact |
| AlmanacECEF-SBAS-AlmanacSet | No impact |
| AlmanacBDS-AlmanacSet | No impact |
| AlmanacBDS-ReducedKeplerianSet | AlmanacBDS-ReducedKeplerianSet is needed to support BDS B2b |
| AlmanacBDS-MidiKeplerianSet | AlmanacBDS-MidiKeplerianSet is needed to support BDS B2b |
| GNSS-UTC-Model | New utcModel is added or reuse utcModel-2 for B2b |
| UTC-ModelSet1 | No impact |
| UTC-ModelSet2 | May be impacted |
| UTC-ModelSet3 | No impact |
| UTC-ModelSet4 | No impact |
| UTC-ModelSet5 | No impact |
| GNSS-AuxiliaryInformation | No impact |
| BDS-DifferentialCorrections | No impact |
| BDS-GridModelParameter | No impact |
| GNSS-RTK-Observations | No impact |
| GLO-RTK-BiasInformation | No impact |
| GNSS-RTK-MAC-CorrectionDifferences | No impact |
| GNSS-RTK-Residuals | No impact |
| GNSS-RTK-FKP-Gradients | No impact |
| GNSS-SSR-OrbitCorrections | May be impacted |
| GNSS-SSR-ClockCorrections | May be impacted |
| GNSS-SSR-CodeBias | No impact |

For the impact of GNSS Assistance Data Request and GNSS Assistance Data Request Elements, as listed below, depends on whether new model (e.g., Ionospheric Model, almanac model etc.) is introduced for BDS B2b signal.

- GNSS Assistance Data Request Elements

| Data Elements and field | Impact description |
| --- | --- |
| GNSS-ReferenceTimeReq | No impact |
| GNSS-ReferenceLocationReq | No impact |
| GNSS-IonosphericModelReq | May be impacted, if new IonosphericModel is introduced for BDS B2b |
| GNSS-EarthOrientationParametersReq | No impact |
| GNSS-RTK-ReferenceStationInfoReq | No impact |
| GNSS-RTK-AuxiliaryStationDataReq | No impact |
| GNSS-TimeModelListReq | No impact |
| GNSS-DifferentialCorrectionsReq | No impact |
| GNSS-NavigationModelReq | No impact |
| GNSS-RealTimeIntegrityReq | No impact |
| GNSS-DataBitAssistanceReq | No impact |
| GNSS-AcquisitionAssistanceReq | No impact |
| GNSS-AlmanacReq | May be impacted, if new modelID is introduced for BDS B2b |
| GNSS-UTC-ModelReq | May be impacted |
| GNSS-AuxiliaryInformationReq | No impact |
| BDS-DifferentialCorrectionsReq | No impact |
| BDS-GridModelReq | No impact |
| GNSS-RTK-ObservationsReq | No impact |
| GLO-RTK-BiasInformationReq | No impact |
| GNSS-RTK-MAC-CorrectionDifferencesReq | No impact |
| GNSS-RTK-ResidualsReq | No impact |
| GNSS-RTK-FKP-GradientsReq | No impact |
| GNSS-SSR-OrbitCorrectionsReq | No impact |
| GNSS-SSR-ClockCorrectionsReq | No impact |
| GNSS-SSR-CodeBiasReq | No impact |

For GNSS Capability Information and GNSS Capability Information Element, similar to GNSS Assistance Data Request Element, whether new capability fields need to be added depends on whether new model is introduced for BDS B2b signal.

For common GNSS information elements, at least BDS B2b signal ID should be added.

Observation 1: In order to support the BDS B2b in A-GNSS, at least the GNSS Assistance Data Elements and Common GNSS Information Elements should be updated.

Observation 2: Whether the GNSS Assistance Data Request Elements and the GNSS Capability Information elements will be impacted depends on whether new model for BDS B2b signal (e.g., new clock Model, Ionospheric Mode, etc.) is introduced.

For GNSS Location Information, GNSS Location Information Request Elements and GNSS Error elements, there is no B2b signal specific information, so there is no impact on it.

Observation 3: There is no impact on GNSS Location Information and GNSS Error Elements.

Based on the above analysis and observations, we propose:

Proposal 1: RAN2 considers the impact of introducing BDS B2b signal on the following signaling:

- GNSS Assistance Data Elements

- GNSS Assistance Data Request Elements

- GNSS Capability Information Elements

- Common GNSS Information Elements

In addition, according to WID [1], some B2b specific assistant information will be introduced. Based on the analysis related to GNSS Assistance Data elements above, in order to support BDS B2b signals in A-GNSS, at least the B2b specific parameters including ionospheric model, clock model, orbit model, almanac model, UTC model and data bit assistance information are required, and those assistant data needs be updated on top of current specification.

Proposal 2: The impact of assistant information for BDS B2b is mainly involves following:

- Ionospheric Model

- Clock Model

- Orbit Model

- Almanac Model

- UTC Model

- Data Bit Assistance

FFS other assistant information for BDS B2b.

# 3. Conclusion

In this contribution we discussed impact related to BDS B2b signal and made the following proposals.

Observation 1: In order to support the BDS B2b in A-GNSS, at least the GNSS Assistance Data Elements and Common GNSS Information Elements should be updated.

Observation 2: Whether the GNSS Assistance Data Request Elements and the GNSS Capability Information elements will be impacted depends on whether new model for BDS B2b signal (e.g., new clock Model) is introduced.

Observation 3: There is no impact on GNSS Location Information and GNSS Error Elements.

Proposal 1: RAN2 considers the impact of introducing BDS B2b signal on the following signaling:

- GNSS Assistance Data Elements

- GNSS Assistance Data Request Elements

- GNSS Capability Information Elements

- Common GNSS Information Elements

Proposal 2: The impact of assistant information for BDS B2b is mainly involves following:

- Ionospheric Model

- Clock Model

- Orbit Model

- Almanac Model

- UTC Model

- Data Bit Assistance

FFS other assistant information for BDS B2b.

# 4. References

- RP-242413, New WID on Introduction of BDS B2b Signal in A-GNSS

- BeiDou Navigation Satellite System (BDS) Signal In Space (SIS) for Open Service Signal B2b was published by the China Satellite Navigation Office in July, 2020, http://www.beidou.gov.cn/xt/gfxz/202008/P020230516558683155109.pdf