3GPP TSG-RAN WG2 Meeting #127bis	R2-2408669

Hefei, China, 14 – 18 October 2024

Agenda item:	4.1

Source:	Nokia

Title:	UE capability constraints for Qoffsettemp

WID/SID:	TEI12 - Release 12

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss the problem of the minimum number of cells for which the UE shall be able to apply Qoffsettemp in case the UE supports the feature RRC Connection Establishment Failure Temporary Qoffset defined in 36.306 subclause 6.7.2 and/or RRC connection establishment failure with temporary offset defined in 38.306 subclause 5.3.

# 2	Discussion

## 2.1	“Chiba” issue

Qoffsettemp handling was introduced to LTE in Rel-12 by R2-142953 36.304 CR 0238 [1] with the following reason:

| Reason for change: | In some deployments, a stationary or slow moving UE may camp on the wrong cell due to artificially strong downlink signal, and PRACH transmissions will fail to reach the NW resulting in the UE being unable to establish a connection. |
| --- | --- |
| Summary of change: | An offset is temporarily applied to the cell on which a configurable number of RRC Connection Establishment failure was detected for cell selection and reselection evaluation. […] |

In the discussions preceding the R2-142953 agreement (R2-1805233 [2] gives a summary with references to previous TDocs), there is a reoccurring topic of Qoffsettemp usage to overcome the so-called "chiba” issue which is typical for large distance cells deployed close to water, where reflected DL signals make the signal measurements in RRC_IDLE and RRC_INACTIVE modes appear artificially high.

Although the above-mentioned discussions concentrate on a single "chiba" cell, this does not preclude areas of overlapping coverage of multiple "chiba" cells, especially in the case of multiple low frequency channels and/or diversified coastline with bays, fjords, islands, peninsulas, etc.

Observation 1: The usage of Qoffsettemp is not limited to a single ”chiba” cell. Scenarios with multiple ”chiba” cells are possible.

## 2.2	“Non-chiba” usage scenarios

Although in the discussions, the only usage example of Qoffsettemp is the "chiba" issue, the normative text for UTRAN, E-UTRAN and NR does not impose any restrictions in this respect. It should therefore be assumed that Qoffsettemp can also be configured in other environments.

Since according to 36.331 subclause 5.3.3.6 and 38.331 subclause 5.3.3.7 the application of Qoffsettemp to a given cell results from multiple RRC Connection Establishment failures (T300 expiry), it is also irrelevant whether the establishment failure occurs due to the DL/UL coverage imbalance characteristic for "chiba" cell or for any other reason that leads to T300 expiry (e,g.: RACH congestion, strong UL interference, network overload, etc.).

Observation 2: The usage of Qoffsettemp is not limited to a ”chiba” cell environment. Other usage scenarios with T300 expiry are not precluded (e.g.: RACH congestion, strong UL interference, network overload, etc.).

## 2.3	UE capability constraints for Qoffsettemp

Currently, the minimum number of cells for which the UE shall be able to apply Qoffsettemp is unspecified. However, given the limited resources of the UE, it would be unreasonable to assume that the UE supports an infinite number of cells with Qoffsettemp (or for example as many as the cell identification range allows).

It is also worth noting that although E-UTRA RRC Connection Establishment Failure Temporary Qoffset and NR RRC connection establishment failure with temporary offset are optional features and both belong to the “optional features without UE radio access capability parameters” category, once it is determined that the feature is supported, the usage of Qoffsettemp is mandatory if all needed conditions in the relevant RRC protocol specification are fulfilled.

Observation 3: Currently, if the feature is supported, the UE shall be able to apply Qoffsettemp to unlimited number of cells which is probably impossible.

The lack of a specified minimum number of supported cells with Qoffsettemp makes it difficult to predict the consequences of configuring this feature in problematic multi-cell environments or using this feature in more complex system scenarios. This in turn may lead to limitations in its use in practice.

Observation 4: The lack of clearly defined UE capability constraints for Qoffsettemp may limit practical applications of the feature due to difficulties in predicting the UE behavior in a multi-cell environment.

In order to solve the above problem, we propose to specify the minimum number of cells for which the UE supporting the feature shall be able to apply Qoffsettemp and define the value of this minimum to 16 cells, separately for each RAT (E-UTRA and NR) and regardless of the UE category (e.g.: ”regular” UE, NB-IoT, CatM, (e)RedCap).

Proposal 1: Specify the minimum number of cells for which the UE supporting Qoffsettemp shall be able to apply Qoffsettemp.

If a decision is made to specify the minimum number of cells, decide from which release the appropriate corrections for E-UTRA and NR should be made.

Proposal 2: RAN2 to decide from which release the appropriate corrections for E-UTRA and NR should be made.

# 3	Conclusion

This document has made the following observations:

Observation 1: The usage of Qoffsettemp is not limited to a single ”chiba” cell. Scenarios with multiple ”chiba” cells are possible.

Observation 2: The usage of Qoffsettemp is not limited to a ”chiba” cell environment. Other usage scenarios with T300 expiry are not precluded (e.g.: RACH congestion, strong UL interference, network overload, etc.).

Observation 3: Currently, if the feature is supported, the UE shall be able to apply Qoffsettemp to unlimited number of cells which is probably impossible.

Proposal 1: Specify the minimum number of cells for which the UE supporting Qoffsettemp shall be able to apply Qoffsettemp.

Proposal 2: RAN2 to decide from which release the appropriate corrections for E-UTRA and NR should be made.

# 4	References

[1]	R2-142953 36.304 CR 0238 “Introduction of RRC Connection Establishment failure temporary Qoffset”, Sony, NTT DoCoMo, Deutsche Telekom, Broadcom Corporation, MediaTek Inc., Huawei, HiSilicon, Samsung, Fujitsu, NEC

[2] 	R2-1805233 “RRC Connection Establishment failure temporary Qoffset handling (AKA Chiba Issue) in NR”, Huawei, HiSilicon, Deutsche Telekom, NTT DoCoMo