# 3GPP TSG-RAN WG2 Meeting #127bis		                 							  R2-2408671

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.2.2

Source:	Robert Bosch GmbH

Title:	Discussions on Energy Status Report in Ambient IoT Devices

Document for:	Discussion and Decision

# Introduction

In the last meeting, RAN2 discussed the ambient IoT functionality aspects and reached the following agreement [1]:

Agreements

RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling.

# Energy Status Report in Ambient IoT Devices

All different types of Ambient IoT (AIoT) devices (Type 1, 2a, 2b) have limitations on power consumption and energy storage. Therefore, energy is a scarce resource for all types of ambient IoT devices which should be consumed in an efficient manner. For the D2R transmissions, a device should have enough energy for a successful transmission. Other than, the UL transmission from a device to a reader can be failed due to the lack of enough energy. When a device is triggered by the network for D2R transmission, it should participate in a contention in the random access procedure and access the channel for the data transmission. Then, the reader may also schedule time slots for the device for its data transmission and the device can use the scheduled resources for its transmission. For all these steps, the device needs to have enough energy.

Observation 1: A successful D2R data transmission is highly dependent on the remaining energy level of an ambient IoT device.

To allocate resources to an AIoT device, the reader may need to know about the energy status of a device. In case of a low energy level in a device, the resource allocation to the device will lead in the wastage of radio resources.

Observation 2: The energy status of an AIoT device should be known to a reader for an efficient resource allocation and avoiding the wastage of radio resources.

Proposal 1: RAN2 to study an energy status report procedure for ambient IoT devices.

In a simplest case, the device can send an indication to the reader about the lack of enough energy for the next D2R transmission. In this case, the reader may avoid the unnecessary allocation of resources to the device.

Proposal 2: A single indication from an AIoT device to the reader to indicate the lack of enough energy for the next D2R transmission.

If the device is capable of energy harvesting, the resource allocation for the device should also consider the energy harvesting procedure and may need to know the type of energy harvesting to estimate the time needed for a device to reach a required level of energy.

Observation 3: Energy harvesting in a device in general and its type can have an impact on the resource allocation for an AIoT device.

Proposal 3: A device may encode n bits in its energy status report for energy harvesting status and its type.

Proposal 4: RAN2 to study the definition of energy status which may include for e.g., the remaining energy level, the number possible messages to transmit, capability of energy harvesting, and its type and the time needed for energy harvesting.

The energy status report including the energy harvesting status or a single indication from the AIoT device to the reader node can be transmitted before the start of the random access procedure or through messages which are exchanged during random access procedure that are msg1 or msg3.

Proposal 5: RAN2 to study the possible options for the transmission of energy status of a device including its energy harvesting status.

A reader or base station can configure an AIoT device to report its energy status to be employed in the resource allocation. A periodic energy status report from AIoT devices can give the reader a useful insight about the energy status of a device but it may impose a high overhead on the network resources. Therefore, AIoT devices may conditionally report their energy status to avoid wastage of resources.

Observation 4: A periodic energy status report by AIoT devices may impose a high overhead on network resources.

Proposal 6: AIoT devices may report their energy status including their energy harvesting type and status, if a set of (pre-)configured conditions are met.

Proposal 7: A reader or base station may configure the set of conditions to be considered in an AIoT device for reporting its energy status.

# Conclusions

Based on the discussion in the contribution, we have the following observations and proposals:

Observation 1: A successful D2R data transmission is highly dependent on the remaining energy level of an ambient IoT device.

Observation 2: The energy status of an AIoT device should be known to a reader for an efficient resource allocation and avoiding the wastage of radio resources.

Observation 3: Energy harvesting in a device in general and its type can have an impact on the resource allocation for an AIoT device.

Observation 4: A periodic energy status report by AIoT devices may impose a high overhead on network resources.

Proposal 1: RAN2 to study an energy status report procedure for ambient IoT devices.

Proposal 2: A single indication from an AIoT device to the reader to indicate the lack of enough energy for the next D2R transmission.

Proposal 3: A device may encode n bits in its energy status report for energy harvesting status and its type.

Proposal 4: RAN2 to study the definition of energy status which may include for e.g., the remaining energy level, the number possible messages to transmit, capability of energy harvesting, and its type and the time needed for energy harvesting.

Proposal 5: RAN2 to study the possible options for the transmission of energy status of a device including its energy harvesting status.

Proposal 6: AIoT devices may report their energy status including their energy harvesting type and status, if a set of (pre-)configured conditions are met.

Proposal 7: A reader or base station may configure the set of conditions to be considered in an AIoT device for reporting its energy status.

# References

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #127”, August 2024.