3GPP TSG-RAN WG2#127bis												R2-2408675

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.9.2 Support of Store & Forward

Source: 	NEC

Title:	        Radio Interface Aspect of Store and Forward

Document for:	 Discussion

# Introduction

So far we have achieved some agreements to clarify scope ,scenario and architecture regarding S&F:

- RAN2 will adopt SA2 agreed full CN and spit-MME payload options as the baseline for further discussion.

- S&F feature is common for NB-IOT and eMTC,

- S&F feature is applicable to both CP and UP solutions

- consider both single satellite pass and multiple satellite pass scenarios

- consider both MO and MT data within scope

Besides, we made following agreements regarding barring and general procedures at RAN2#126 and RAN2#127:

Agreements at RAN2#126: 1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available) 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed. 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN. Agreement at RAN2#127 4. UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) 5. RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. 7. RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

Based on all above agreements, we share our view on UE AS behaviour, procedure and barring aspects from radio interface point of view.

# Discussion

## Scenario specific issues

SA2 agree split MME and full CN on-board architecture:

- With Split MME on board

Part of MME function will be on satellite, and other part of MME will be on ground.

- With full CN on board

The whole CN will be on board, this may be beneficial NAS procedures, e.g., NAS procedure like attach/TAU procedure might be able to complete during the coverage of one satellite and during one round of RRC connection.

Regardless split MME or full CN architecture, there is no continuous full connection from UE to the data network. i.e. there is no end to end the data transmission connection. Due to lack of end to end data transmission connection, a MO data would need to be sent and stored at satellite when UE firstly connects to a eNB, the responding data from peers would need to be transmitted back to UE when UE secondly connects to the same or a different eNB. A UE connect to satellite for S&F service would inevitably need to reconnect to the same or different satellite to continue the data transmission.

In case of UP-CIOT procedure, the data transmission will rely on stored UE context at UE and network side. if the UE connects to a satellite different from the previous/anchor satellite, context retrieving is needed. Without available feeder link, this will cause further complication and delay. For simplicity, it is better that the UE connects to the same eNB/satellite where its UE context was suspended.

Because SA2 discussed the possibility that MME provides the UE with a list of satellites IDs, during attach/TAU. The UE may uses the satellites in the list for MO/MT data/signalling transfer with the CN. RAN2 could continue discuss AS aspect on how to use a recommended satellite/set of satellite for camping and RRC connection establishment.

RAN2 discuss how UE should be encouraged to camp/reconnect to the same satellite/satellite sets (provided by upper layer) at least when there is unfinished store and forward data/signalling transmission.

In WID justification, it is mentioned the benefits associated with the support of Store&Forward (S&F) Satellite operation:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

From above justification, a common scenario would be

- From gateway and core network perspective, there is no continuous feeder link availability.

- from UE /service link perspective, There is likely no enough satellite/cells to provide a continuous coverage, which means UE/NW supporting store and forward will experience discontinuous coverage .

Service link becoming unavailable may lead to either cell change (if there is neighbour cell) or discontinuous coverage behaviour (if there is no neighbour cell).

Considering the cases when there is no neighbour cell, for UE in RRC_IDLE, the network may provide information for operating during discontinuous coverage (e.g., ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage). And UE shall use these discontinuous coverage information to predict its in and out of coverage time window. If UE is out of coverage, it could suspend all AS idle procedure for power saving.

For UE in RRC_CONNECTED, we expect NW will release the UE connection before it is out of coverage. Otherwise, upon declaring RLF, UE may determine not to reestablishment the RRC connection by implementation (e.g., based on ephemeris parameters).

UE(s) who support Store&Forward feature, support discontinuous coverage-like feature upon service link becoming unavailable and there is no neighbor cell:

- For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations related to NTN.

- For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

## Barring and S&F indication

We agreed:

- RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN. - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

When a satellite is circling around providing S&F service, it can be assumed that the service link is always on. From time to time, the feeder link connection with GW on ground will be established for a short time widow. During this short otime window, both service link and feeder link will be available, the cell could operate in normal rather than in S&F mode. Based on the above cited agreement, NW should indicate “In S&F operating” while feeder link is not available) and indicate “off S&F operating” (while feeder link is available). It can mean to set a one bit IE with TRUE and FALSE value.

On the other hand, it depends on how long the feeder link will be available, and whether some UEs may benefit from the time window of full connection, it is up to NW implementation to decide if it worth to change the indicator from TURE to FALSE  or keep the value TURE when feeder link is available. Also this update will lead to system information modification procedure unless we agree another way. This is another aspect the NW should consider if it worth to update the value.

RAN2 confirm that S&F operation mode indication is a 1bit IE and can be set to TURE (while feeder link is not available) or FALSE(while feeder link is available). It is up to network implementation whether to update S&F operation mode indication bit value to FALSE(while feeder link is available), or to keep the TURE value all the time

It is noted that for UE not support S&F feature including Rel19 and legacy UE, they will not read the S&F indicator.  they will be barred by legacy cellBarred and cellBarredNTN, regardless of the value of S&F mode. for UE support S&F feature, they will read the S&F indicator, either the value is set to TURE or FALSE, UE supporting S&F feature will be able to access the cell, in order bar UE supporting S&F feature, we propose to use the absence of the indicator.

Extra barring bits for Rel-19 UE supporting S&F is not needed. UE supporting S&F feature will be barred if S&F indicator is absent (potentially also cellBarred and cellBarredNTN set to barred)

## Basic procedure over radio interface

In general, S&F data transmission will experience two stages:

- When service link is available while feeder link is unavailable, UL/DL data transmission happens over Uu interface. this should be discussed in RAN2, taking into account which part of CN is onboard.

- When feeder link is available while service link is unavailable, UL/DL data transmission happens over Nx interface. What is Nx depending on which part of CN is on board, but Nx interface procedure is obviously out of RAN2 scope and should be discussed in other WG (e.g., RAN3 or SA2) .

RAN2 discussion focus on Uu procedure when service link is available but feeder link is not available.

We agreed the basic procedure for MO as follows RAN2#126, later in RAN2#127, we agree to consider both MO and MT data :

“For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it; 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN; 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available); 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again”

When service link is available but feeder link is not available, there might be either uplink data/signalling at UE side or downlink data/signalling at satellite side waiting for transmission. Fortunately, MT can reuse MO procedure apart from additional paging step.

Hence we could revise the agreement on MO procedure above to cover both MO and MT, and focus on Uu interface only. Following steps could be taken as baseline for S&F satellite operation:

- the eNB send paging to UE if there is stored data/signalling at the eNB for the UE (FFS is a RAN initiated  paging or CN initiated paging).

- the UE establish RRC connection with the eNB

- The UE sends uplink data/signalling to the eNB for storing and forwarding

- the eNB forward the stored downlink data/signalling to the UE

- the eNB release the RRC connection

This MO/MT data transmission procedure

Figure 2.2-1: Example of procedure when service link is available

RAN2 revisit the MO procedure agreed in RAN2#126, and agree following legacy procedure as starting point for further discussion. (covering both MO and MT UL/DL data transmission):

- the eNB send paging to UE if there is stored data/signalling at the eNB/satellite for the UE.

- the UE establish RRC connection with the eNB

- the UE sends uplink data/signalling to the eNB for storing and forwarding

- the eNB forward the stored downlink data/signalling to the UE

- the eNB release the RRC connection

# Conclusion

In summary, we would like to share our view on S&F as follows:

- Service link becoming unavailable may lead to either cell change (if there is neighbour cell) or discontinuous coverage behaviour (if there is no neighbour cell).

Proposal 1:	RAN2 discuss how UE should be encouraged to camp/reconnect to the same satellite/satellite sets (provided by upper layer) at least when there is unfinished store and forward data/signalling transmission.

Proposal 2:	UE(s) who support Store&Forward feature, support discontinuous coverage-like feature upon service link becoming unavailable and there is no neighbor cell:

-	For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations related to NTN.

-	For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

Proposal 3:	RAN2 confirm that S&F operation mode indication is a 1bit IE and can be set to TURE (while feeder link is not available) or FALSE(while feeder link is available). It is up to network implementation whether to update S&F operation mode indication bit value to FALSE(while feeder link is available), or to keep the TURE value all the time

Proposal 4:	Extra barring bits for Rel-19 UE supporting S&F is not needed. UE supporting S&F feature will be barred if S&F indicator is absent (potentially also cellBarred and cellBarredNTN set to barred)

Proposal 5:	RAN2 discussion focus on Uu procedure when service link is available but feeder link is not available.

Proposal 6:	RAN2 revisit the MO procedure agreed in RAN2#126, and agree following legacy procedure as starting point for further discussion. (covering both MO and MT UL/DL data transmission):

1)	the eNB send paging to UE if there is stored data/signalling at the eNB/satellite for the UE.

2)	the UE establish RRC connection with the eNB

3)	the UE sends uplink data/signalling to the eNB for storing and forwarding

4)	the eNB forward the stored downlink data/signalling to the UE

5)	the eNB release the RRC connection

# References

[1] R2-24xxxxx, Report of 3GPP TSG RAN WG2 meeting #127, ETSI MCC