3GPP TSG RAN WG2 Meeting #127bis                                                          R2-2408677

# Hefei, China, October 14-18, 2024

Agenda item:		8.5.3

Source:			III

Title:		On-demand SIB1 for Idle/Inactive mode UEs

Document for:	 	Discussion

- Introduction

A Rel-19 work item on network energy saving enhancements was approved in RAN#102 and continuously revised in RAN#103, RAN#104 and RAN#105, whose up-to-date objectives [1] are listed as follows.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

For the second issue, i.e., on-demand SIB1 for UEs in idle/inactive mode, it used to be in study phase of the work item, and was approved as normative work at RAN#105, which in RAN2’s scope is mainly based on the agreements [2] made in previous RAN2#127, including:

Agreements on OD-SIB1: Scenarios: 1. No consensus for RAN1 case 3 in RAN2. OD-SIB1 Validity: 2. We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell). 3. Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement). WUS configuration of NES cell from NES cell: 4. Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures. MSG3 based OD-SIB1 REQ: 5. Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell). Access restriction for legacy UEs: 6. Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells. Impacts to RRC connected UEs: 7. RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell). OD-SIB1 SI conclusion: 8. RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

This contribution provides some input on WUS configuration of NES cell including its identification, provisioning and validity (or synchronization) for further discussions.

- Discussion

Cases studied by WGs

In [3], it concludes 3 cases studied by WGs, and they are:

- Case 1: UE obtains UL WUS configuration from NES Cell, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

- Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell

- Case 3: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on Cell A, UE receives on-demand SIB1 from Cell A

It is agreed that Case 2 (Option 1+B+X) is the only normative work for on-demand SIB1 in Rel-19. For UL WUS configuration, it is initially obtained from Cell A and then used by Rel-19 NES UEs to camp on an NES cell. Later, it might be updated on NES cell itself and then conveyed to Cell A. In this regard, it might create a time lag between retrieving and using UL WUS configuration for some Rel-19 NES UEs at certain time instances.

Also for access restriction for legacy UEs, one fundamental problem is: how to recognize an NES cell (with on-demand SIB1), and this leads to the identification of an NES cell, which has not been fully discussed yet.

Identification of an NES cell

In section 6.2 of [4], the structure of MIB (aka PBCH payload) is illustrated, where only 1 bit (spare) is reserved. Therefore, it is very challenging to utilize the bit as the identification of an NES cell without any impact on legacy systems.

MIB ::= SEQUENCE {

systemFrameNumber BIT STRING (SIZE (6)), 6bit

subCarrierSpacingCommon ENUMERATED {scs15or60, scs30or120}, 1bit

ssb-SubcarrierOffset INTEGER (0..15), 4bit

dmrs-TypeA-Position ENUMERATED {pos2, pos3}, 1bit

pdcch-ConfigSIB1 INTEGER (0..255), 8bit

cellBarred ENUMERATED {barred, notBarred}, 1bit

intraFreqReselection ENUMERATED {allowed, notAllowed}, 1bit

spare BIT STRING (SIZE (1)), 1bit

}

Alternatively, ssb-SubcarrierOffset may also be used to indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB. In this case, the field pdcch-ConfigSIB1 indicates the frequency positions where the UE may find SSB with SIB1 or the frequency range where the network does not provide SSB with SIB1. The UE determines from MIB that a CORESET#0 is not present if kSSB > 23 for FR1 or if kSSB > 11 for FR2. However, the impact brought about by such identification needs to be further studied.

Observation 1. Identification of NES cell with on-demand SIB1 by PBCH payload of NES cell may bring much impact on legacy systems, and needs to be further studied.

Proposal 1. Identification of NES cell with on-demand SIB1 by WUS configuration introduces less impact on legacy systems and is preferred.

Provisioning of WUS configuration of an NES cell

In the section 5.2 of [4], it is stated that the physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2,976 bits. Therefore, if Cell A’s existing SIB1 is used to configure on-demand SIB1 related configuration for neighbor NES cells, a size problem might arise, especially when the same Cell A carries such information of more than one NES Cells.

Observation 2. If Cell A’s existing SIB1 is used to configure on-demand SIB1 related configuration for neighbor NES cells, a size problem might arise, especially when the same Cell A carries such information of more than one NES Cells.

Proposal 2. When Cell A is used to configure on-demand SIB1 related configuration for neighbor NES cells, a new SIB is preferable and needs to be further studied.

Validity of WUS configuration of an NES cell

Once a Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, which may follows the procedures of SI change indication specified in the section 5.2 of [4]. Such updates need to be conveyed to Cell A as soon as possible; otherwise, the time lag of WUS configuration updates between Cell A and NES cell might causes problems for those Rel-19 NES UEs which already obtain the configuration from Cell A and are about to camp on the NES cell.

Observation 3. If WUS configuration of an NES cell is updated, it might cause a time lag between Cell A and NES cell in Case 2.

In the section 7.3.3 of [5], it is stated that change of system information only occurs at specific radio frames, i.e. the concept of a modification period is used. When the network changes (some of the) system information, it first notifies the UEs about this change, i.e. this may be done throughout a modification period. In the next modification period, the network transmits the updated system information. Upon receiving a change notification, the UE acquires the new system information from the start of the next modification period. The UE applies the previously acquired system information until the UE acquires the new system information.

UL WUS configuration transmission from NES Cell to Cell A and SI modification on NES cell should collaborate to mitigate the problems caused by the time lag of WUS configuration between Cell A and NES cell.

Proposal 3. Mechanism considering UL WUS configuration transmission from NES Cell to Cell A and SI modification on NES cell needs to be studied to mitigate the problems caused by the time lag of WUS configuration updates between Cell A and NES cell.

- Conclusion

In this contribution, we have the following observations and proposals:

Observation 1. Identification of NES cell with on-demand SIB1 by PBCH payload of NES cell may bring much impact on legacy systems, and needs to be further studied.

Proposal 1. Identification of NES cell with on-demand SIB1 by WUS configuration introduces less impact on legacy systems and is preferred.

Observation 2. If Cell A’s existing SIB is used to configure on-demand SIB1 related configuration for neighbor NES cells, a size problem might arise, especially when the same Cell A assists more than one NES Cells.

Proposal 2. When Cell A is used to configure on-demand SIB1 related configuration for neighbor NES cells, new SIB is preferable and needs to be further studied.

Observation 3. If WUS configuration of an NES cell is updated, it might cause a time lag between Cell A and NES cell in Case 2.

Proposal 3. Mechanism considering UL WUS configuration transmission from NES Cell to Cell A and SI modification on NES cell needs to be studied to mitigate the problems caused by the time lag of WUS configuration updates between Cell A and NES cell.

References

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR.

[2] RP-241866 Status report for WI Enhancements of network energy savings for NR.

[3] RP-242384 Way forward on NES checkpoint.

[4] 3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[5] 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".