3GPP TSG RAN WG2 Meeting #127bis                                                            R2-2408679

# Hefei, China, October 14-18, 2024

Agenda item:		8.3.3

Source:			III

Title:		Discussion on measurement event predictions

Document for:	 	Discussion

- Introduction

A Rel-19 study item on AI/ML mobility enhancements was approved in RAN#102 and revised in RAN#103 and RAN#105 respectively, whose up-to-date objectives [1] are listed as follows.

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: • AI/ML based RRM measurement and event prediction, • Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] • Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] • HO failure/RLF prediction (UE sided model) [RAN2] • Measurement events prediction (UE sided model) [RAN2] • Study the need/benefits of any other UE assistance information for the network side model [RAN2] • The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] • NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 • Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] • NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID • Potential specification impacts of AI/ML aided mobility [RAN2] • Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] • Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed • Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) • NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. • NOTE 5: Avoid the overlaps with RAN2 work for evaluation • NOTE 1: RAN1/3 work can be triggered via LS • NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 • NOTE 3: Two-sided model is not included

For measurement event predictions, the latest agreements [2] were made in RAN2#125bis, including:

Agreements:

1. At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed.

2. Clarifications on what is being as input should be provided with results.

3. Start with A3 as a baseline.

4. Measurement event prediction study can start after some further progress on RRM measurement prediction has been made.

Progress in RRM measurement prediction (AI 8.3.2) and HO failure/RLF prediction (AI 8.3.4) has been made in recent RAN2 meetings, and therefore the agreements [3] related to measurement event prediction in RAN2#127 are listed below for clarifying the use cases of measurement event prediction.

Agreement on simulation results of RRM measurement prediction:

1. Companies are encouraged to considers both prediction from low-frequency cell to high-frequency cell and prediction from high-frequency cell to low-frequency cell, but only low to high is expected.

2. For the agreed frequencies for inter-frequency case, only one UE speed is considered for inter-frequency prediction in simulation, e.g., 30km/h.  Companies can consider other speeds for other frequencies if they chose to simulate them

3. For temporal domain case B prediction, the input is historical measurement values and the output is the values at the subsequent time instances that measurement is skipped, i.e., the prediction is always after the measurement and is at future time instance(s).

Agreement on RLF prediction:

1. Both direct and indirect are allowed.  Companies should indicate what they used and what inputs they are using.

- Output for direct: probability of RLF within an window

- Output for indirect: predicted SINR. Based on predicted SINR, the time instance the RLF occurs can be determined without further AI/ML models

2. Companies should report the prediction time window they have used in the simulations.

This contribution provides some input regarding measurement event prediction for further discussion.

- Discussion

According to the section 9.2.3.2 of [4], it is illustrated in the Figure 1 that a complete handover operation is composed of 3 steps: Handover Preparation, Handover Execution and Handover Completion, where the main responsibilities of each step are also depicted. Same as RRM measurement prediction, RRM measurement event prediction is expected to take effect in the Handover Preparation step.

Figure 1: 3 steps in a complete handover operation

Take legacy Event A3 for example. It is triggered when a neighbor cell becomes better than a special cell (SpCell) by an offset, where the measurement report is based on UE-sided measured (signal) facts. From the viewpoint of measurement event predictions, it is to explore the possibility that the UE sends the measurement report prior to the actual occurrence of the event (in the Handover Preparation step), thereby giving the network additional time to prepare the target node/cell and send the HO command (originally in the Handover Execution step) in advance.

Similar to RRM measurement prediction, if measurement event could be successfully predicted by utilizing an AI/ML model taking current and historical measurements of the serving cell and neighbor cells as input, it will have the benefit of early access to the overall operations of a prediction-based handover.

Observation 1. Taking current and historical measurements of the serving cell and neighbor cells as input, an AI/ML model can be utilized to assist in measurement event prediction, whose behaviour is very similar to that of RRM measurement prediction.

Like the issues of direct and indirect RLF prediction being discussed in HO failure/RLF prediction (AI 8.3.4), they will be discussed in measurement event prediction as well. When any L3 RSRP measurement results, be they actual or predicted, are reported to the serving cell, current event measurement mechanism will be indiscriminately triggered to deal with the measurement results. In this regard, if the results of RRM measurement prediction are available, they can be readily re-used for measurement event prediction as indirect prediction; otherwise, current and historical measurements are to be used to perform measurement event prediction as direct prediction.

Observation 2. If the results of RRM measurement prediction are available, they can be readily re-used for measurement event prediction as indirect prediction; otherwise, current and historical measurements are to be used to perform measurement event prediction as direct prediction.

Proposal 1. To distinguish the input of L3 RSRP measurement results as actual or predicted, new reporting mechanisms are to be further studied.

However, every prediction comes with the risk of prediction inaccuracy. When a prediction fails, some operations performed in advance might prove to be invalid and need to be aborted. Therefore, some fallback procedures are required to ensure the in-progress operations of handover with less system impact.

Observation 3. If measurement event prediction fails, some fallback procedures are required to ensure the in-progress operations of handover with less system impact.

Proposal 2. Determining whether a measurement event prediction fails needs to be further studied.

Proposal 3. Fallback procedures to ensure the in-progress operations of handover with less system impact needs to be further studied.

- Conclusion

In this contribution, we have the following observations and proposals:

Observation 1. Taking current and historical measurements of the serving cell and neighbor cells as input, an AI/ML model can be utilized to assist in measurement event prediction, whose behaviour is very similar to that of RRM measurement prediction.

Observation 2. If the results of RRM measurement prediction are available, they can be readily re-used for measurement event prediction as indirect prediction; otherwise, current and historical measurements are to be used to perform measurement event prediction as direct prediction.

Proposal 1. To distinguish the input of L3 RSRP measurement results as actual or predicted, new mechanisms are to be further studied.

Observation 3. If measurement event prediction fails, some fallback procedures are required to ensure the in-progress operations of handover with less system impact.

Proposal 2. Determining whether a measurement event prediction fails needs to be further studied.

Proposal 3. Fallback procedures to ensure the in-progress operations of handover with less system impact needs to be further studied.

References

[1] RP-242393 Revised SID on AIML for mobility in NR.

[2] R2-2404102 Report of 3GPP TSG RAN WG2 meeting #125bis.

[3] RP-241744 Status report for SI Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[4] 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".