3GPP TSG-RAN WG2 Meeting #127bis	R2-2408682

Hefei, China, Oct 14th – 18th, 2024

Source: 	ITRI

Title:  	The Rel-19 ID for subsequent inter-CU LTM

Agenda Item:	8.6.2 Inter-CU LTM

Document for:	Discussion and decision

Introduction

In RAN plenary #105, a revised Work Item on NR mobility enhancements Phase 4 was approved in RP-242356 [1]. Objective #1 of this Work Item is to specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM).

As described in the Work Item, the Rel-18 introduced LTM can offer improvements in handover latency and interruption time compared to Layer 3 based mobility. However, its operation is only supported for mobility between cells of the same gNB (same CU). Depending on the deployment of the network this may significantly limit the opportunities to use LTM. To gain the benefits of LTM for a far greater number of handovers, Objective #1 of Rel-19 NR mobility enhancements is to enable LTM operation between cells of different gNBs (i.e. inter-CU). The content of Objective #1 is listed below.

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

Discussion

In the previous RAN2 meeting, we reached an agreement that a new Rel-19 ID will be introduced in RRC as baseline and if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update [2].

RAN2#127 Agreement:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

As shown in the first section, support for subsequent LTM mobility procedure is listed within Objective #1 aiming at inter-CU LTM. In addition, the following agreements on Rel-19 inter-CU LTM were made in RAN2#125bis meeting [3]:

RAN2#125bis Agreements:

- Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

- UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Based on Objective #1, Rel-19 inter-CU LTM should support subsequent LTM mobility procedures, where RRC configuration between cell switches is avoided. That is, the RRC configuration only occurs once, i.e. after the LTM candidate cell preparation and before the first LTM cell switch. According to the RAN2#125bis agreements shown above, the RRC configuration can contain mixture of intra-CU and inter-CU LTM candidate cells. An inter-CU LTM cell switch requires PDCP re-establishment, including security key update, while an intra-CU LTM cell switch does not. In addition, RAN2 agreed in the previous meeting that a new Rel-19 ID in the RRC configuration can be used to differentiate the types of cell switches, i.e. whether PDCP re-establishment, including security key update, is required. In other words, the Rel-19 ID can be used to distinguish between cells belonging to the same CU and a different CU. For example, in the LTM RRC configuration, each LTM candidate target cell has its associated Rel-19 ID and the serving cell does as well. The cell(s) belonging to the same CU are associated with the same Rel-19 ID, while the cells belonging to different CUs are associated with different Rel-19 IDs. In this way, when a cell switch is triggered, the UE can tell whether the target cell and the serving cell are from the same CU or not by comparing their associated Rel-19 IDs in the RRC configuration.

Proposal 1: The Rel-19 ID can be used to distinguish between cells belonging to the same CU and a different CU.

Proposal 2: Each LTM candidate target cell has its associated Rel-19 ID and the serving cell does as well.

- The cell(s) belonging to the same CU are associated with the same Rel-19 ID.

- The cells belonging to different CUs are associated with different Rel-19 IDs.

Conclusion

In this contribution, the following proposals are made:

Proposal 1: The Rel-19 ID can be used to distinguish between cells belonging to the same CU and a different CU.

Proposal 2: Each LTM candidate target cell has its associated Rel-19 ID and the serving cell does as well.

- The cell(s) belonging to the same CU are associated with the same Rel-19 ID.

- The cells belonging to different CUs are associated with different Rel-19 IDs.

Reference

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4, RAN#105, September 2024.

- R2-24079xx Draft Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands, 19 - 23 August, 2024.

- R2-2404002 Draft Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China, 15 - 19 April, 2024.