3GPP TSG-RAN WG2 #127-bis	R2-2408687

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.2.5

Source:	Ericsson

Title:	Topology 2 and DO-A for Ambient IoT

Document for:	Discussion/Decision

# Introduction

Topology 2 considerations were discussed in RAN2#127 with some agreements below being made:

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by A-IoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

This contribution discusses in detail some open aspects of A-IoT with a focus on Topology 2 considerations, DO-A assessment.

# 2	Discussion

## 2.1	Topology 2 aspects

### 2.1.1	Protocol stack options

It was discussed in RAN3 to have a common network architecture for both Topology 1 and Topology 2 [2]. In terms of protocol stack, RAN3 already agreed and captured in TR38.769 three stack options to be studied, i.e., RRC based solution, NAS based solution, and UP based solution. RAN2 impact from the three options are different. The differences among the three options mainly depend on which sublayer of the Intermediate UE handles the A-IoT procedure. In other words, it is a matter of how A-IoT communication is visible to serving gNB.

Note, that RAN3 also introduced an Editorial Note stating, that it needs to be studied for all these options it needs to be studied how (AIoT) radio resource control can be enabled.

RAN2 impact from Topology 2 depends on which sublayer of intermediate UE (RRC, NAS, or UP) handles A-IoT communication.

First, as all three options are captured in current TR38.769 by RAN3, it is proposed that RAN2 study all the three protocol stack options rather than discussing how to down-select the options in current phase of study.

RAN2 to study impact of all three protocol stack options, i.e., RRC based, NAS based and UP based solution for Topology 2 without down-selection, and capture outcome in the TR.

Excerpt from TR 38.769 §6.4.2.1:

- RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed by gNB explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

- NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

- UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

For UP based solution, the A-IoT data is delivered over PDU session user plane of the Intermediate UE, which are transparent towards NG-RAN. That is, relying on the exchange of A-IoT application level data at inventory and command alone does not allow the NG-RAN to control A-IoT radio resources. This is especially true if one considers a setup of A-IoT RAN as a network of multiple readers (irrespective of whether topology 1 or 2 is deployed) covering a large geographical area where AIoT service is provided, which calls for central point of coordination at the AIoT enabled gNB.

The observation above is also applicable to the NAS based solution where the A-IoT data transmission is only aware by the Intermediate UE and CN-CP (AMF/AIoTF). In both UP based and NAS based solutions, NG-RAN, however, needs to be aware of the A-IoT procedure with relevant control information in order to control radio resources used by A-IoT. The control information can be provided together with UE NAS messages in NAS based option. However, for UP based solution, how control information related to a A-IoT procedure can be provided to NG-RAN needs to be addressed, e.g., either by Intermediate UE or CN or both. This requires coordination with other WGs (RAN3, SA3).

For UP based solution and NAS based solution, exchange of A-IoT data between intermediate UE and CN is transparent towards NG-RAN. NG-RAN just needs to be aware of relevant control information of A-IoT procedure to control radio resources.

For UP based solution, control information related to A-IoT procedure can be provided to the serving NG-RAN either by intermediate UE or CN.

From a RAN2’s perspective, for UP based solution, one open aspect related to handling A-IoT data over Uu is whether it is necessary to treat A-IoT data differently from the legacy DRB data. For example, as detailed in section 2.1.3, there would need some changes in handling of temporary out of connection scenario for an ongoing A-IoT procedure compared to legacy NR, it would be beneficial to have a DRB dedicated for A-IoT data delivery. Details about L2/L1 functions and configurations for such a A-IoT DRB can be further studied in WI phase, if UP based solution is adopted.

For UP based solution, RAN2 to study and decide whether/how to distinguish A-IoT data delivery from legacy DRB handling, e.g., how to map A-IoT data onto separate DRB(s), L2 functions.

We think NAS based solution has minimum RAN2 impact. This solution has mainly SA and CT impacts. We assume A-IoT data is transferred as a Intermediate UE NAS message which is piggybacked in an RRC message over a signaling radio bearer. A RAN2 question is whether a new SRB dedicated for A-IoT needs to be defined, for example, considering the possible different handling of this SRB data in case of e.g., temporary out of connection scenario. This question is relevant to RRC based solution as well.

For CP based solution (both NAS and RRC based), RAN2 to study and decide whether/how to distinguish A-IoT data delivery over RRC signaling from legacy SRB handling, e.g., whether and how to define a new SRB for A-IoT data.

Between RRC based solution and NAS based solution, system architecture wise, there are no significant differences between them. From CN point of view, the only difference is on whether the device message delivery:

- In NAS based solution, the device message is embeded within AIOT UE NAS message. The protocol stack can be depicted as below:

Figure 1: Example of NAS based protocol stack

As one can see, this stack figure only shows the transport of AIoT application data via diverse NAS layers, but it does not show how AIoT RAN is triggered to performe AoT radio resource control, a detail to be studied by RAN3.

- In RRC based solution, the device message is delivered over NGAP towards NG-RAN, and then over RRC towards Intermediate UE. The protocol stack can be depicted as below:

Figure 2: Example of RRC based protocol stack

Comparing those two options, RRC based solution brings more impacts to NG-RAN, but results in less overhead from two aspects:

- AMF is not involved in communicating AIoT application data to the AIoT device (AMF could be involved when paging of the Intermediate UE if needed).

- Therefore no UE NAS overhead when delivering device message.

- Most likely alignment with topology 1.

From an SA2 and RAN3 point of view, it might look as if the RRC based solution is more efficient.

In addition, given that resource allocation is taken care by the RRC sublayer in legacy NR, we think RRC based protocol stack would naturally allow serving gNB to control radio resources to be used by A-IoT communication.

RRC based protocol stack option would naturally allow serving gNB to control radio resources to be used by A-IoT air interface.

### 2.1.2	A-IoT resource allocation

It is already agreed in RAN2#127 that network is in control of resources to be used by A-IoT air interface. In addition to resource allocation, we think RAN2 should study how serving gNB control Intermediate UE(s) in terms of reader selection and handling the RRC state and temporary out of connection irrespective of protocol stack option. We discuss our views on some of the aspects in this and subsequent sections.

Irrespective of protocol stack option, RAN2 to study how gNB controls intermediate UE(s) regarding intermediate UE (re)selection, radio resource allocation, coordination among intermediate UEs, and handling RRC state and temporary out of connection scenario.

As for Intermediate UE (re)selection, we assume the UE is authorized to be a reader by CN however, RAN should be involved in selecting what Intermediate UEs to reach out for A-IoT communication based on some RAN metrics such as Uu channel quality, which RRC state the UE is currently in, or mobility status.

As for radio resource allocation for AIoT services, there are some aspects to study, for example how (based on what) a serving gNB can allocate resources, how to define radio resources for A-IoT services, signaling for provisioning resources.

First, it is beneficial for the Intermediate UE to provide some scheduling request/assistance information concerning the need of A-IoT resources to the serving gNB. Detailed assistance information can be further studied, including the resource parameters defined by RAN1 e.g., time, frequency, chip duration, transmission round, etc.

Intermediate UE may provide assistance information to serving gNB via dedicated RRC signaling regarding the need of A-IoT resources to support scheduling and allocation. FFS details for assistance information.

Regarding resource structure and definition, A-IoT radio resource can be defined as Intermediate UE specific/dedicated resources or a common resource pool that multiple Intermediate UEs can share. Allocation granularity is another aspect to consider, i.e., serving gNB may allocate resource for every single transmission/message over A-IoT air interface, or every pair of transmissions, or for whole access/inventory round. Especially when the gNB schedules dedicated resources per transmission pair(s) or access round(s), the Intermediate UE would need to autonomously select resources for each R2D and D2R transmission among the dedicated resources.

The gNB assigns dedicated resources to intermediate UE for A-IoT interface. FFS on resource allocation granularity, i.e., per transmission, per D2R and R2D transmission pair or per access round.

Intermediate UE autonomously selects resources to each D2R and R2D transmission among the dedicated resources which are assigned by the gNB for transmission pair(s) or access round(s).

The gNB assigns dedicated resources to intermediate UE for Uu interface, which are isolated from the dedicated resources for A-IoT interface.

### 2.1.3	RRC state

A Intermediate UE can be in any RRC state, the gNB needs ensure that the Intermediate UE is in RRC_CONNECTED when performing A-IoT procedures. This is because A-IoT procedures such as inventory, command, disabling are expected to require relatively long-duration communication between Intermediate UE and serving gNB to accomplish, which is not suitable for Intermediate UE in RRC_IDLE/RRC_INACTIVE. In addition, in order for the gNB to assign resources to Intermediate UE for A-IoT traffic using the legacy scheduling procedure (e.g., dynamic scheduling or configured scheduling), Intermediate UE needs to be in RRC_CONNECTED. Hence, in case the Intermediate UE is in RRC_IDLE or RRC_INACTIVE, gNB needs to transition it to RRC_CONNECTED, e.g., by means of legacy paging.

In addition to RRC state handling, for radio resource management and coordination among Intermediate UEs, the legacy RRC signaling framework (interface between Intermediate UE and serving gNB) may need to incorporate new signaling procedures/messages e.g., to indicate and schedule a particular Intermediate UE to perform A-IoT procedures, or to avoid a particular Intermediate UE occupy air interface too long due to serving A-IoT devices.

A-IoT procedures are expected to require relatively long-duration communication between intermediate UE and serving gNB, which is not suitable for intermediate UE in RRC_IDLE/RRC_INACTIVE.

Intermediate UE stays in RRC_CONNECTED during an A-IoT procedure.

Same as in legacy, intermediate UE can be moved to RRC_IDLE or RRC_INACTIVE if there is no on-going A-IoT procedure.

### 2.1.4	Temporary out of connection

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In our understanding, “Temporarily Out of Connection” in general means the connection to the serving cell becomes unstable or UE may lose/release RRC connection to serving cell soon but still stays in RRC connected. More specifically, it may include the following cases/events:

- The UE detects physical layer problems and the RLF timer (i.e., T310) is still running. In this case, the number of out of sync instances has reached a configured number (e.g., N310).

- The UE is performing (conditional) HO and the HO timer (e.g., T304) is still running.

- The UE is performing RRC reestablishment and the reestablishment timer (i.e., T301 or T311) is still running.

Given the UE reader is still in RRC_CONNECTED when temporarily out of connection occurs, RAN2 can study this case and develop corresponding solutions.

“Temporarily out of connection” in general means the connection to the serving cell becomes unstable or intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected.

When an intermediate UE detects temporary out of connection, its RRC connection to the serving gNB is not stable and even may be released. The Intermediate UE’s allocated resources for A-IoT data transmissions and receptions over legacy Uu interface and A-IoT interface are not available accordingly to the legacy, i.e., this is because usage of these resources may create interference to other UEs which are of decent connections to the gNB. Thus, there would be interruption for ongoing A-IoT procedure/transactions (if there is any) performed via the Intermediate UE. It is necessary to discuss how to handle on-going A-IoT procedure when temporary out of connection (event) is detected by an intermediate UE.

RAN2 to study how to handle on-going A-IoT procedure when temporary out of connection is detected by intermediate UE.

We think it is important to have network control over behavior of Intermediate UE and A-IoT devices during out of connection duration. First, network can select another Intermediate UE to serve A-IoT services, if available. In this case, serving Intermediate UE is indicated to stop serving A-IoT devices.

Network can select a new intermediate UE prior to occurrence of temporary out of connection.

## 2.2	DO-A Assessment

The focus for traffic types in the SID scope is DO-DTT and DT so far. It is, however, mentioned in the SID that:

- From RAN#104, the study will assess whether the harmonized air interface design can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design is/are not sufficient for the DO-A use case.

For DO-A use cases, a device typically triggers an UL transmission autonomously. Whereas for DO-DTT and DT use cases, for any UL transmission of devices, the gNB/intermediate Intermediate UE needs to send a triggering message beforehand which may comprise resource assignment information based on which the devices can perform UL transmissions either in a backscatter manner or actively generated by devices.

One of the aspects to be considered is how devices supporting DO-A use cases (e.g., device type 2b) initiate random access to deliver DO-A traffic considering the RAN2#125b agreement below:

- Random Access is triggered by the reader

In our understanding, if the active devices upon having UL data can initiate random access themselves without being triggered by network, there needs a separate DL signaling channel to specify resource information for the active devices, similar to the legacy random access with SIB signaling. This is significantly different from current design solution being developed in RAN2 and RAN1 and thus not considered as part of harmonized design. We think random access for delivering DO-A traffic can still be first triggered by reader as long as requirements from DO-A use cases are met. Actual access procedure is then triggered by the device itself.

Random access for DO-A use cases can be first triggered by the reader and then initiated by the device.

Regarding DO-A use cases, one example is smart laundry, as an indoor inventory use case (also indoor sensor use case, see also clause 5.15 in [3]). Another class of DO-A use case is indoor sensors (e.g., see also clause 5.6, 5.13 in [3]). We assume that DO-A traffic (sensory data) typically has stricter end-to-end latency requirement compared to non DO-A use cases. As an example, end-to-end latency requirement in the use case of 5.13 base station machine room environmental supervision is 1s. This means devices that deliver DO-A traffic cannot wait too long for a reader to trigger them to perform random access. In this case, network/reader should provide a frequent signaling channel to allow devices supporting DO-A traffic to access and send their buffered DO-A traffic by mimicking the legacy procedure where the gNB can signal UEs of RACH resources in SIB1. This can be done e.g., by sending the A-IoT paging/the initial trigger message more often even though reader is not requested to do so by CN. We think this can be left to the gNB/Intermediate UE implementation to trigger/send periodic A-IoT paging/initial trigger message.

To not alert passive devices unintentionally (i.e., no service request from CN), there may need to be an indication from the reader to target only one device type, e.g., device type 2b so that devices with other types can skip monitoring, which can be beneficial from device energy saving perspective.

Latency requirement for DO-A traffic types restricts the maximum waiting time of devices if they are triggered by reader.

gNB/Intermediate UE provides frequent DL signaling to allow DO-A devices to access and deliver their buffered traffic. Periodic A-IoT paging can be used as the frequent DL signaling.

Devices supporting DO-A traffic type may or may not have UL data when receiving an A-IoT paging message. In case devices have no UL data to deliver, it would be beneficial to define device behavior, i.e., whether to reply to the A-IoT paging or not and what information to contain in the UL reply message. For example, a straightforward option is devices supporting DO-A simply ignore the A-IoT paging in this case. Another option is devices always send UL report without UL data, e.g., containing only a device ID of some form. This helps harmonize better current solution for DT and DO-DTT traffic types. Yet another option is network/reader dynamically indicates to the DO-A supporting devices in the A-IoT paging message itself whether UL report is required or not. The indication can be device type/group specific or device specific. It is proposed that network sends an indication to indicate whether to respond A-IoT paging message in case of no UL data.

Devices supporting DO-A traffic type may or may not have UL data to deliver when receiving A-IoT paging.

gNB/intermediate UE sends an indication indicating DO-A devices whether to respond to A-IoT paging in case of no UL data.

For better harmonization with DT and DO-DTT, DO-A devices can report device ID in case of no UL data when receiving A-IoT paging.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 impact from Topology 2 depends on which sublayer of intermediate UE (RRC, NAS, or UP) handles A-IoT communication.

Observation 2	For UP based solution and NAS based solution, exchange of A-IoT data between intermediate UE and CN is transparent towards NG-RAN. NG-RAN just needs to be aware of relevant control information of A-IoT procedure to control radio resources.

Observation 3	For UP based solution, control information related to A-IoT procedure can be provided to the serving NG-RAN either by intermediate UE or CN.

Observation 4	RRC based protocol stack option would naturally allow serving gNB to control radio resources to be used by A-IoT air interface.

Observation 5	A-IoT procedures are expected to require relatively long-duration communication between intermediate UE and serving gNB, which is not suitable for intermediate UE in RRC_IDLE/RRC_INACTIVE.

Observation 6	Same as in legacy, intermediate UE can be moved to RRC_IDLE or RRC_INACTIVE if there is no on-going A-IoT procedure.

Observation 7	“Temporarily out of connection” in general means the connection to the serving cell becomes unstable or intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected.

Observation 8	Latency requirement for DO-A traffic types restricts the maximum waiting time of devices if they are triggered by reader.

Observation 9	Devices supporting DO-A traffic type may or may not have UL data to deliver when receiving A-IoT paging.

Observation 10	For better harmonization with DT and DO-DTT, DO-A devices can report device ID in case of no UL data when receiving A-IoT paging.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to study impact of all three protocol stack options, i.e., RRC based, NAS based and UP based solution for Topology 2 without down-selection, and capture outcome in the TR.

Proposal 2	For UP based solution, RAN2 to study and decide whether/how to distinguish A-IoT data delivery from legacy DRB handling, e.g., how to map A-IoT data onto separate DRB(s), L2 functions.

Proposal 3	For CP based solution (both NAS and RRC based), RAN2 to study and decide whether/how to distinguish A-IoT data delivery over RRC signaling from legacy SRB handling, e.g., whether and how to define a new SRB for A-IoT data.

Proposal 4	Irrespective of protocol stack option, RAN2 to study how gNB controls intermediate UE(s) regarding intermediate UE (re)selection, radio resource allocation, coordination among intermediate UEs, and handling RRC state and temporary out of connection scenario.

Proposal 5	Intermediate UE may provide assistance information to serving gNB via dedicated RRC signaling regarding the need of A-IoT resources to support scheduling and allocation. FFS details for assistance information.

Proposal 6	The gNB assigns dedicated resources to intermediate UE for A-IoT interface. FFS on resource allocation granularity, i.e., per transmission, per D2R and R2D transmission pair or per access round.

Proposal 7	Intermediate UE autonomously selects resources to each D2R and R2D transmission among the dedicated resources which are assigned by the gNB for transmission pair(s) or access round(s).

Proposal 8	The gNB assigns dedicated resources to intermediate UE for Uu interface, which are isolated from the dedicated resources for A-IoT interface.

Proposal 9	Intermediate UE stays in RRC_CONNECTED during an A-IoT procedure.

Proposal 10	RAN2 to study how to handle on-going A-IoT procedure when temporary out of connection is detected by intermediate UE.

Proposal 11	Network can select a new intermediate UE prior to occurrence of temporary out of connection.

Proposal 12	Random access for DO-A use cases can be first triggered by the reader and then initiated by the device.

Proposal 13	gNB/Intermediate UE provides frequent DL signaling to allow DO-A devices to access and deliver their buffered traffic. Periodic A-IoT paging can be used as the frequent DL signaling.

Proposal 14	gNB/intermediate UE sends an indication indicating DO-A devices whether to respond to A-IoT paging in case of no UL data.

# 4	References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024

R3-244387, “On A-IoT RAN functional and logical architecture”, Ericsson, RAN3#125.

TR22.840, “Study on Ambient power-enabled Internet of Things”.