3GPP TSG RAN WG2 #127-bis		R2- 2408691

Hefei, October 14th – 18th, 2024

Agenda Item:	8.1.5

Source:	BT Plc

Title:	Downselection of the AI_ML model delivery options

Document for:	Discussion/Decision

# Introduction

During Rel-18, the study for AI/ML model transfer/delivery was captured in TR 38.843 [1]. In there, section 7.2.1.4 Model transfer/delivery does the analysis how models for AI/ML can be transferred. The following Table 1 and Table 2 summarizes the conclusions.

Table 1 Need to consider standardization solutions for transferring/delivering AI/ML models

Whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) is unclear from the outcome of the present study

- Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. Note: The relationships between model transfer/delivery solutions and use cases can be derived from what is captured in clauses 7.3.2, 7.3.3, and 7.3.4.

Table 2 Transfer/deliver model solutioions

Different model transfer/delivery solutions

- A1: Large, no upper limit model/model parameter size, - A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs), - A3: Network controllability on model transfer/delivery (e.g., management decision at gNB), - A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

In this TDoc, the intetion is to downselect the options based on current deployments and market tractions.

# Discussion

## MNO requirement

The solutions summarized on Table 1 can be categorized into three different categories.

- RAN

- Core and LMF

- OAM

Since the OTT solution has no impact on 3GPP specifications, we consider there is not need to further discuss.

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed.

It is important for MNOs to have full control of transfer and delivery of AI/ML models to ensure the network performance. MNOs need a solution that guarantees customers are not impacted by AI/ML model delivery. Hence, before the model is delivered, it is important to be capable to evaluate network congestion, traffic types, slices, etc.

MNOs have gNBs with different radio capabilities being the number of carriers and bandwidth key elements. The own mobility nature of UEs implies that they will cross areas of coverage with different number of carriers. The MNO should avoid transferring the AI/ML model when the total bandwidth on the coverage area creates undesired delays affecting critical services like voice.

Observation 1: MNOs have to be capable to decide when the AI/ML model is transferred

The lack of MNO control over the model transfer, may result on an undersired network overload. For example, if the model is transferred with high periodicity, it will result in an unnecessary radio network overload.

Observation 2: MNOs without full control of AI/ML model delivery will result on an undersired and unnecessary extra overhead.

For all these reasons, it is imperative that MNOs have full control over the delivery process.

Proposal 2: It is required a standardized mechanism that offers full control to MNOs over the AI/ML model transfer/delivery is required in Release 19.

## AI/ML model transfer/delivery by gNB, CN – LMF, OAM

After the study [1], and recent agreements in RAN1 [2], there are several entities identified as candidates to deliver the model as captured in Table 1 and Table 3.

Table 3: CSI feedback enhancements from [1]

### 7.2.2 CSI feedback enhancement The following set of objectives have been identified for the two-sided CSI compression use case. […] For data collection, model transfer/delivery, and function-to-entity mapping analysis, various scenarios unfold for both the two-sided CSI compression use case, as well as for the UE-side CSI prediction use case, when the data generation and termination entities differ. For instance, for: - Model Training: o For the two-sided CSI compression use case, training data can be generated by either the UE or the gNB, depending on specific requirements, while the termination point for training data may include the gNB, OAM, Over-The-Top (OTT) server or UE.  Note: RAN2 identified the case in which Core Network may be used for model training. However, no study was conducted since this is beyond the scope of this Working Group.

Conclusions from Table 3 indicates gNB and OAM may be included while RAN1 clearly stated that the core network is beyond their scope, therefore, they are not in a position to include it. It is understood that RAN1 has not performed any study or analysis on the core network hence, it cannot be excluded.

Observation 2: Based on Note of 2.2.1 [1], the core network for the two-sided use case was skipped from RAN1 study

The gNB can transfer the model but it will require additional storage capacity. On real deployments, all gNBs in the network need to be updated with additional storage. MNOs cannot ensure that all gNBs can be updated in time. On specific scenarios like RAN sharing, perhaps the gNB cannot be updated due to lack of business agreements.

Once the gNB is responsible to deliver the model, it will require to execute additional SW ruitines to transfer the models to its neighbours. In cases like CHO where different gNB might be involved before the HO is executed, the model will be required to be transferred to several gNBs and keep them updated to ensure the model can continue its transference successfully.

Observation 3: The gNB solution adds challenges to real deployments to widely deploy the feature, specially when different pre-Rel-19 features start to be deployed

From 3GPP side, OAM can support multivendor but that feature is not implemented. Deployments require a OAM per RAN vendor. Then, it is not clear how one OAM belonging to one RAN vendor will transfer the model to a gNB belonging to a different vendor. From standards points of view, this can be handled but it is important to remaind the lack of interest to support inter-RAT OAM features. Taking into account a real deployment, each MNO may have several RAN vendors providers based on the deployed solutions, e.g., macro, small, pico and femto cells. If the retransmission of the model requires inter-OAM coordination, the complexity increase.

Observation 4: Current implementations lacks for support of inter-RAN OAM capabilities and market shows no interest in support them

The core network can handle above problems since data is stored in a single location and all RAN vendors must interact with it. Hence, the core network cannot be excluded at this stage. gNB and OAM solutions identified in [1] rely on the core network.

Observation 5: Core Network seems to have the best balance between pros and cons

Since it is not clear which is the best entity to deliver the model and core network was not considered during the study, RAN2 should not exclude the core network.

Proposal 3: RAN2 is asked to create a pros/cons list for the Core Network, gNB and OAM to transfer/deliver the AI/ML model considering real deployment scenarios (e.g., mobility, inter-vendor compatibility, costs, etc.) before the downselection

## AI/ML model transfer/delivery over CP or UP

Table 2 points to different aspects to be considered. A1 – “Large, no upper limit model/model parameter size” makes reference to container size. The CP has a limitation as defined in [1]. The UP does not have such limitation but that may result in unnecessary large container.

Proposal 4: The maximum size of the container used to transfer/deliver the AI/ML model needs to be standardized

RAN2 needs to decide between the CP or the UP to transfer/deliver the model.

With CP solution, it is likely that segmentation will be required. Segmentation of CP paquets will increase the load and the overhead and that may have a severe impact on all UEs. During the design of the delivery mechanisms, it is imperative to avoid any impact on other UEs. By any mean, this new mechanism can create CP congestion. But even without congestion, the fact that extra traffic is added on CP, it is likely to cause undesired delay to critical and low latency traffic being scheduled in the cell.

On live deployments, MNOs uses DSS. That limit the number of symbols the PDCCH. Market has a lack of traction of UE capabilities to support additional NR symbols when DSS like pdcch-MonitoringSingleOccasion or pdsch-MappingTypeB-Alt-r16. That implies, DSS bands are in fact limited to one symbol for the PDCCH.

Observation 6: MNOs have different 5G SA approaches that leads to different challenges on the CP

Some bands have limited bandwidth where MNOs have to deal with CP congestion. MNOs cannot ensure wide bandwidth, CA or DC on all locations. Then, deliver the model over the CP will be realistically limited to areas with enough radio resources and sufficient bandwidth.

Observation 7: Model delivery over CP will introduce additional load and will have an impact all UEs on the cell

The UP offers higher flexibility to NW than CP. The network has several mechanisms to deliver the model. For example, traffic can be assigned with a specific 5QI (Table 2 – A4). Besides, cells are aware of the instataneous radio conditions of each UE. The gNB is aware of the traffic load in the area. With this information, model transfer/delivery scheduling can be optimized.

Proposal 5: RAN2 to focus on the UP as the solution to transfer/deliver the AI/ML model

# Conclusions

During the TDoc, we have made the following observations

Observation 1: MNOs have to be capable to decide when the AI/ML model is transferred

Observation 2: MNOs without full control of AI/ML model delivery will result on an undersired and unnecessary extra overhead.

Observation 2: Based on Note of 2.2.1 [1], the core network for the two-sided use case was skipped from RAN1 study

Observation 3: The gNB solution adds challenges to real deployments to widely deploy the feature, specially when different pre-Rel-19 features start to be deployed

Observation 4: Current implementations lacks for support of inter-RAN OAM capabilities and market shows no interest in support them

Observation 5: Core Network seems to have the best balance between pros and cons

Observation 6: MNOs have different 5G SA approaches that leads to different challenges on the CP

Observation 7: Model delivery over CP will introduce additional load and will have an impact all UEs on the cell

For those reasons, it is proposed

Proposal 1: Solution 4a, OTT without 3GPP impact, captured on [1] is no further discussed.

Proposal 2: It is required a standardized mechanism that offers full control to MNOs over the AI/ML model transfer/delivery is required in Release 19.

Proposal 3: RAN2 is asked to create a pros/cons list for the Core Network, gNB and OAM to transfer/deliver the AI/ML model considering real deployment scenarios (e.g., mobility, inter-vendor compatibility, costs, etc.) before the downselection

Proposal 4: The maximum size of the container used to transfer/deliver the AI/ML model needs to be standardized

Proposal 5: RAN2 to focus on the UP as the solution to transfer/deliver the AI/ML model

# References

- TR 38.843 - Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

- Report of 3GPP TSG RAN WG1 #118 v0.3.0