3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408693

Hefei, China, 14 – 18 October, 2024

Agenda item:	8.7.1

Source:	InterDigital

Title:	Discussion on multi-modality LS

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

SA2 replied to RAN2 LS on multi-modality [R2-2407936] and asked RAN2 to provide feedback on the usage of multi-modality awareness in RAN:

1. Overall Description: SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information. 2. Actions: To RAN WG2, SA WG4, RAN WG3. ACTION: SA2 asks RAN2, RAN3 to provide feedback if any, and SA4 and RAN3 to review RAN2 feedback from XR service perspective and NG-RAN protocols impacts perspective, respectively.

# 2	Discussion

In the previous RAN2#127 meeting, the following use cases for utilization of MM information were already agreed:

RAN2 considers that based on multi-modal information: - The gNB may perform joint admission control. Details can be left up to RAN3 in potential WI phase. FFS if MMSID can be used for this purpose. The gNB may consider this information during QoS flow to DRB mapping (up to gNB implementation)

Furthermore, the following on PDU set discard across QoS flows was also agreed:

RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

This information can be indicated to SA2 in the reply LS.

Proposal 1: Indicate the RAN2 already agreed potential usage of multi-modal information to SA2.

Since RAN2 will not specify MM based enhancements in Rel-19, the response on possible use cases does not need to be restricted to the RAN2 already agreed potential usage of multi-modal information but other proposals discussed in RAN2 could also be indicated as potential use cases while these have not been agreed for now. Such proposals include items, like scheduling enhancements (e.g., through LCP considering the synchronization requirements across DRBs) and multiple active DRX configurations (e.g., different traffic patterns for different multi-modal flows).

Proposal 2: Indicate the other possible use cases for multi-modal information to SA2 not agreed by RAN2 so far, like scheduling enhancements and multiple active DRX configurations.

Furthermore, previously RAN2 specifically used the MMSID as example to determine the multi-modality nature of different flows in its LS to SA2. It should be clarified in a reply LS that it is up to SA2 to discuss and consider any other information or ID to be provided to RAN that can still be used to identify the inter-relation/multi-modality information between flows, ie., MMSID is merely an option.

Proposal 3: Clarify in the reply LS that it is up to SA2 to discuss any information or ID (on top of/as alternative to MMSID) to be provided to RAN (if feasible) which could be used to identify the multi-modality information between flows.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: Indicate the RAN2 already agreed potential usage of multi-modal information to SA2.

Proposal 2: Indicate the other possible use cases for multi-modal information to SA2 not agreed by RAN2 so far, like scheduling enhancements and multiple active DRX configurations.

Proposal 3: Clarify in the reply LS that it is up to SA2 to discuss any information or ID (on top of/as alternative to MMSID) to be provided to RAN (if feasible) which could be used to identify the multi-modality information between flows.