3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408695

Hefei, China, 14 – 18 October, 2024

Agenda item:	8.7.4.2

Source:	InterDigital

Title:	DSR enhancements for UL scheduling

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The WID objective wrt. DSR enhancements was focused as follows [RP-241771]:

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

# 2	Discussion

The following agreements on DSR enhancements were made in RAN2#126 and RAN2#127, respectively:

Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

As it was agreed a single triggering threshold is used as in Rel-18, there should be no impact to DSR triggering, i.e., the UE reports more than one remaining time in the DSR MAC CE when the remaining time for more than one PDU (set) is less than the remainingTimeThreshold.

However, the FFS on any constraints on how the NW configures the DSR triggering and reporting thresholds was left. Already in Rel-18, the UE may report delay-critical data in the DSR that has remaining time more than the remainingTimeThreshold (ie., when pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold). It would seem reasonable to keep this definition for the triggering threshold (ie., remainingTimeThreshold) and requiring the triggering threshold to be the largest of the configured threshold values. Then, for each of the additional thresholds (with lower value than the triggering threshold), the UE would report all the delay-critical data that has remaining time up to the threshold value. After each of these additional thresholds have been encoded into the DSR, the UE then reports the remainder of the delay-critical data with any remaining time. Hence, a requirement could be specified that all the additional thresholds have the time threshold shorter than the remainingTimeThreshold (ie., the triggering threshold).

Proposal 1: DSR triggering is not impacted from reporting of multiple pairs of remaining time and buffer size for one LCG.

Proposal 2: Specify a requirement that the NW sets the DSR triggering threshold (ie., remainingTimeThreshold) as the largest time threshold among the multiple remaining time thresholds configured.

Addressing the FFS on PDU set importance, we think it is unnecessary to include any additional information like the importance in the DSR MAC CE. The importance is useful for discarding at PDCP. It does not impact the treatment of the data during LCP or the reporting of the remaining time and buffer size in the DSR.

Proposal 3: PDU set importance does not need to be included in the DSR MAC CE.

Multiple reporting thresholds should neither impact on the definition of delay critical data which is defined by the remaining time threshold used to trigger the DSR. This is also since single triggering threshold is used as in Rel-18.

Proposal 4: Delay critical data definition is not changed.

Furthermore, it was discussed whether to include the non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. It should be noted that such data includes only non-delay critical data that neither the RLC SDU nor a segment thereof has been submitted to the lower layers by RLC entity – RLC data PDUs that are pending re-transmission (with or without delay critical RLC SDU (segment)) are already considered as delay critical data. Hence, it was argued by chipset vendors in the previous RAN2 meeting that such non-delay critical data ahead of delay critical data in the buffer could be handled by UE implementation (e.g., not assigning the SDU with PDCP SN and consequently not doing any preprocessing before a grant is received). While this could be acknowledged, it is worth noting that this compromises the pre-processing capabilities in the UE which was one of the design factors for NR UP protocols in Rel-15. It would be hence preferable to retain the design factor of allowing pre-processing and support reporting of the non-delay critical data ahead of delay critical data in the buffer size calculation for DSR.

Proposal 5: Include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: DSR triggering is not impacted from reporting of multiple pairs of remaining time and buffer size for one LCG.

Proposal 2: Specify a requirement that the NW sets the DSR triggering threshold (ie., remainingTimeThreshold) as the largest time threshold among the multiple remaining time thresholds configured.

Proposal 3: PDU set importance does not need to be included in the DSR MAC CE.

Proposal 4: Delay critical data definition is not changed.

Proposal 5: Include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR.