3GPP RAN WG2 Meeting #127-bis	R2-2408696

# Hefei, China, 14 – 18 October, 2024

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Discussion on RLC retransmission enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

In the last RANP #105, the revised WID for XR is agreed [1], with the following objective for RLC enhancements:

- Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

In the previous meetings [2][3], agreements are made on RLC retransmission enhancement topic and provided in the appendix. In this contribution we further discuss the RLC retransmissions enhancement topic. Specifically, we discuss the procedure for autonomous retransmission and polling enhancement to support timely RLC retransmissions.

# Discussion

In the RAN2 #126 meeting, it was agreed to consider two options (autonomous retransmission, polling enhancement) to support timely RLC retransmission. Status report enhancement was discussed; however, it was agreed to focus enhancement on the uplink. In the following, we discuss the necessity/benefits to support each option.

Autonomous retransmission:

In the current operation of RLC Acknowledgement Mode in NR, the UE first performs the initial transmission of an RLC PDU. Afterwards, the UE only triggers retransmission of an RLC PDU if it receives an RLC status report from the gNB indicating the NACK for the RLC PDU. For RLC status reporting, the gNB can send a NACK status for an RLC PDU when detecting an SN gap between two consecutively received RLC PDUs or a missing segment of a RLC SDU [4]. Such a procedure may cause a long delay for a retransmission of an RLC SDU (segment) which may not be suitable for an RLC SDU with a small packet delay budget (PDB). This is because, after such a long delay, the retransmission of the RLC SDU (segment) may arrive beyond the PDB of the SDU and, hence impacting the QoS.

Therefore, enhancing the RLC re-transmission for the operation of the RLC AM can be beneficial. To enhance the RLC re-transmission for an RLC SDU with small PDB, the delay gap between the initial transmission and the re-transmission of the RLC SDU can be reduced. One possibility is to allow the UE to retransmit an RLC SDU without waiting for the RLC status reporting from the network. For example, the UE can retransmit the RLC SDU if the remaining time of the SDU is smaller than a configured threshold. Such retransmission can help improve the successful reception of the RLC SDU with small delay budget. Alternatively, the UE may fail to transmit a resulting RLC PDU due to an improper scheduling of transmission parameters such as the resource size, MCS, beam, etc. Therefore, with multiple HARQ retransmissions, the UE may still fail to successfully transmit the RLC PDU. In such cases, it can be beneficial for the UE to trigger a retransmission of the RLC SDU using another set of transmission parameters. Specifically, the UE may retransmit the RLC SDU, if the number of HARQ retransmissions for the RLC PDU exceeds a threshold.

For autonomous RLC retransmission based on the remaining time of a SDU, the network can properly configure a delay threshold to minimize the capacity loss while still guarantee the QoS (e.g., reliability) of the service. Regarding retransmission based on the number of HARQ retransmissions, capacity loss is minimal as the UE may still need to retransmit the SDU as it has not been received successfully.

Proposal 1:	Support the UE to retransmit a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Polling enhancement:

Regarding the polling procedure in NR, the UE is allowed to poll the RLC reception status of the gNB by setting the polling bit in the RLC header to one in the following cases:

    - The amount of data without polling is larger than a threshold (i.e., PDU_WITHOUT_POLL >= pollPDU; or BYTE_WITHOUT_POLL >= pollByte).

    - Its buffer (initial and retransmission buffer) will be empty after the current RLC PDU.

    - Its window is stalling (e.g., the gap between the current SN and the ACKed SN is greater than the window size).

In XR applications, the receiver decodes the received data per PDU set. Therefore, the reception status of each PDU set is more important than the reception status of each PDU in a PDU set. Moreover, in an XR application, the PDU sets may arrive at the UE periodically with a certain jitter. It is also expected that the amount of data per PDU set may vary from one PDU set to another. Specifically, the number of PDUs and the number of bytes per PDU set can be different among PDU sets.

If the current polling procedure is used, in which the UE polls the gNB after a configured number of PDUs or bytes, it is most likely that the UE may poll the RLC status of the gNB in the middle of a PDU set. As a result, the reception status of each PDU set may not be reported. To reduce the delay in determining the reception status of a PDU set, the UE can be configured with a smaller values of pollByte and/or pollPDU. However, this configuration may result in high signaling overhead as the UE may need to receive many more RLC status reports when the values of pollByte and/or pollPDU is small. Therefore, it may be beneficial to enhance the polling procedure, which may help the UE to receive the RLC status of one or more PDU sets. For example, the polling bit can be set based on the transmission of a PDU set (e.g., the last PDU of a PDU set) or it can be set periodically based on the PDU set traffic pattern.

Observation 1: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Proposal 2:	Support polling enhancements to allow the UE requesting the status report after transmissions of each uplink PDU set.

For delay-critical RLC SDU, upon its transmission, it is beneficial to be aware of the reception status of the RLC SDU as soon as possible. It is because the Tx RLC entity may then trigger retransmission of the RLC SDU to guarantee that the RLC SDU can reach the Rx entity before the PSDB. However, in general, the Rx RLC entity transmits the status report based on the polling indication from the Tx entity. Therefore, it is beneficial to allow the Tx RLC to set the polling indication for delay-critical RLC SDU.

Proposal 3:	Support the Tx RLC entity to include a polling indication when the remaining time of a RLC SDU being smaller than a configured threshold.

# Conclusion

In this contribution, the following observations are made:

Observation 1: 	Polling the gNB after a configured number of PDUs or bytes does not inform the UE of the RLC reception status of each PDU set.

Based on this, the following proposals are made:

Proposal 1:	Support the UE to retransmit a RLC SDU if the remaining time of the SDU is smaller than a configured threshold or the number of HARQ retransmissions is greater than a configured threshold.

Proposal 2:	Support polling enhancements to allow the UE requesting the status report after transmissions of each uplink PDU set.

Proposal 3:	Support the Tx RLC entity to include a polling indication when the remaining time of a RLC SDU being smaller than a configured threshold.

# References

- RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3”, Nokia, 2024-09.

- RAN2#125bis chair notes.

- RAN2#126 chair notes.

- 3GPP TS 38.322, Radio Link Control (RLC) protocol specification, v.18.0.0, 2023-12.

# Appendix: Agreements on RLC enhancement for XR

RAN2#125b Agreements

- Agreements: We focus on RLC AM. RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR.

RAN2#126 Agreements

Agreements: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed, or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic