3GPP RAN WG2 Meeting #127-bis	R2-2408697

# Hefei, China, 14 – 18 October, 2024

Agenda Item:	8.7.5

Source:	InterDigital

Title:	Avoiding unnecessary RLC re-transmissions

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion

# 1. Introduction

In the previous meetings [1][2][3], agreements are made on avoiding unnecessary RLC re-transmissions and provided in the appendix. In this contribution we further discuss the topic. Specifically, we further discuss the pros and cons of different approaches for avoiding unnecessary re-transmission and the detailed procedures to support the feature.

# Discussion

In the last meetings, for avoiding unnecessary RLC AM retransmission, RAN2 agreed to further study Rx-initiated, Tx-initiated, and new combined Tx and Rx approaches and choose one of them. In the following, we provide our analysis of the three approaches.

Rx-initiated approach:

In this approach, for proper advancing of the transmitting window, the receiver may indicate the abandoned SDUs to the Tx (e.g., indicate in a status report the abandoned SDUs as ACKs). Afterward, the transmitter may discard the abandoned SDUs and advance its transmitting window. One remaining issue for this approach is to determine when an SDU should be abandoned. One possibility discussed in the last meeting is to use a timer (e.g., reordering timer) to determine whether an SDU is abandoned. Specifically, the Rx may start a timer upon detection of an SN gap in the reception buffer and send a report indicating the abandoned SDUs (e.g., the SDUs within SN gap) upon the expiry of the timer.

In our view, one benefit of this approach is simplicity as it may rely on the existing report (e.g., status report) procedure to advance the transmitting window. There may be no spec impact for the Tx as it just processes the status report as in legacy. Even if the status report is not received successfully by the Tx entity, e.g. upon HARQ failure, the Tx entity can still retransmit the unnecessary SDUs that will be eventually discarded by the Rx entity eventually once out of the window. Therefore, this approach does not have a big specification impact. However, one drawback of this approach is that the Rx entity is not aware of the delay budget of a SDU at the transmit entity as it is not aware of the arrival time of each SDU at the Tx. Afterall, this discussion on avoiding unnecessary retransmissions stemmed from avoiding retransmissions of packets that have exceeded the PDB, which is better known at the transmitter rather than the receiver. Hence, it may be a challenge for the Rx to properly determine when to provide the status report for abandoning certain SDU.

Observation 1: 	The Rx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: Simplicity and less specification impact. Rely on existing status report at the receiver to indicate abandoned SDUs as ACKs.

- Drawback: the Rx entity may not be aware of the delay budget of a SDU to properly determine whether/when an SDU should be abandoned.

Tx-initiated approach:

The purpose of avoiding unnecessary retransmissions is to avoid retransmissions of the SDUs that have exceeded the delay budget, which is known at the Tx not the Rx. It is because Tx has full knowledge of when/whether an SDU is obsoleted and should be abandoned (e.g., based on expiry of a discard timer). Upon abandoning one or more obsoleted SDUs, the Tx can notify the receiver about the obsolete SDUs, and the Rx may update the receiving window according to the information from the Tx. Therefore, the Tx-initiated approach can completely avoid unnecessary retransmissions.

However, RAN2 need to further discuss how to report the abandoned SDUs and the subsequent procedure if the report is not decoded at the Rx. It is because if the report is not decoded at the Rx, it may create an SN gap and stall the receiving window. Moreover, RAN2 may need to further discuss what the Rx to report in a status report when it receives a notification about abandoned SDUs from the Tx.

Observation 2: 	The Tx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: the Tx entity is fully aware of the PDB and whether/when a SDU should be abandoned. Therefore, it can avoid unnecessary retransmissions more accurately.

- Drawback: the Tx-initiated approach can require more specification impact, especially if the report is not decoded at the Rx entity, potentially creating an SN gap and stalling the RLC AM receive window.

Combined Tx and Rx approach:

After analysing the pros and cons of the Tx-initiated approach and Rx-initiated approach, RAN2 has agreed to consider a combined Tx and Rx approach. Specifically, in the combined Tx and Rx approach, Tx side stops to retransmit obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach and the Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach. In our view, the combined Tx and Rx approach inherit the advantage of the Tx-initiated approach by allowing the Tx entity to avoid unnecessary retransmission accurately. In addition, it also helps to avoid stalling window as in the Rx-initiated approach. Therefore, we propose to support the combined Tx and Rx approach to avoid unnecessary retransmission.

Proposal 1: 	Support a combined Tx and Rx approach for avoiding unnecessary retransmission.

It was agreed that as baseline, the Rx window advances before Tx window advance. Therefore, the Tx RLC entity may need to wait for a status report from the Rx RLC entity before advancing its window. The Tx RLC entity is aware of the obsolete RLC SDUs. In our view, it is beneficial to support the Tx RLC entity to indicate the obsolete/discarded RLC SDUs. It is because the Tx RLC entity is fully aware of the status of each obsolete/discarded RLC SDUs and relying on the Rx RLC entity to send the status report based on its local timer may incur delay in advancing its window. The Rx RLC entity, upon reception of such indication from the Tx RLC entity may then advance the receiving window and trigger status report to support advancing the Tx RLC window.

Proposal 2: 	Support Tx RLC entity to indicate the Rx RLC entity about the obsolete/discarded SDUs.

Proposal 3: 	Support the Rx RLC entity to trigger status report when it receives the indication about the obsolete/discarded SDUs from the Tx RLC entity.

It was agreed that the Rx RLC entity may stop to receives an obsolete SDU based on its local timer. Such timer may be used in case the Rx RLC entity fails to receive the indication of the obsolete/discarded SDUs from the Tx RLC entity or no indication is sent. In our view, the timer should be started upon detection of an SN gap in the received RLC buffer. The Rx RLC entity then move its receiving window upon expiry of the timer. To help moving the Tx window, the Rx RLC entity needs to send the status report to the Tx RLC entity after moving its receiving window. Therefore, the Rx RLC entity also needs to send the status report to the Tx RLC upon the expiry of the timer as well.

Proposal 4: 	To support moving the Tx and Rx windows, support the introduction of a timer, in which the Rx RLC entity starts the timer upon detection of an SN gap and triggers status reporting upon expiry of the timer.

The purpose of the status report from the Rx RLC entity is to help the Tx RLC entity to move its transmitting window. In the legacy behavior, the Tx RLC entity forward its transmitting window based on ACK report for an SDU. Therefore, in the status report, the Rx RLC entity also can indicate the obsolete/discarded SDUs as ACK(s).

Proposal 5: 	Support the Rx AM RLC entity to report the obsolete/discarded RLC PDUs as ACK in the status report.

# Conclusion

In this contribution, the following observations are made:

Observation 1: 	The Rx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: Simplicity and less specification impact. Rely on existing status report at the receiver to indicate abandoned SDUs as ACKs.

- Drawback: the Rx entity may not be aware of the delay budget of a SDU to properly determine whether/when an SDU should be abandoned.

Observation 2: 	The Tx-initiated approach for avoiding unnecessary retransmissions has the following pros and cons:

- Pro: the Tx entity is fully aware of the PDB and whether/when a SDU should be abandoned. Therefore, it can avoid unnecessary retransmissions more accurately.

- Drawback: the Tx-initiated approach can require more specification impact, especially if the report is not decoded at the Rx entity, potentially creating an SN gap and stalling the RLC AM receive window.

Based on this, the following proposals are made:

Proposal 1: 	Support a combined Tx and Rx approach for avoiding unnecessary retransmission.

Proposal 2: 	Support Tx RLC entity to indicate the Rx RLC entity about the obsolete/discarded SDUs.

Proposal 3: 	Support the Rx RLC entity to trigger status report when it receives the indication about the obsolete/discarded SDUs from the Tx RLC entity.

Proposal 4: 	To support moving the Tx and Rx windows, support the introduction of a timer, in which the Rx RLC entity starts the timer upon detection of an SN gap and triggers status reporting upon expiry of the timer.

Proposal 5: 	Support the Rx AM RLC entity to report the obsolete/discarded RLC PDUs as ACK in the status report.

# References

- RAN2#125bis chair notes.

- RAN2#126 chair notes.

- RAN2#127 chair notes.

# Appendix: Agreements on RLC enhancement for XR

RAN2#125b Agreements

- Agreements: We focus on RLC AM. RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

RAN2#126 Agreements

Agreements: For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives: - Rx initiated approach - Tx initiated approach RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer. For Tx initiated approach: - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs - Tx side stops retransmit obsolete SDUs - Rx side updates state variables according to the information from Tx side For Rx initiated approach: - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter. - Tx side just processes the status report as in legacy - FFS how Rx side determines that an SDU should be abandoned

RAN2#127 Agreements

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first. In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer. In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach