3GPP TSG-RAN WG2 #127-bis	R2-2408701

Hefei, China, 14th – 18th October 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on A-IoT functionalities

Document for:	Discussion and decision

Agenda Item:	8.2.2

- Introduction

This contribution provides our view on open issues related to A-IoT specific functionalities.

- Discussion

  - Information visible to reader

Following agreements are reached in RAN2#127 [1]. In this section, we discuss whether the agreed information is desired to be mandatorily or optionally informed from CN to reader.

Agreements

- At least the following information are considered useful to be visible to the reader from CN

-	The service type of A-IoT (e.g. inventory, command) . FFS if more information on command type (e.g. read/write/disable) is useful

-	targeted for one or more than one devices;

-	approximate number of target devices (if available).

FFS on mandatory/optional

Service type (e.g., inventory, command)

The reader can determine the radio resources for D2R message transmission and indicate it in an R2D message. For instance, it is agreed that the A-IoT paging message from the reader indicates the resource information for access occasions for Msg1 transmission [2]. Besides, it can be discussed whether a Msg2 includes indication of radio resources for consequent Msg3 transmission.

The size of D2R message (more correctly, the size of upper layer message packed in the D2R message) depends on service type (inventory or command, read or write). Therefore, it is reasonable if the reader can efficiently allocate radio resources for D2R message transmission based on the service type information delivered from the CN. The service type information is useful in all usecases; thus it is desirable that the service type is mandatorily informed to the reader as far as possible from CN perspective.

Proposal 1.	From RAN2 perspective, it is desirable for the CN to mandatorily inform service type of AIoT (inventory, write command, or read command).

Targeted for one or multiple devices

From reader viewpoint, if the paging request is targeting only one device, the reader can drastically reduce the number of access occasions for 3-step/2-step CBRA, or even can choose contention-free access to quickly complete A-IoT random access procedure.

In the paging request from CN to reader, if the targeted devices are indicated using specific device ID(s), the reader can implicitly know the number of targeted devices. But if the target is indicated using group ID(s) or without using ID (i.e., requiring paging to all devices nearby), it is difficult for the reader to guess the number of target devices. In this case the assistance indication from the CN to the reader is useful.

Therefore, whether the number of targeted devices is one or more should be optional information and should be informed when the reader cannot guess the number of targeted devices.

Proposal 2.	From RAN2 perspective, it is desirable for the CN to optionally inform whether the number of targeted devices is one or more. It should be informed when the target devices are indicated using group ID or without ID.

Approximate number of targeted devices

The reader can moderately determine the number of access occasions for A-IoT Msg1 transmission if the approximate number of targets is informed.

This information should be optional in the same manner as “one or more targets” information.

Proposal 3.	From RAN2 perspective, it is desirable for the CN to optionally inform an approximate number of targeted devices. It should be informed when the target devices are indicated using group ID or without ID.

  - Message size/status information

In this section we discuss the need of introduction of message size/status information.

The purpose of message size/status information is to allow the reader to allocate sufficient/minimize radio resources based on reported upcoming D2R message size. In current discussion, we find the three kinds of D2R messages have a flexible size, (1) A-IoT Msg1 for 2-step CBRA, (2) A-IoT Msg3 for 3-step CBRA, (3) D2R messages after completion of A-IoT random access.

But we would like to note the timing when the D2R message size/status is determined.

- A-IoT Msg1 for 2-step CBRA: The size/status of A-IoT Msg1 is determined when the device receives A-IoT paging message which includes command. Note that the size/status would be stable in inventory usecase.

Figure 1. Timing of Msg1 size/status determination in 2-step CBRA.

- A-IoT Msg3 for 3-step CBRA: The size/status of A-IoT Msg3 is determined when the device receives A-IoT paging message which includes command. Note that the size/status would be stable in inventory usecase.

Figure 2. Timing of Msg3 size/status determination in 3-step CBRA.

- D2R messages after completion of A-IoT random access: May depend on upper layer protocol, but it should be natural to assume that size/status of D2R message is determined when the device receives the previous R2D message.

In short, excluding when a command message is delivered in the paging message in case (2), the size/status of D2R messages is determined when the device receives the previous R2D message. Given that situation, the device can only report the size/status at the same time with the D2R message itself, thus the size/status reporting does not help the reader anymore.

One exception is when a command message is delivered in the paging message in case (2). In this case, ideally, the device can report the size/status of A-IoT Msg3 in A-IoT Msg1. However, if we go this way, the A-IoT devices have to be equipped with a memory/buffer large enough to temporarily store Msg3, which may be a burden for the device implementation. We disagree to support size/status reporting just for this corner case.

Another exception is, however, when layer-2 segmentation is agreed to be introduced. If the device supports layer-2 segmentation, the device generates a large message and splits it, e.g., into 1st segment and 2nd segment. Then the device transmits 1st segment to the reader and simultaneously stores the 2nd segment in its memory/buffer. Therefore, if we introduce layer-2 segmentation, the device has to be equipped with a memory/buffer to store the 2nd segment. Therefore, we think that we can study the size/status reporting of 2nd segment which is attached to transmission of 1st segment.

Proposal 4.	The need of message size/status information via D2R message depends on  RAN2 conclusion on whether to support layer 2 segmentation.

Proposal 5.	If layer 2 segmentation is not supported, message size/status information is not introduced.

  - Energy status report

Following agreement was made in RAN2#127.

Agreements

-	RAN2 will study the need and use of energy status report to be delivered from the device to the reader in case the device does not have energy for the follow up messages.   FFS details on signaling

When to use energy status report

We understand that the purpose of the energy status report is for the device to report that the device cannot continue following procedure due to energy status in its battery. Therefore, it is straightforward to support the energy status report at least attached with A-IoT Msg1.

Other options are to attach with A-IoT Msg3 or with D2R messages to deliver upper layer message after completion of A-IoT random access. But considering that the messages after A-IoT Msg3 are basically exchanged between the device and CN, i.e., only upper layer messages are expected to utilized after A-IoT random access completion, we think it is more natural that the device reports its energy status via upper layer message directly to the CN if it has energy issue.

Proposal 6.	Energy status report is delivered in AIoT Msg1.

Content of energy status report

Energy status report at least needs 1 bit to indicate the cancel of following procedure.

Other candidates are remaining battery level and time estimated for recovery. However, battery level and time to recovery highly depend on the device implementation, which would make it hard to test whether the report content is correct. Moreover, the battery itself would degrade especially in long-term usage like other IoT usecases, which makes it even harder for the NW to consider reported battery level and time to recovery.

Proposal 7.	Energy status report is a 1-bit indication to report whether the device can continue the procedure.

Whether to omit other content in A-IoT Msg1

We already agreed that A-IoT Msg1 has other content, e.g., random ID generated in the device. But if the device attaches the energy status report to A-IoT Msg1, the following procedure is expected to be canceled. This means that contents in Msg1 like random ID does not help the procedure anymore. We think that we should discuss if the device can omit other content than energy status in A-IoT Msg1.

Proposal 8.	For energy status report delivered in AIoT Msg1, discuss whether the device can omit other information, e.g., random ID, from AIoT Msg1.

- Summary and proposal

Proposal 1.	From RAN2 perspective, it is desirable for the CN to mandatorily inform service type of AIoT (inventory, write command, or read command).

Proposal 2.	From RAN2 perspective, it is desirable for the CN to optionally inform whether the number of targeted devices is one or more. It should be informed when the target devices are indicated using group ID or without ID.

Proposal 3.	From RAN2 perspective, it is desirable for the CN to optionally inform an approximate number of targeted devices. It should be informed when the target devices are indicated using group ID or without ID.

Proposal 4.	The need of message size/status information via D2R message depends on  RAN2 conclusion on whether to support layer 2 segmentation.

Proposal 5.	If layer 2 segmentation is not supported, message size/status information is not introduced.

Proposal 6.	Energy status report is delivered in AIoT Msg1.

Proposal 7.	Energy status report is a 1-bit indication to report whether the device can continue the procedure.

Proposal 8.	For energy status report delivered in AIoT Msg1, discuss whether the device can omit other information, e.g., random ID, from AIoT Msg1.

- References

[1]	Chair notes, RAN2#127.

[2]	Chair notes, RAN2#126.