3GPP TSG-RAN WG2 #127-bis	R2-2408702

Hefei, China, 14th – 18th October 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on A-IoT paging message

Document for:	Discussion and decision

Agenda Item:	8.2.3

- Introduction

This contribution provides our view on open issues related to A-IoT paging.

- Discussion

  - Content required for A-IoT paging message

Random access type

RAN2 already agreed to study multiple schemes of A-IoT random access procedure, 3-step CBRA, 2-step CBRA and contention-free access.

After the Ambient IoT device receives an A-IoT paging message, contents and timing of the first message sent by the device are different as following way.

- In 3-step CBRA, the device transmits A-IoT Msg1 containing a random ID generated by the device at an access occasion randomly selected by the device.

- In 2-step CBRA, the device transmits A-IoT Msg1 containing upper layer data at an access occasion randomly selected by the device.

- In contention-free access, the device transmits a first D2R message at an access occasion designated by the network.

Therefore, the device has to be aware of which type of A-IoT random access procedure the device is required before it sends the first message in order to determine its contents and timing. Therefore, an indication of A-IoT random access type is needed in the A-IoT paging message.

Observation 1.	Ambient IoT device has to be aware of which type of A-IoT random access is required to determine contents and timing of the first message.

In terms of usecase, we have not excluded any random access type for any usecase (inventory, command only, nor inventory and command), so the indication of random access type is beneficial for all usecases.

Proposal 1.	Random access type (i.e., 3-step CBRA, 2-step CBRA, or contention-free access) is indicated in A-IoT paging messages for all usecases (inventory, command only, and inventory and command).

  - How to avoid duplicated response from devices

In RAN2#127, we have reached an agreement to support mechanism to allow A-IoT reader to transmit multiple A-IoT paging messages associated with the same request from CN [1]. In this case the device could transmit multiple responses (i.e., A-IoT Msg1) associated with the same CN request, thus it is FFS how to deal with it.

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

The benefit of multiple transmission of A-IoT paging messages is the compensation for temporary degradation of radio condition. For example, a device can recognize the A-IoT paging by the reader even when some of A-IoT paging messages are lost and cannot reach the device.

The simplest solution is that the device reacts only once per CN request, i.e., the device ignores A-IoT paging message if the paging is associated with the same CN request as a past A-IoT paging message. To support this feature, the device has to be aware of which A-IoT paging messages are based on the same request from CN. This awareness can be easily realized if the reader informs the association between A-IoT paging messages and paging request from the CN, e.g., the reader informs an ID of CN request in the A-IoT paging message.

Proposal 2.	Identifier of the paging request from CN is indicated in A-IoT paging message.

  - Applicable scenario of “command only” usecase

RAN2 already agreed to study “Command only” use case where the command is included in the A-IoT paging message [2].

Agreements (omit) 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

The command message is generated in the CN and delivered in upper layer. From AS perspective, the command message is regarded as upper layer data.

We would like to note that current RAN2 discussion is based on assumption on presence of security protection in upper layer, considering that the upper layer messages can contain data important for secure operation, e.g., command to control Ambient IoT devices or permanent device ID to track devices.

As legacy, the security protection is an end-to-end protection. In other words, two terminated nodes share a security key which is hidden to any other nodes, to enable encrypt/integrity protection and decrypt/integrity checking only in the terminated nodes. If the security protection in upper layer is supported in the same way, as shown in Figure 1, the command message sent in A-IoT paging message can be decoded by only one device.

Figure 1. Upper layer security and A-IoT paging message with command.

Therefore, we propose to assume that the “Command only” use case is only when one device is targeted by A-IoT paging message.

Observation 2.	Given that security protection is supported in upper layer, upper layer data for a command message provided in an A-IoT paging message may not be able to be decoded by multiple Ambient IoT devices since the upper layer security is E2E protection.

Proposal 3.	RAN2 assume that only one device can be targeted in A-IoT paging message in “Command only” use case.

- Summary and proposal

Observation 1.	Ambient IoT device has to be aware of which type of A-IoT random access is required to determine contents and timing of the first message.

Proposal 1.	Random access type (i.e., 3-step CBRA, 2-step CBRA, or contention-free access) is indicated in A-IoT paging messages for all usecases (inventory, command only, and inventory and command).

Proposal 2.	Identifier of the paging request from CN is indicated in A-IoT paging message.

Observation 2.	Given that security protection is supported in upper layer, upper layer data for a command message provided in an A-IoT paging message may not be able to be decoded by multiple Ambient IoT devices since the upper layer security is E2E protection.

Proposal 3.	RAN2 assume that only one device can be targeted in A-IoT paging message in “Command only” use case.

- References

[1]	Chair notes, RAN2#127.

[2]	Chair notes, RAN2#126.