3GPP TSG-RAN WG2 #127-bis	R2-2408703

Hefei, China, 14th – 18th October 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on A-IoT random access

Document for:	Discussion and decision

Agenda Item:	8.2.4

- Introduction

This contribution provides our view on open issues related to A-IoT random access procedure.

- Discussion

  - Failure/success indication

In RAN2#127, we agreed to study failure/access indication by A-IoT reader to inform failure/success of D2R messages [1].

Agreements

-	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

In this section, we discuss the need and usecase of failure/success indication for A-IoT Msg3 in 3-step CBRA and A-IoT Msg1 in 2-step CBRA.

Indication of failure/success of A-IoT Msg3 in 3-step CBRA

Figure 1. 3-step CBRA procedure

Before 3-step CBRA, the reader transmits A-IoT paging message to targeted device(s) or to every device (without targeting to specific devices).

Then the reader would receive D2R A-IoT Msg1 which includes a random ID generated in a device. Please note that we have not agreed to allow devices to indicate a device ID in A-IoT Msg1, therefore the reader cannot know which device transmitted the Msg1.

The random ID included in the Msg1 is echoed in following R2D A-IoT Msg2.

In A-IoT Msg3 the device transmits upper layer message, which the reader transparently forward to the CN.

Now we would like to discuss how the reader indicates the recipients of failure/success indication of A-IoT Msg3. The Msg3 would content a permanent device ID (if it is in inventory usecase or inventory and command usecase), but the permanent ID is assumed to delivered in upper layer message that the reader cannot decode. This means that, as depicted in Figure 2, the reader cannot know from which device the reader successfully received the Msg3. The opposite is also true, i.e., the reader cannot know from which device the reader failed to receive Msg3.

Figure 2. Indication of device identity in 3-step CBRA procedure

We think that a simple solution is to let the device to indicate the random ID in A-IoT Msg3 which is same as the random ID reported in A-IoT Msg1. Then the reader can inform random IDs, which is determined based on successfully received A-IoT Msg3, in the failure/success indication as recipients of the indication.

Proposal 1.	For 3-step CBRA, the device informs the random ID (same as that in A-IoT Msg1) in A-IoT Msg3.

Proposal 2.	For 3-step CBRA, the R2D message for failure/success indication of A-IoT Msg3 includes random ID(s) that the reader finds missing or successfully received.

Moreover, we are wondering if the “success indication” from the reader is correct/can be trusted. This is because, as shown in Figure 3, the last message of A-IoT random access procedure is upper layer message which the reader cannot decode. It is the CN that can judge whether the message is finally successfully decoded. On the other hand, at least the failure indication by the reader is always correct, thus we prefer to only support failure indication by the reader.

Therefore, we would like to just clarify that the success indication by the reader means completion of the A-IoT random access procedure but does not mean that the upper layer procedure (e.g. inventory) is successfully completed.

Figure 3. Only the CN can judge successful completion?

Proposal 3.	For 3-step CBRA, success indication does not mean upper layer procedure (e.g. inventory) is successfully completed.

Indication of failure/success of A-IoT Msg1 in 2-step CBRA

Figure 4. 2-step CBRA procedure

For 2-step CBRA, we would like to point out that the reader can only know random IDs which is successfully received by the reader, while the permanent device IDs are reported via upper layer message in Msg1 thus the reader cannot be aware. Therefore, in our understanding, the reader can only indicate successful completion of 2-step CBRA, by indicating received random IDs.

Proposal 4.	For 2-step CBRA, the R2D message for success indication of A-IoT Msg1 includes random ID(s) that the reader finds successfully received.

In the same way as 3-step CBRA, we believe that the success indication of A-IoT Msg1 does not mean completion of upper layer procedure (e.g. inventory).

Proposal 5.	For 2-step CBRA, success indication does not mean upper layer procedure (e.g. inventory) is successfully completed.

- Summary and proposal

Proposal 1.	For 3-step CBRA, the device informs the random ID (same as that in A-IoT Msg1) in A-IoT Msg3.

Proposal 2.	For 3-step CBRA, the R2D message for failure/success indication of A-IoT Msg3 includes random ID(s) that the reader finds missing or successfully received.

Proposal 3.	For 3-step CBRA, success indication does not mean upper layer procedure (e.g. inventory) is successfully completed.

Proposal 4.	For 2-step CBRA, the R2D message for success indication of A-IoT Msg1 includes random ID(s) that the reader finds successfully received.

Proposal 5.	For 2-step CBRA, success indication does not mean upper layer procedure (e.g. inventory) is successfully completed.

- References

[1]	Chair notes, RAN2#127.