3GPP TSG-RAN WG2 Meeting #127bis  	                            R2-2408713

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.3

Source: 	Sony

Title:	On-demand SIB1 for IDLE/INACTIVE UEs

Document for:	Discussion

# Introduction

In RANP#105, following objectives for on-demand SIB1 for NES WI were agreed in RP-242354:

2. Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we discuss WUS configuration aspects and UE behaviour for not able to acquire OD-SIB1 as well as access restrictions for legacy UEs.

# Discussion

2.1 WUS configuration from NES cell or any cell

In RAN2#127 meeting, it was agreed that once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES cell, but it is still open that whether cells other than Cell A can provide the UL WUS configuration.

If the NES cell doesn’t provide UL WUS configuration, then for the UE who performs the cell selection procedure and identify the NES cell as candidate cell from radio link quality point of view, it will then find it out that this cell is a NES cell and won’t provide UL WUS configuration. As a result, the UE won’t be able to camp on the NES cell because it is impossible for this UE to send on-demand SIB1 request without knowing the UL WUS configuration. So as a consequence, if the NES cell doesn’t provide UL WUS configuration, it may impose implicit restriction on the UE to select the NES cell to camp on during initial cell selection.

If the NES cell provides its UL WUS configuration and this configuration may be valid for more than one NES cell, then it is possible for the UE to camp on the NES cell normally. Then it is feasible for UE to send on-demand SIB1 request to and get reply from NES cell. This is beneficial to support other scenarios in the future and won’t impose any cell selection/reselection restrictions for UE to select the NES cell.

Proposal 1: Network can include WUS config from any type of cell.

For the UE connected to any cell i.e. NES cell or Cell A, it can receive the WUS configuration in RRC release message. On the other hand, as the Cell A may also provide the UL WUS configuration for the same NES cell, the mismatched configuration between Cell A and NES cell may not be an issue as dedicated configuration takes priority.

Proposal 2: Network can include WUS config in RRC Release message and this configuration takes priority over WUS configuration received from system information.

2.2 UL WUS configuration for on-demand SIB1 and on-demand SIB1 acquisition procedure

In RAN1#116 meeting, it was agreed that for study of UL WUS design, consider at least PRACH as a starting point. In Rel-15, on-demand SIB transmission is supported via IDLE/INACITVE UE to trigger a random access procedure. In MSG1 solution, a subset of the PRACH resources is associated with the requested SI, and network will reply a MAC subPDU with RAPID only for acknowledgement.

It is important that network can identify the request to transmit on-demand SIB1 from very beginning, in order to reduce the access delay for UE. And it was agreed in RAN2#125bis that UL WUS configuration includes at least RACH configuration. Therefore, the network should reserve PRACH resources that is associated with on-demand SIB1 transmission request.

Proposal 3: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

SIB1 is an essential SIB for UE and contains important information e.g., initial access information, cell barring information for UE to further access the network, therefore it is vital to specify the UE’s behaviour when UE fails to acquire the SIB1, in order to allow the UE to take immediate action to re-acquire the SIB1.

In RAN2#126 meeting, RAN2 discussed the UE behaviour after RACH failure of on-demand SIB1 request. In our opinion, the overall failure cases include during and after RACH to acquire the SIB1 are listed as follows:

- The RACH is completed unsuccessfully due to the pre-configured maximum number of preamble transmissions is reached.

- The RACH procedure is successful but UE fails to receive the SIB1 from network.

In either of the above listed cases, UE’s behaviour needs to be specified and RAN2 can discuss the options listed as follows.

Option 1: Follow “unable to acquire MIB” behaviour i.e. override IFRI bit.

“3>	perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs or intraFreqReselection2RxXR for 2Rx XR UEs, is set to allowed;”

Option 2: Follow “unable to acquire SIB1” behaviour and respect IFRI bit.

For both options, UE will initiate RACH for on-demand SIB1 if the newly selected cell is a NES cell and its on-demand SIB1 is not being broadcasted.

Proposal 4: RAN2 to discuss the options to further specify when UE fails to acquire the on-demand SIB1.

Option 1: Follow “unable to acquire MIB” behaviour i.e. override IFRI bit.

Option 2: Follow “unable to acquire SIB1” behaviour and respect IFRI bit.

2.3 Access restrictions for legacy UEs

In RAN2#127 meeting, the access restrictions for legacy UEs if on-demand SIB1 is introduced has been discussed and we have some options listed as follows.

Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded. - Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB. - Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB. - Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

We think option 1 is a straightforward option as it can guarantee to bar the legacy UE. The cell supporting OD-SIB1 should set the cellBarred as “barred”, therefore for the legacy UE receiving this bit will not proceed anymore. On the other hand, a NES UE supporting the OD-SIB1 can ignore this field and if the cell is not broadcasting the OD-SIB1 and the UE has the up-to-date UL WUS configure, it can initiate the OD-SIB1 acquisition procedure.

With Option 2, by setting ssb-SubcarrierOffset to a no SIB1 indication (which is not very clear how to indicate it) may work when SIB1 is actually broadcasted. But there is no guarantee that a legacy UE will always find SIB1 being broadcasted when it tries to perform cell selection/reselection. In our understanding legacy UE behaviour on missing SIB1 is not very clearly implemented.

Option 3 relies on other cells to update the excluded list and not a good solution impacting the existing deployments and change the broadcasted parameters of a cell when a new feature is introduced in a neighbouring cell.

Therefore, we propose:

Proposal 5: Set cellBarred bit to barred in MIB in order to bar the legacy UEs to access the cells supporting on-demand SIB1.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Network can include WUS config from any type of cell.

Proposal 2: Network can include WUS config in RRC Release message and this configuration takes priority over WUS configuration received from system information.

Proposal 3: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

Proposal 4: RAN2 to discuss the options to further specify when UE fails to acquire the on-demand SIB1.

Option 1: Follow “unable to acquire MIB” behaviour i.e. override IFRI bit.

Option 2: Follow “unable to acquire SIB1” behaviour and respect IFRI bit.

Proposal 5: Set cellBarred bit to barred in MIB in order to bar the legacy UEs to access the cells supporting on-demand SIB1.