3GPP TSG-RAN WG2 Meeting # 127bis 	                R2- 2408714

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.2

Source: 	Sony

Title:	LTM for Inter-CU

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new WI of mobility enhancements for NR has been agreed which contains the following bullet point [1]:

- Specify support for inter-CU Layer 2 Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured.

  - As secondary priority, support the case when NR-DC is configured, and CU is acting as SN and MCG is unchanged.

  - As secondary priority, support the case when NR-DC is configured, CU is acting as MN and SCG is unchanged or SCG is released.

    - Note: The case that LTM is configured in both MCG and SCG is excluded

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling.

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

In RAN2#125Bis, the following agreements were reached:

Agreements on scenarios:

1.	RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios.

2.	Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch.

3.  UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells.

Agreements on latency analysis:

4.	Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM.

Agreements on early sync phase:

- Early DL and UL sync is also supported for inter-CU LTM.  Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval.

- PDCCH ordered early RACH is supported for inter-CU LTM.

- For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option.

Agreements on LTM cell switch execution phase:

Upon inter-CU LTM execution, UE performs

- MAC reset

- RLC re-establishment

- PDCP re-establishment

- Security key update

FFS if there is an inter-CU LTM w/o security key change.

In RAN2#126, the following agreements were reached:

Agreements on inter-CU LTM:

Clarification of DC

- An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Xn-based and N2-based inter-CU LTM

- Xn-based inter-CU LTM is prioritized in Rel-19.

Stage-2 signaling flows and procedures

- The preparation of inter-CU LTM configuration is initiated by the source gNB-CU.

- For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM.

- The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells.

- In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed).

Preparation:

RRC Configuration/structure

- The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

LTM Candidate ID

- For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs.

Max number of LTM candidate IDs

- The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.

Early sync:

RAR based option

- RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM.

Execution:

LTM Cell Switch Command

- R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

LTM Cell switch completion

- Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

- The LTM completion defined for intra-CU LTM is followed for R19 LTM.

In RAN2#127, the following agreements were reached:

Agreements on inter-CU LTM

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least an inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this contribution, we discuss how to support the LTM for inter-CU w/o security key change, and how to reduce the resource consumption (e.g., RACH, configured grant) for the candidate target cells for both intra-CU and inter-CU scenarios.

## Mobility Enhancements

## 2.1 LTM for Inter-CU

In Rel-18, LTM enhancements for intra-CU was specified which provided some mobility benefits of reducing HO latency and interruption time. However, there is a restriction that only mobility between cells of the same gNB (intra-CU) were supported in Rel-18 as shown on Figure 1.

In Rel-19, in order to expand the benefits of LTM, the new WID contains further enhancement for LTM to support inter-CU, i.e., cells of different gNBs as shown on Figure 2. Obviously, this comes with some challenges, for example there is a PDCP re-establishment resulting in data loss and there will be a greater number of handovers with LTM due to increased number of ping-pongs between source and target cells as observed in Rel-18. These number of ping pongs are not affected whether source and target cells are controlled by different CUs, but in facts, are related to the fluctuations of the radio conditions as L1 measurement is used to trigger LTM handover.

The main reason of PDCP re-establishment is the security key change due to source cell and target cell are controlled by different CUs. The security key change will require PDCP/RLC/MAC reset/re-establishment making LTM performance similar to current L3 handover. So, changing security keys every time when such HO takes place and ping-pong or subsequent handover either intra CU or inter CU will result in more data loss (due to PDCP re-establishments).

One solution is that DU2 connects to CU1 in addition to connecting to CU2 as shown in Figure 3 and sends/receives data traffic to CU1. In fact, RAN3 needs to define tunnel endpoints and in this case, the endpoints could be either CU1-CU2-DU2 or CU1-DU2 and in our understanding both options are already supported in RAN3 specifications. At the same time, RRC is still terminated in CU1. In other words, there is F1-U path between DU2 and CU1 for this UE. This additional path should be only for the purpose of additional F1-U path and other management responsibilities of DU2 remain with CU2.

Just to clarify that this solution does not violate current RAN3 design assumption that a DU is controlled by a single CU only. In fact, similar mechanism has been used for SDT anchor cell and NR-DC, as explained in our joint contribution [3]. In addition, we think similar design principle has been used for mobile IAB-DU migration procedure, as described in 38.300:

- The mobile IAB-node can perform the mobile IAB-DU migration procedure, where a new logical mobile IAB-DU is established on the mobile IAB-node and the initial logical mobile IAB-DU is released some time later. During this procedure, the UEs connected via the mobile IAB-node are handed over from the initial logical mobile IAB-DU, referred to as the source logical mobile IAB-DU, to the new logical mobile IAB-DU, referred to as the target logical mobile IAB-DU. The details of this procedure are defined in TS 38.401 [4]. Enhancements related to BAP for mobile IAB-DU migration are defined in TS 38.340 [31].

Hence, we propose that:

Proposal 1: RAN2 to agree that inter-CU LTM w/o security key change is supported by allowing target DU to receive control and user plane data from source CU.

Proposal 1 is needed to address ping-pong handovers. However, at some stage, a security key change will be required, and it should be up to network to configure if a security key change is required for both intra and inter CU LTM, in other words, intra CU and inter CU handovers are transparent to UE and UE simply acts based on the configuration. This configuration should also indicate if security key change is required or not.

## 2.2 LTM Resource consumption for the target cells

In Rel-18 LTM, resources (e.g., RACH, configured grant) for the target cell are allocated in the RRC reconfiguration messages. As the resources are pre-allocated for multiple candidate cells even if they are not used, there is an issue of resource consumption (i.e., allocated but not used at the end). Hence, how to minimize the resource consumption is an issue to be solved for both intra-CU and inter-CU deployment scenarios.

In the current LTM procedure, the resources (e.g., RACH, configured grant) for the target cell are allocated in the RRC reconfiguration message (Step 2 of Figure 4 below). For example, in order for a UE to switch to the target cell, the UE should transmit RACH to the target cell and these RACH resources can be pre-allocated based on contention-free random-access (CFRA) to determine the timing advance. Otherwise, the UE should apply contention-based RACH transmission (CBRA) to determine the timing advance.

In addition, for RACH-less where the UE knows the timing advance, e.g., small cells where TA=0 or the UE derives the TA based on timing difference of the serving cell and the target cell, the UE applies configured grant (CG) resources to transmit the first message (e.g., data) to indicate its arrival at the target cell where the CG resources are pre-allocated for each target cell in step 2 (Figure 4).

Figure 4. Rel-18 LTM procedure (Figure 9.2.3.5.2-1. Signalling procedure for LTM) [2]

In order to minimize the resource consumption, the resources that are allocated in step 2 can be deactivated by default, and then when the serving cell receives L1 measurement report and makes the decision to issue the LTM command for a target cell (for example index 1), the resources are activated only for that target cell (index 1). This means that activation message should be sent from the serving cell to the decided target cell.

Proposal 2: RAN2 to discuss how minimize the resource consumption (e.g., RACH, configured grant) for the candidate target cells.

## Conclusions

In this contribution, we have discussed how to support the LTM for inter-CU w/o security key change, and how to reduce the resource consumption (e.g., RACH, configured grant) for the candidate target cells for both intra-CU and inter-CU scenarios. We have the following proposals.

Proposal 1: RAN2 to agree that inter-CU LTM w/o security key change is supported by allowing target DU to receive control and user plane data from source CU.

Proposal 2: RAN2 to discuss how minimize the resource consumption (e.g., RACH, configured grant) for the candidate target cells.

## References

[1] RP-234036, “New WID: NR mobility enhancements Phase 4,” RAN#102.

[2] TS 38.300 V18.0.0, Section 9.2.3.5 (Figure 9.2.3.5.2-1. Signalling procedure for LTM).

[3] R2-2405469, “Inter-gNB LTM and moving PDCP anchor”, Qualcomm Incorporated, NTT DOCOMO, Sony