3GPP TSG-RAN WG2 Meeting #127bis  	                            R2-2408716

Hefei, China, Oct 14th – 18th, 2024							                (Resubmission of R2-2406629)

Agenda Item:	8.8.5

Source: 	Sony

Title:	Satellite switch with re-sync in regenerative payload

Document for:	Discussion

# Introduction

In RAN2#126 meeting, we have agreements on support of regenerative payload as following:

Agreements:

- Regarding potential issues with RRC re-establishment, RAN2 will wait for RAN3 input, if any

- Regarding potential issues with support of Inactive state, RAN2 will wait for RAN3 input, if any

- RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed

- RAN2 assumes that network verified UE positioning is supported for regenerative architecture

- RAN2 assumes that the typical setting for common TA and kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero.

In this contribution, we discuss the issues of satellite switch with re-sync in regenerative payload.

# Discussion

In Rel-18 NTN, during satellite switch in the quasi-earth fixed scenario with the same SSB frequency and the same gNB, the satellite switch with re-sync procedure is supported. The satellite switch with re-sync can avoid L3 mobility for the UEs in the cell and the UEs can start synchronizing with the target satellite before the source satellite stops serving the cell for soft satellite switch over. As a network implementation, the cells will maintain the same PCI on the geographical area covered by the quasi-earth fixed beam.

In the regenerative payload architecture, the gNBs will change during the satellite switch over, therefore, it is difficult to support the satellite switch with re-sync in regenerative payload without any change.

Observation 1: It’s difficult to support satellite switch with re-sync in regenerative payload architecture without any changes compared to Rel-18 NTN.

The issues are listed as following:

For hard switch, it may be up to network implementation that the same SSB frequency and same PCI should be deployed between the satellites on board in order to support the satellite switch with re-sync. The feasibility of such deployment should be explored. Furthermore, the UE context information transfer between the gNBs on board of the satellites should be considered as well.

For soft switch, besides the issues list above, the issue on the mismatched configuration of service time and start service time between two gNBs needs to be addressed, as already discussed in RAN2#125bis meeting.

Furthermore, RAN3’s discussion on the switch over for regenerative payload may also have impact on the satellite switch with re-sync.

Proposal 1: RAN2 should wait for RAN3’s progress on regenerative payload architecture in order to decide whether to enhance the satellite switch with re-sync for regenerative payload.

# Conclusion

We have the observation as following:

Observation 1: It’s difficult to support satellite switch with re-sync in regenerative payload architecture without any changes compared to Rel-18 NTN.

We propose RAN2 to discuss following proposals:

Proposal 1: RAN2 should wait for RAN3’s progress on regenerative payload architecture in order to decide whether to enhance the satellite switch with re-sync for regenerative payload.