3GPP TSG-RAN WG2 Meeting # 127bis 	                R2- 2408717

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.11.2

Source: 	Sony

Title:	Random access for SBFD Operation

Document for:	Discussion and Decision

# Introduction

3GPP has been working for enhancing duplexing operation for TDD based on subband duplex (SBFD) operation at gNB side within a TDD carrier [1-2]. The motivations of enhancing duplexing operation at least for TDD are for improving system capacity, reducing delays, and improving uplink coverage.

A Rel-19 Work Item (WI) on Duplex Evolution is therefore agreed to specify the requirements for FD-TDD where FD-TDD is performed at the gNB, i.e., the gNB can transmit and receive data/signals to/from the UEs at the same time on the same frequency band, whilst the UE is maintained as HD-TDD. That is, full duplex TDD is achieved at the gNB by scheduling a UE in the DL and scheduling another UE in the UL within the same OFDM symbol.

RAN1#116 Agreement:

- For RRC connected mode UEs, at least cell-specific configuration on time and frequency (working assumption) location of SBFD subbands is supported within a TDD carrier.​

RAN1#117 Agreement:

- For RRC connected mode UEs, SBFD subband time period:

  - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

  - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- For cell-specific indication of SBFD subband frequency location, adopt the following option:

  - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

RAN1#116bis Agreement:

- Confirm the working assumption:​

  - Working assumption​

    - For SBFD aware UEs in RRC CONNECTED state, support CBRA and CFRA in SBFD symbols.​

RAN1#117 Agreement:

Confirm the following working assumption.

Working Assumption

For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported.

- For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc.

- For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc.

UE is not required to support both options.

RAN1#117 Agreement:

- Support random access in SBFD symbols for UEs in RRC_IDLE/INACTIVE mode.

RAN1#118 Working Assumption:

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

RAN2#127 Agreements:

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS.

RAN2 to strive for a common SBFD CBRA framework independent of RRC state.

FFS whether/how early indication is used during a SBFD RA procedure.

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

In this contribution, we discuss the support SBFD subband configuration and random access in SBFD operation.

## SBFD Subband Configuration

RAN1 made some progress with several agreements for RRC connected mode UEs: the time and frequency locations of UL subband and DL subband(s) are explicitly configured in a cell specific manner. In other words, the time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs in advance.

For SBFD time location, either one or two patterns can be configured:

- When only one TDD-UL-DL pattern is configured as an example in Figure 1, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- When two TDD-UL-DL patterns are configured as an example in Figure 2, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

Figure 1. SBFD time location with only one TDD-UL-DL pattern configured.

Figure 2. SBFD time location with two TDD-UL-DL patterns configured.

Furthermore, RAN2 made an agreement that Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration).

However, the remaining issue is is how to model the frequency location of the UL SBFD subband. RAN1 seems to have ruled out to model as a bandwidth part (BWP) because the assumption is that the center frequencies should be aligned for configured DL and UL BWP pair. But there could be at least two options:

Option 1. Common frequency resource within a DL BWP: It may be possible to configure the UL SBFD frequency location as “common frequency resource” associated with DL BWP similar to the frequency resource configuration for MBS multicast in the earlier releases (Rel-17/18).

TS 38.211:

### 4.4.6	Common MBS frequency resource

A common MBS frequency resource is a contiguous set of common resource blocks. The starting position NMBS,istart,μ of the common MBS frequency resource i is defined relative to point A and the size of the common MBS frequency resource is given by NMBS,isize,μ. Resource blocks in a common MBS frequency resource are numbered in the same way as resource blocks in clause 4.4.4.4 with NBWP,istart,μ and NBWP,isize,μ replaced by NMBS,istart,μ and NMBS,isize,μ, respectively.

TS 38.331:

#### –	CFR-ConfigMulticast

The IE CFR-ConfigMulticast indicates UE specific common frequency resource configuration for multicast for one dedicated BWP.

CFR-ConfigMulticast information element

-- ASN1START

-- TAG-CFR-CONFIGMULTICAST-START

CFR-ConfigMulticast-r17::= SEQUENCE {

locationAndBandwidthMulticast-r17              INTEGER (0..37949)       OPTIONAL,    -- Need S

pdcch-ConfigMulticast-r17                      PDCCH-Config                             OPTIONAL,    -- Need M

pdsch-ConfigMulticast-r17                      PDSCH-Config                             OPTIONAL,    -- Need M

sps-ConfigMulticastToAddModList-r17            SPS-ConfigMulticastToAddModList-r17      OPTIONAL,    -- Need N

sps-ConfigMulticastToReleaseList-r17           SPS-ConfigMulticastToReleaseList-r17     OPTIONAL     -- Need N

}

SPS-ConfigMulticastToAddModList-r17 ::=	SEQUENCE (SIZE (1..8)) OF SPS-Config

SPS-ConfigMulticastToReleaseList-r17 ::= SEQUENCE (SIZE (1..8)) OF SPS-ConfigIndex-r16

-- TAG-CFR-CONFIGMULTICAST-STOP

-- ASN1STOP

CFR-ConfigMulticast field descriptions

locationAndBandwidthMulticast Frequency domain location and bandwidth for MBS multicast. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 [19] with assumptions as described in TS 38.213 [13], clause 12, i.e. setting N^size_BWP=275. The first PRB is a PRB determined by subcarrierSpacing of the associated BWP and offsetToCarrier corresponding to this subcarrier spacing. If not configured, the UE applies the value of locationAndBandwidth of the DL BWP in which the cfr-ConfigMulticast is configured.

For connected mode UEs, since a UE is switching between different BWPs (note that for TDD DL and UL are pair with same index with aligned center frequency), it may be reasonable to configure the UL SBFD “common” frequency resource associated with a DL BWP, hence it becomes a BWP specific. Examples of this is shown on Figure 3 in which the UL SBFD “common” frequency resource can be added when a UE activates DL BWP#1, and in later time if the UE switches to DL BWP#2, it releases the first the UL SBFD “common” frequency resource and adds a new UL SBFD “common” frequency resource (using add and release function).

Figure 3. Frequency location of the UL SBFD subband is modelled as UL SBFD “common” frequency resource associated with DL BWP.

Option 2. Intersection between cell-specific UL subband and active DL BWP in SBFD symbols: Another way is to fix the frequency location of the UL SBFD subband in the system carrier bandwidth as shown on Figure 4. The UL SBFD is determined by the intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. For example, a UE has an active BWP#1 where the intersection of f2 to f3 defines the UL subband#2 in the current active BWP1. However, if the UE switches to BWP2 (activated), the UE has another intersection of f1 to f2 that defines the UL subband#3 in the new active BWP#2.

Figure 4. Frequency location of the UL SBFD subband is modelled as intersection between cell-specific UL subband and active DL BWP in SBFD symbols.

Comparing these two options, when frequency location of the UL SBFD subband is modelled as intersection between cell-specific UL subband and active DL BWP in SBFD symbols, then this may limit the number of BWPs that can be configured for the UE. However, the benefit may be that Idle and connected UEs can share the same “single” UL SBFD subband which can be configured in SIB1. On the other hand, it may be simpler (less complex) to model the UL SBFD frequency location as “common frequency resource” associated with DL BWP as spec already supports similar configurations for MBS.

Proposal 1: For connected mode UEs, SBFD frequency location is configured within the associated DL BWP named as an UL SBFD “common” frequency resource (i.e., BWP specific configuration).

## RACH configuration for SBFD Subband

Work Item (WI) captures to specify SBFD operation to support random access in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs. The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage. RAN2 agreed that 4-step RACH is supported while 2-step RACH is FFS. Based on the above justification of reducing the delays and increasing the capacity, it is reasonable to support 2-step RACH in UL SBFD symbols as well.

Observation 1: The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Proposal 2: Support 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

RO Type selection:

For SBFD-aware UE, after selecting the SSB, the SSB may be associated with both SBFD RO and legacy RO, that is there are two types of RO for a PRACH transmission. The SBFD-aware UE needs to decide which type of RO to use for its PRACH transmission.

It is obvious that legacy UEs are not able to use SBFD ROs (i.e., they can only use legacy non-SBFD ROs) whilst SBFD-aware UEs can select both SBFD ROs and non-SBFD ROs, and that may result in less use of SBFD ROs and more congestion in non-SBFD ROs. For example, for early deployment of the SBFD feature where number of SBFD-aware UEs are small. In this case of early deployment of the SBFD feature, where SBFD-aware UEs are small, a higher selection probability for SBFD ROs may be needed so that SBFD-aware UEs can select SBFD ROs more often compared to legacy non-SBFD ROs. On the contrary, when the deployment of the SBFD feature is popular (i.e., a significant number of UEs supports the feature), equal selection probability for SBFD ROs and non-SBFD ROs may be needed. Hence, a mechanism to perform some kind of load balancing in the system is needed.

Furthermore, cell-edge UEs may be susceptible to interferences for both legacy ROs and SBFD ROs, but for UE-to-UE cross-link interferences (CLI) in the adjacent subbands in the serving cell, the interference certainly impacts SBFD ROs but not in the legacy ROs. This emits from high transmission power for the cell-edge UEs in the UL subband that contains SBFD ROs, which creates interferences in the adjacent DL subbands and as a result affects the UE’s DL coverage. Hence, in this case, it is preferable that SBFD-aware UE at the cell-edge should have a lower probability for selecting SBFD ROs compared to SBFD-aware UEs in the cell-centre, so that interference at the cell-edge is significantly reduced in practice.

In the above scenarios, the network can manage the RO resources for load balancing by employing a probability PSBFD for a UE to use a SBFD RO (additional RO). For example, if PSBFD = 0.7, then the UE has a probability of 70% in selecting an SBFD RO and 30% probability of selecting a legacy RO. The probability PSBFD can be configured in the SIBs. This will enable the network to perform load balancing on the two different types of ROs.

Observation 2: SBFD-aware UE can access both SBFD ROs and non-SBFD ROs while legacy UE can only access non-SBFD ROs. ROs load balancing mechanism will be beneficial to ensure the ROs resources are optimally utilized.

Proposal 3: For RO Type selection, network to configure selection conditions for SBFD-aware UE to select either SBFD ROs or legacy ROs.

Proposal 4: Introduce a configurable probability PSBFD, where the UE has a probability of PSBFD in selecting an additional RO and a probability of 1 - PSBFD in selecting a legacy RO. The value PSBFD can be configured in the SIB.

RO Type fallback:

If the SBFD-aware UE selects SBFD ROs initially for RACH attempts/transmissions, after a number of retransmissions, the UE should be able to fallback to different selection probabilities. For example if the probability PSBFD, where the UE has a probability of PSBFD in selecting an additional RO and a probability of 1 - PSBFD in selecting a legacy RO, then two probabilites can be swapped. In this case, the new probabilities will be Pnon-SBFD for selecting a legacy RO and 1 - Pnon-SBFD for selecting an additional RO (SBFD). Alternatively, for the fallback, the UE can use a different PSBFD, for exmaple if the PSBFD = 0.7 is for initial transmissions, it may be 0.5 for the fallback, thereby giving a higher probabiliyt of selecting the legacy ROs.

Proposal 5: After a number of RACH retransmissions, a UE should use a different selection probabilities for the additional ROs and legacy ROs, for example swapping the two probabilities.

Early Indication:

In the last meeting, RAN2 discussed whether/how early indication is used during a SBFD RA procedure. In our understanding, the early indication is needed only for scheduling message 3, because if SBFD-aware UE selects SBFD ROs then the network can identify the SBFD-aware UE from the selected SBFD ROs, and it can schedule message 3 in SBFD or non-SBFD PUSCH resources. However, when SBFD-aware UE selects non-SBFD ROs then the network can not identify the SBFD-aware UE, hence the network should schedule message 3 to non-SBFD PUSCH resources. Propopents for early idication think that the early indication is needed in order the network to be able to schedule message 3 of SBFD-aware UE to SBFD PUSCH resources. If this is needed then preamble partitioning on non-SBFD ROs is the simpliest approach.

Observation 3: The early indication is needed only for scheduling message 3 on the SBFD PUSCH resources for SBFD-aware UEs when PRACH is selected from non-SBFD ROs.

Proposal 6: For early indication, the following methods can be used to distinguish an SBFD UE from a legacy UE at the gNB:

- PRACH from SBFD slot comes from SBFD UE

- Distinguished using different RO, i.e., PRACH from RO belonging to the additional (SBFD) PRACH configuration in PRACH configuration Option 2, comes from SBFD UE, whilst PRACH from RO belonging to legacy PRACH configuration comes from legacy UE.

- If the SBFD UE and legacy UE share an RO in a legacy UL slot, preamble partitioning is used on these ROs to distinguish between SBFD UEs and legacy UEs.

## Conclusions

In this contribution, we have discussed the support SBFD subband configuration and random access in SBFD operation. We have the following observations and proposals:

Observation 1: The motivation of supporting random access in UL SBFD symbols is to increase RACH capacity, reduce delays, and improve the uplink coverage.

Observation 2: SBFD-aware UE can access both SBFD ROs and non-SBFD ROs while legacy UE can only access non-SBFD ROs. ROs load balancing mechanism will be beneficial to ensure the ROs resources are optimally utilized.

Observation 3: The early indication is needed only for scheduling message 3 on the SBFD PUSCH resources for SBFD-aware UEs when PRACH is selected from non-SBFD ROs.

Proposal 1: For connected mode UEs, SBFD frequency location is configured within the associated DL BWP named as an UL SBFD “common” frequency resource (i.e., BWP specific configuration).

Proposal 2: RAN2 agrees to support 2-step RACH in UL SBFD symbols for RRC_CONNECTED and RRC_IDLE/INACTIVE UEs.

Proposal 3: For RO Type selection, network to configure selection conditions for SBFD-aware UE to select either SBFD ROs or legacy ROs.

Proposal 4: Introduce a configurable probability PSBFD, where the UE has a probability of PSBFD in selecting an additional RO and a probability of 1 - PSBFD in selecting a legacy RO. The value PSBFD can be configured in the SIB.

Proposal 5: After a number of RACH retransmissions, a UE should use a different selection probabilities for the additional ROs and legacy ROs, for example swapping the two probabilities.

Proposal 6: For early indication, the following methods can be used to distinguish an SBFD UE from a legacy UE at the gNB:

- PRACH from SBFD slot comes from SBFD UE

- Distinguished using different RO, i.e., PRACH from RO belonging to the additional (SBFD) PRACH configuration in PRACH configuration Option 2, comes from SBFD UE, whilst PRACH from RO belonging to legacy PRACH configuration comes from legacy UE.

- If the SBFD UE and legacy UE share an RO in a legacy UL slot, preamble partitioning is used on these ROs to distinguish between SBFD UEs and legacy UEs.

## References

# [1] RP-232763, 3GPP TR 38.858 V2.0.0, Study on Evolution of NR Duplex Operation.

# [2] RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, RAN#104.