3GPP TSG-RAN WG2 Meeting #127bis	R2-2408718

Hefei, China, October 14th – 18th , 2024

Agenda Item: 	8.5.4

Source:		Sony

Title:			Considerations on adaptation for paging signals

Document for:	Discussion & Decision

# Introduction

At RAN#105 meeting, a revised version for the WID for Enhancements of network energy savings for NR was approved including the following objectives [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Note: No modification of SSB will be discussed under this objective - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

At RAN2#127 [2], the following agreements were made:

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

Also it was agreed to have the following post email discussion [3].

[POST127][109][NES] (OPPO)

In this contribution, we discuss our view on some RAN2 related aspects related to adaptation of paging signals.

# Discussions on Adaptation of paging signals

The idle and inactive state UEs requires the network to be active at least in a UE DRX bases. Though the number of Idle/Inactive UEs may be unknown, therefor gBN has be awake constantly to cover all possible paging occasion, since the POs of UEs are distributed in time based on the IMSI.

SIBs and reference signals need to be transmitted periodically to support the UE measurements and this is done in relation to paging occasions (paging frames).

During nighttime, the UE activity is low, and hence the load on the network is also low, which would give opportunities for the network/base station to sleep.

Though anyway all UEs in idle mode needs to be served. The SSB needs to be transmitted as well as reference symbols for measurements and demodulation, and potential paging message needs to be transmitted in respectively Paging Frame/Paging Occasion.

As agreed in RAN2#125bis, the two options to a discuss and down select from are

For adaptation of paging occasions in time domain, RAN2 to study 
a) bundle paging frames and 
b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

At RAN2#127 [2] initial discussions started on the adaptation of paging signal for network power save purpose. It was agreed to further discuss direction related to N extension vs PF bundling in an email discussion.

We like to share our view on a slightly modified proposal where we give a slightly other perspective than captured tin the agreements.

As for option b above we agree that increasing the interval between the PF may allow the gNB to sleep for a longer period.

It would be beneficial to instead group the UEs to a small set of paging occasions as long as the rate of paging messages is low in the system. Then the gNB transmitter and receiver only needs to be awake around these occasions in order to serve the existing UEs in the network. The UEs will read SIBs and make RRM measurements around these paging occasions as well so to serve them the SSBs only need to be transmitted around the paging occasions.

The paging occasions for a group may be defined by a new “IMSI” used in the whole group. This new parameter is used instead of the UEs IMSI when calculating the paging occasion as defined in 38.304 [4], the UEs will thereby use the same paging occasion as indicated in Figure 1. A similar solution to move paging occasions was used to avoid paging collisions for Multi-SIM UEs in LTE, Rel-17.

Proposal 1: Specify grouping the paging occasions for UEs in idle or inactive state based when camping in an “active” NES cell.

Figure 1: Group the DRX of the UEs in the cell allowing the gNB to sleep between the group DRX

# Summary

In this contribution, we have discussed our views on aspects related to. adaptation of paging signals

Proposal 1: Specify grouping the paging occasions for UEs in idle or inactive state based when camping in an “active” NES cell.

# References

- RP-242354, Revised WID: Enhancements of network energy savings for NR

- Minutes RAN2#127

- POST127][109][NES] (OPPO)