3GPP TSG-RAN WG2 Meeting #127bis	R2-2408721

# HefeiChina,  October 14th – 18th ,  2024                                       Revision of R2-2407227

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.305 | CR | 0176 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DRX and PRS alignment for positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sony, Ericsson, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the WID: - Specify solutions for alignment between (e)DRX in RRC_INACTIVE and PRS configurations [RAN2]. RAN2 discussed both solutions a) to align PRS with (e)DRX and b) (e)DRX with PRS. However, the later (b) is not captured in the specification. Based upon RAN2#125bis agreement, stage2 change would be needed to reflect that it is possible to fulfil the DRX and PRS alignment from gNB. Agreement: Stage 3 impact is not specified any more to align PRS to fixed (e)DRX. Can consider towards next meeting if some modification is needed to the related NOTE in stage 2. gNB is aware of UE specific DRX/eDRX values which it would have received from AMF in the Core Network Assistance Information to assist UE configuration and paging in RRC_INACTIVE. gNB also receives assistance data from LMF for positioning purpose; for example for LPHAP UEs to configure SRS validity area in TS 38.455 section 9.2.27: Positioning Validity Area Cell List; O; 9.2.93; YES; ignore. Validity Area Specific SRS Information; O; 9.2.96; YES; ignore. gNB also receives PRS configuration from other cases such as to be able to configure the measurement gaps appropriately. This implies gNB has necesaary information from AMF and LMF on DRX and PRS peridocity and gNB can align these two. |
| Summary of change: |  | Include the change that gNB can align (e)DRX and PRS in section 7.9 Impact Analysis Impacted 5G architecture options: NR SA, Impacted functionality: DRX and PRS alignment from gNB. Only gNB functionality impacted. Inter-operability: - No inter-operability foreseen. However, if NW does not implement the CR - UE may not be able to receive PRS when it wakes up after (e)DRX cycle. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-19 WI will be incomplete. Low Power High Accuracy Positioning may not be achieved in some deployments. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The CR is based upon version 18.2.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Beginning of Changes

## 7.9	Positioning in RRC_INACTIVE state

Positioning may be performed when a UE is in RRC_INACTIVE state. Any uplink LCS or LPP message can be transported in RRC_INACTIVE state. If the UE initiated data transmission using UL SDT, the network can send DL LCS, LPP, RRC Release message (e.g. to configure SRS for UL positioning, if it is supported) and SP Positioning SRS Activation/Deactivation MAC CE to the UE without the need of state transition.

Periodic and Semi-persistent UL-SRS transmission for positioning can be supported in RRC_INACTIVE. The UL-SRS for positioning configuration in RRC_INACTIVE state may be associated with an SRS Validity Area. The SRS Validity Area contains a list of cells in which the SRS for positioning configuration in RRC_INACTIVE state is valid. When configured with an SRS configuration for positioning along with SRS Validity Area, if the UE reselects to another cell within the SRS Validity Area during SRS transmission, the UE continues the SRS transmission, subject to validation for SRS transmission as specified in TS 38.321 [39] and TS 38.331 [14]. When the UE reselects out of the SRS for positioning Validity Area during SRS transmission, the UE may send an "RRC Resume Request" message to the network for SRS configuration request.

NOTE:	Aperiodic UL-SRS transmission for positioning is not supported in RRC_INACTIVE.

The SRS for positioning configuration in RRC_INACTIVE state may be pre-configured in the target device. The target device may send an "RRC Resume Request" message to the network when a configured periodic or triggered location event has been detected to request activation of the pre-configured SRS for positioning. For preconfigured multiple SRS configurations, the UE is configured with only one SRS for positioning configuration for each validity area.

The gNB, while configuring the (e)DRX cycle for UEs in RRC_INACTIVE state (described in clause 9.2.10 of TS 38.300 [52]), may also align the (e)DRX cycle with DL-PRS periodicity.

End of Changes