3GPP TSG-RAN WG2 Meeting #127bis		R2-2408722

Hefei, China, Oct 14th – 18th, 2024

Agenda Item: 	8.7.6

Source:		Sony

Title:	Recommended bit rate based XR rate control

Document for:	Discussion & Decision

# Introduction

XR WID was updated in RANP_105 to include following objectives for uplink congestion signalling [1]:

Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion

In this contribution we propose to use the recommended bit rate MAC-CE for XR rate control and some details.

# Discussions

RAN aware XR rate control can help the XR application to adjust its codec bit rate according to radio conditions and ensure user experience.

RAN2 agreed in the past that it is feasible for RAN to estimate the congestion information per QoS flow and per DRB in both UL and DL direction. For UL, RAN can do it without any UE impacts. If a RAN node is congested, then it can signal the congestion indication to the XR application which in turn will adjust the XR data rate for DL direction. At the same time, RAN should be able to reduce the data rate for XR application on the uplink direction.

For rate control, we think the use of Recommended bit rate MAC-CE can be enhanced and used in this case. Existing RRC messages can be used for this purpose.

Proposal 1: Both UL/DL bit rate recommendation and UL/DL bit rate recommendation query RRC messages are supported for XR UL rate control.

On the question of per QoS flow/per DRB, current MAC-CE is per LCH ID. We should continue the same. Also existing data rate values seems sufficient and no need to introduce new values.

Proposal 2: On per QoS flow/per DRB, current MAC-CE is per LCH ID. We should continue the same.

# Summary

In this contribution, we have the following proposals for RAN2 to agree:

Proposal 1: Both UL/DL bit rate recommendation and UL/DL bit rate recommendation query RRC messages are supported for XR UL rate control.

Proposal 2: On per QoS flow/per DRB, current MAC-CE is per LCH ID. We should continue the same.

# References

- RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3,” RAN#105.

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

Annex: 38.300

#### 16.2.1.1	RAN-assisted codec adaptation

RAN-assisted codec adaptation provides a means for the gNB to send codec adaptation indication with recommended bit rate to assist the UE to select or adapt to a codec rate for MMTEL voice or MMTEL video. The RAN-assisted codec adaptation mechanism supports the uplink/downlink bit rate increase or decrease. For a bearer associated with configuration of MBR greater than GBR, the recommended uplink/downlink bit rate is within boundaries set by the MBR and GBR of the concerned bearer.

For uplink or downlink bit rate adaptation, gNB may send a recommended bit rate to the UE to inform the UE on the currently recommended transport bit rate on the local uplink or downlink, which the UE may use in combination with other information to adapt the bit rate, e.g. the UE may send a bit rate request to the peer UE via application layer messages as specified in TS 26.114 [24], which the peer UE may use in combination with other information to adapt the codec bit rate. The recommended bit rate is in kbps at the physical layer at the time when the decision is made.

The recommended bit rate for UL and DL is conveyed as a MAC Control Element (CE) from the gNB to the UE as outlined in Figure 16.2.1.1-1.

Figure 16.2.1.1-1: UL or DL bit rate recommendation

Based on the recommended bit rate from the gNB, a UE may initiate an end-to-end bit rate adaptation with its peer (UE or MGW). The UE may also send a query message to its local gNB to check if a bit rate recommended by its peer can be provided by the gNB. The UE is not expected to go beyond the recommended bit rate from the gNB.

The recommended bit rate query message is conveyed as a MAC CE from the UE to the gNB as outlined in Figure 16.2.1.1-2.

Figure 16.2.1.1-2: UL or DL bit rate recommendation query

A prohibit timer can be configured per logical channel by the network to limit UEs sending frequent query MAC CEs. Independent prohibit timers are used for each direction (uplink and downlink) to prohibit the UE from retransmitting exactly the same query MAC CE to the gNB during the configured time.