3GPP TSG-RAN WG2 Meeting # 127bis 	                R2-2408723

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.12.2

Source: 	Sony

Title:	Enhancement for Asymmetric DL sTRP/UL mTRP

Document for:	Discussion and Decision

# Introduction

In RAN#105, revised WI of NR MIMO Phase 5 has been agreed which contains the following bullet point from [1]:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2:

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources.

RAN1#116bis Agreement:

Support applying PL offset on PDCCH-order PRACH towards a UL TRP in FR1.

- Note: The DL reference timing determination for PDCCH-order PRACH transmission to an UL TRP is still based on the DL RS defined in current RAN4 specification.

- Above is subject to a separate UE capability signaling.

RAN1#117 Agreement:

Introduce a new RRC parameter per BWP/CC to indicate that two separate SRS CLPC adjustment states are configured for SRS in a BWP/CC.

RAN1#117 Agreement:

For the association between PL offset and joint/UL TCI state, support the following:

- Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

RAN1#117 Agreement:

For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS.

RAN1#118 Agreement:

For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH.

RAN1#118 Agreement:

Study whether to support Type 3 PHR reporting in a serving cell/BWP where the UE is configured with two separate SRS CLPC adjustment states.

- Continue to study whether to support including PL offset in the calculation of Type 3 PHR.

In this contribution, we discuss some enhancements for asymmetric DL sTRP/UL mTRP deployment scenarios and provide some proposals at the end.

## Enhancement for Asymmetric DL sTRP/UL mTRP

In commercial NR deployment [2], it is identified that in some dense macro cell operation, UL traffic is a bottleneck. Nevertheless, it is well-known that a HetNet deployment may offload some UL traffic from the macro-gNB to the small cells. Hence, the small cells are also beneficial for UE in the cell edge performing uplink transmission as the UE can use a lower transmission power thereby reducing UL interferences. However, for cases where UL traffic is the issue rather than the DL, the legacy HetNet deployment will introduce DL interferences among the TRPs in a HetNet cell. A deployment scenario of asymmetric DL sTRP/UL mTRP could help in this case. An example of deployment scenario for asymmetric DL sTRP/UL mTRP is shown in Figure 1. It is assumed that UL TRP, which is capable of UL reception only and operates as UL-only cell, is deployed within the coverage of macro cell TRP which provides both DL and UL, and its coverage is overlapped with the macro cell TRP because the UL TRP does not provide any DL signals. A UE transmit to the UL TRP and receives DL signal from the macro cell TRP as shown in Figure 1.

Figure 1: Example of asymmetric DL sTRP/UL mTRP deployment scenario

DL sTRP should be considered in the deployment scenario because the UL TRP does not provide any DL signal/channel including CORESET(s) for a UE. To operate DL sTRP, an ideal fronthaul connection is established between the macro cell TRP and each of UL TRPs. In addition, to simplify the scenario, the UE should support and be configured with only one UL-only TRP at a given time (in addition to the legacy macro TRP), and from system perspective a UE should initially receive information about the existence of the UL-only TRPs in the cell from the network via system information (SIB) or RRC signalling.

Furthermore, if a UE is close to macro-cell, it is not reasonable to configure an UL-only TRP. Therefore, how to determine if a UE is close to anyone of the UL-only TRPs is important. In practice, if the UE transmits some signals (like SRS) and these signals are received on both macro TRP and UL-only TRP, the network can determine whether the UE is close to an UL-only TRP. Subsequently, the network can schedule PDCCH order to the UE to transmit PRACH towards the UL-only TRP to stablish synchronisation and pathloss.

Alternatively, it may be also reasonable that the UE discovers if it is close to the UL-only TRP by measuring DL RSRP of the downlink TRP and comparing against a pre-configured RSRP threshold. If the DL RSRP is below to the RSRP threshold, the UE assumes the signal toward macro TRP is very weak and must look for an UL-only TRP. In this case, the UE may try to transmit SRS or PRACH periodically with beam sweeping toward UL-only TRP.

Proposal 1: For asymmetric DL sTRP/UL mTRP, a UE should only support and be configured with only one UL-only TRP at a given time (in addition to the legacy macro TRP).

Proposal 2: For asymmetric DL sTRP/UL mTRP, the existence of the UL-only TRPs in the cell should be configured from the network via system information or dedicated signalling.

UL-only cell TA determination:

In the deployment of DL sTRP/UL mTRP scenarios, there are two path-losses (PL) at the UE, one for the macro linked to DL TRP and another for the UL-only TRP, and the PL difference between UE to DL TRP and UE to UL-only TRP could be very large. In addition, the propagation delay could also be large, hence TA would be significantly different between macro TRP and UL-only TRP.

Figure 2: Determination of UL propagation delay at the UL-only TRP (URP).

As shown on Figure 2 above, the UE receives the DL slot/subframe from the macro-TRP where the propagation delay is tp1 at the UE receiver (Rx). Then the UE transmits random-access preamble (PRACH) based on UE Rx timing (from macro-TRP) toward the UL-only TRP. The propagation delay between the UE and the UL-only TRP is tp2.  Since the UE transmits its PRACH at an offset tp1 from the boundary of the DL slot, the UL-only TRP will receive the UE’s PRACH at an offset of tp1 + tp2 from the boundary of the DL slot. Meanwhile the macro-TRP will receive the PRACH at an offset of 2*tp1 from the boundary of the DL slot. The network can therefore work out the propagation delay tp2, from the UE to the UL-only TRP (URP).

The network determines the UL propagation delay toward the UL-only TRP based on Figure 2 as follows:

tp2 = (tp1 +tp2)- (2*tp1)/2,

Where tp1 is one-way propagation delay between the network and the UE based on the macro-TRP, 2*tp1 is already known from TA value of the macro-TRP. The quantity of tp1 +tp2 is measured at the UL-only TRP receiver, hence it is known value from the measurement. Note that the DL reference timing for the UL-only TRP is based on DL slot/subframe from the macro-TRP.

Therefore, TA for UL-only TRP is twice of one-way propagation delay = 2*tp2.

In other words, TA for UL-only TRP (URP) is twice of one-way propagation delay = 2*((tp1 +tp2)- (2*tp1)/2).

The network then can signal to the UE the legacy TA for the macro TRP and a new TA for the UL-only TRP, hence the UE should maintain two TAs.

Proposal 3: For asymmetric DL sTRP/UL mTRP, support two separate TAs: one legacy TA for macro TRP and another new TA for the UL-only TRP.

## Conclusions

In this contribution, we have discussed some enhancements for asymmetric DL sTRP/UL mTRP deployment scenarios and we have the following proposals.

Proposal 1: For asymmetric DL sTRP/UL mTRP, a UE should only support and configured only one UL-only TRP at a given time (in addition to the legacy macro TRP).

Proposal 2: For asymmetric DL sTRP/UL mTRP, the existence of the UL-only TRPs in the cell should be configured from the network via system information (SIB).

Proposal 3: For asymmetric DL sTRP/UL mTRP, support two separate TAs: one legacy TA for macro TRP and another new TA for the UL-only TRP.

## References

[1] RP-242394, “NR MIMO Phase 5,” RAN#105.

[2] RWS-230248, “Views on Rel-19 MIMO/UL enhancements,” NTT DOCOMO, 3GPP TSG RAN Rel-19 workshop.