3GPP TSG-RAN WG2 Meeting # 127bis 	                R2-2408724

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.6.4

Source: 	Sony

Title:	Conditional Intra-CU LTM

Document for:	Discussion and Decision

# Introduction

In RAN#104, a revised WI of mobility enhancements for NR has been agreed which contains the following bullet point [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM.

  - Aim to support conditional LTM including subsequent LTM.

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC.

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint.

In this contribution, based on the WID, we discuss how to support the conditional Intra-CU LTM.

## Conditional Intra-CU LTM

Rel-18 LTM (L1/L2 triggered mobility) is specified as shown on Figure 1 below. The LTM procedure is started from the network side after receiving L1 measurement reports from a UE, and then the network sends an LTM command (via MAC CE) to the UE to change the serving cell. The target cell is already prepared where the UE received all necessary configurations via RRC signalling. The main motivation is to reduce the mobility latency.

Figure 1. Rel-18 LTM procedure (Figure 9.2.3.5.2-1. Signalling procedure for LTM) [2]

The detailed procedure for LTM is as follows based on Figure 1 above, copied here from [2]:

1.	The UE sends a MeasurementReport message to the gNB. The gNB decides to configure LTM and initiates LTM preparation.

2.	The gNB transmits an RRCReconfiguration message to the UE including the LTM candidate configurations.

3.	The UE stores the LTM candidate configurations and transmits an RRCReconfigurationComplete message to the gNB.

4a.	The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command. The UE may activate and deactivate TCI states of LTM candidate cell(s), as triggered by the gNB.

4b.	The UE may perform UL synchronization with LTM candidate cell(s) before receiving the cell switch command, by using UE-based TA measurement, if configured, and/or by transmitting a preamble towards the candidate cell, as triggered by the gNB. When UE-based TA measurement is configured, UE acquires the TA value(s) of the candidate cell(s) by measurement. UE performs early TA acquisition with the candidate cell(s) as requested by the network before receiving the cell switch command as specified in clause 9.2.6. This is done via CFRA triggered by a PDCCH order from the source cell, following which the UE sends preamble towards the indicated candidate cell. In order to minimize the data interruption of the source cell due to CFRA towards the candidate cell(s), the UE does not receive random access response from the network for the purpose of TA value acquisition and the TA value of the candidate cell is indicated in the cell switch command. The UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity.

5.	The UE performs L1 measurements on the configured LTM candidate cell(s) and transmits L1 measurement reports to the gNB. L1 measurement should be performed as long as RRC reconfiguration (step 2) is applicable.

6.	The gNB decides to execute cell switch to a target cell and transmits an LTM cell switch command MAC CE triggering cell switch by including a target configuration ID which indicates the index of the candidate configuration of the target cell, a beam indicated with a TCI state, or beams indicated with DL and UL TCI states, and a timing advance command for the target cell, if available. The UE switches to the target cell and applies the candidate configuration indicated by the target configuration ID.

7.	The UE performs the random-access procedure towards the target cell, if UE does not have valid TA of the target cell as specified in clause 5.18.35 of TS 38.321[6].

8.	The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random-access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

The steps 4-8 can be performed multiple times for subsequent LTM cell switch executions using the LTM candidate configuration(s) provided in step 2.

In Rel-19, the WID contains to specify support of conditional Intra-CU LTM in order to increase robustness, further reduce the latency and mobility failures. It is our understanding that the conditional LTM for intra-CU is like legacy CHO where the network configures some conditions (e.g., based on L1 measurements) for UE to evaluate and then when the condition(s) are met, the LTM will be executed. In other words, a UE receives an LTM command in advance that linked to some conditions to evaluate to execute the LTM cell switch for a target cell, and if the conditions for the LTM command are satisfied for the target cell, the UE executes LTM cell switch command.

The conditions could be L1 measurements which can be based on event LTM3 (Beam of candidate cell becomes amount of offset better than beam of serving cell) or LTM5 (Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2).

Observation 1: The conditional LTM for intra-CU is like legacy CHO where the network configures some conditions (e.g., based on L1 measurements) for UE to evaluate and then when the condition(s) are met, the LTM will be executed.

For Rel-19, it is more likely that the LTM cell switch command in step 6 is modified to link some conditions that a UE to evaluate before executing the cell switch command.

Observation 2: For Rel-19, it is more likely that the LTM cell switch command in step 6 is modified to link some conditions that a UE to evaluate before executing the cell switch command.

Based on the above observation, one issue is whether one cell or multiple cells are supported to be evaluated for conditional LTM (i.e., after receiving the cell switch command(s)). Similar to legacy CHO, it is more likely that a UE will evaluate more than one cell. This will enable to support conditional LTM for subsequent LTM as well. The number of cells for conditional Intra-CU LTM should be small, probably not more than three.

Proposal 1: For conditional Intra-CU LTM, the network should configure some conditions (e.g., based on L1 measurements) for UE to evaluate and then when the condition(s) are met, the LTM will be executed.

Proposal 2: For conditional Intra-CU LTM, multiple cells are supported to be evaluated for conditional LTM.

Proposal 3: The number of cells for conditional Intra-CU LTM should be small, probably not more than three.

## Conclusions

In this contribution, we have discussed how to support the conditional Intra-CU LTM, and we have the following observations and proposals:

Observation 1: The conditional LTM for intra-CU is like legacy CHO where the network configures some conditions (e.g., based on L1 measurements) for UE to evaluate and then when the condition(s) are met, the LTM will be executed.

Observation 2: For Rel-19, it is more likely that the LTM cell switch command in step 6 is modified to link some conditions that a UE to evaluate before executing the cell switch command.

Proposal 1: For conditional Intra-CU LTM, the network should configure some conditions (e.g., based on L1 measurements) for UE to evaluate and then when the condition(s) are met, the LTM will be executed.

Proposal 2: For conditional Intra-CU LTM, multiple cells are supported to be evaluated for conditional LTM.

Proposal 3: The number of cells for conditional Intra-CU LTM should be small, probably not more than three.

## References

[1] RP-242356, “Revised Work Item: NR mobility enhancements Phase 4,” RAN#105.

[2] TS 38.300 V18.0.0, Section 9.2.3.5.