3GPP TSG-RAN WG2 Meeting #127bis						      R2-2408735

Hefei, P. R. China, 14 – 18 October, 2024

Title: 		Discussion on LCM for UE-sided model for Beam Management use case

Source: 	Huawei, HiSilicon

Agenda item:	8.1.2.2

Document for:	Discussion/Decision

# Introduction

In RAN2 #127 meeting, RAN2 made the following agreements for the UE-side model for BM case based on the [POST126][032] email discussion [1]. The majority FFSs was contained in the LS to RAN1 [2] to check RAN1 view (marked as yellow), and the paper mainly discusses the FFSs not covered by the LS (marked as blue) and some other important issues.

Agreements on definitions 1 Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling) 2 Applicable functionalities refers to functionalities that the UE is ready to apply for inference 3 Activated functionalities refers to functionalities already enabled for performing inference Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

# Discussion

## Applicable functionality determination

The network side additional conditions are needed to be known to the network as part of the functionality applicability reporting procedure, to ensure the consistency across the training and inference for UE-sided model. NW-side additional condition is assumed as associated ID in RAN2 and the overall procedure to align the training and the inference can be described as follows:

- First, the UE trains the model based on the dataset and associates it with the related associated ID, if provided by the network.

- The UE reports the applicable functionality to the NW, which may be associated with the NW-side additional condition identified by the associated ID.

- NW selects the applicable functionality and determines the inference configuration based on the NW-side additional condition.

For NW-side additional condition, we have the following FFS for step 3, and RAN2 has included a corresponding question in the LS to RAN1 [2].

- “FFS for step 3:  2) Network may provide NW-side additional condition.  FFS on the RRC signalling and whether it is mandatory or optional.

- Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?”

In our view, associated ID is optional for the whole LCM process, i.e. even for training phase the NW might not provide it and the UE should still be able to determine the applicability of the functionality based on implementation, but this needs to be confirmed by RAN1 based on the question above.

However, we believe associated ID can be optional in step 3 regardless of RAN1 reply which is related to functionality applicability reporting options, as discussed in previous RAN2 meetings. The UE reporting behavior can be as follows depending on whether the network indicates or does not indicate the NW-side additional condition in step 3, i.e., the associated ID:

- If the network indicates the associated ID, the network can indicate the network-side additional condition it is interested in. In this case, the UE reports only the applicable functionalities corresponding to the indicated associated ID(s). This is aligned with Option 2 discussed in [POST126][032] email discussion [1], in which the UE decides the applicable functionalities based on NW-side additional conditions indicated by the NW:

- “Option 2: UE decision: UE decides the applicable functionalities based on NW-side additional conditions indicated by the NW and UE-side additional conditions internally known by UE.”

- If the network does not indicate the associated ID, the UE can determine which of the functionalities are applicable from UE perspective considering model availability and UE side additional conditions and report those functionalities together with the corresponding associated IDs, if available. Based on this, the network can select and configure the selected applicable functionality for inference according to the current network-side addition condition. This is aligned with Option 1 discussed in [POST126][032] email discussion [1].

- “Option 1: Joint decision by UE and NW: UE reports the applicable functionalities and the related associated ID to the NW to make the final decision.”

For option 1, when the NW side additional condition changes, the NW can do an update based on the associated ID reported earlier. In some situations, option 1 may cause some signaling overhead in case the network is able to currently support only a very few NW-side additional conditions/associated IDs. For, example, if the UE has applicable functionality for associated ID #1, #2, and #3, but the current NW supports only ID#4, then the UE reporting of other IDs is meaningless.

For option 2, NW may indicate the interested associated IDs when requesting the applicable functionality reporting, and then the UE only needs to report the applicable functionalities associated with the indicated associated IDs. Compared with option 1, the UE has more information (i.e. associated ID), and thus it can do the filtering. However, if the NW-side additional condition changes to the one not requested earlier from the UE, the network will have to ask the UE to report the applicable functionalities again, with the new associated ID.

Based on the above analysis, both option 1 and option 2 are useful and the choice between them will depend on the NW implementation and current NW-side additional conditions. We think it should be left to NW implementation which option to use at a certain time. If the NW indicates associated ID(s) to UE, the UE checks the indicated associated ID for determination of applicable functionalities; otherwise, the UE only checks UE-side additional conditions and model availability and reports applicable functionalities together with the supported associated IDs, if available.

Proposal 1: For applicable functionalities determination, both option 1 (joint decision by UE and NW) and option 2 (UE decision) should be supported in the following way:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available. (Option 1)

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition. (Option 2)

## Applicable functionality reporting procedure

For step 4, i.e., the applicable functionality reporting, we have the following FFS.

- FFS for step 4:  2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

Based on the previous discussion, the UE can report the applicable functionality in two ways:

- Proactive reporting: the UE reports applicable functionalities based on the network configuration and when applicability of some functionality changes the UE updates the information

- Reactive reporting: the UE reports applicable functionalities as requested by the network configuration, the reporting is one-shot.

The two procedures are quite similar. The main potential difference mentioned by the companies lies in the contents of the configuration provided in Step 3 and whether Step 5 is needed.

(a)                                                                      (b)

Figure 1. Applicable functionality reporting: (a) proactive reporting, and (b) reactive reporting

It was agreed that the UAI framework will be used at least for proactive reporting. Whether the UAI can be used also for the reactive reporting is FFS. In our opinion UAI is appropriate for both types of reporting. We think that the fact the UE reports applicable functionalities upon being configured to do so by the network, is exactly what reactive reporting means. Such behavior is a usual way of implementing UAI reporting for various features and we plan to follow this principle for applicable functionality reporting, as per the previous agreement:

- “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

Observation 1: UAI can be used to support both proactive and reactive applicable functionality reporting.

Some companies indicated the RRCReconfigurationComplete is necessary for the handover cases, which can support the applicable functionality reporting as soon as the UE moves to the target node.  This is related to the topic of AIML continuity which needs to consider also the granularity of the associated IDs, i.e. an area to which they apply. On this aspect, RAN1 has the following working assumption from RAN1#118 meeting.

RAN1#118 meeting progress: Agreement Confirm the following Working assumption. Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study)

Based on the above working assumption, associated IDs for one applicable functionality may not be the same from one cell to another cell. It means that when the UE moves from one cell to another cell, the UE needs to update the reporting. For example, first the UE is in cell 1, it reports the applicable functionality and its associated IDs #1, #2, and #3. When the UE moves to cell 2, it reports the applicable functionality and its associated IDs #5, #6, and #7. Associated IDs #1, #2, and #3 should be allocated and valid in cell 1, while associated IDs #5, #6, and #7 should be allocated and valid in cell 2.

Each time the UE changes the serving cell, the legacy applicable functionality information will be invalid. When the UE connects to a new cell, the NW needs to trigger the applicable functionality reporting for the new cell, or the UE needs to update and report the applicable functionality automatically. It means that each time the cell change happens, we need to go through the applicable functionality reporting and inference configuration once again in order to configure the proper functionality in the new cell, which can cause AI/ML operation interruption. In principle, this does not cause major issues as the UE may simply perform legacy BM operation during this short period of time. However, it would be useful to have a mechanism to guarantee the continuity of the AI/ML operation even the associated ID is allocated locally in one cell.

Similar issue exists if the associated IDs can be applicable for multiple cells. When the UE moves from one cell group to another cell group, the previous applicable functionality information can be invalid and the UE needs to update the reporting.

Proposal 2: As the associated ID is not globally unique, it is proposed RAN2 to discuss how to guarantee the continuity of the AI/ML operation during mobility between the cells.

As the baseline to support handover scenario, the UE can send the UAI after the handover since there is no urgency for the applicable functionality reporting on the target node as the UE/NW may rely on legacy beam management operation. If the target node needs to know the applicable functionality urgently, for example, to guarantee the service continuity of the AI/ML, applicable functionality reporting in the HO complete message does not solve the problem, since the target node needs the applicable functionality information before preparing the HO configuration.

Observation 2: Usage of RRCReconfigurationComplete message for applicable functionality reporting does not improve the continuity of AIML feature.

Proposal 3: Applicable functionality reporting via RRCReconfigurationComplete is not supported.

If a continuity of AIML based beam management is needed during handover, one of the options is for the source node to help the target node to collect the applicable functionality information. For example, the source node can get the applicable functionality information via UAI based on the inter-gNB interaction with the target node prior to the handover, and then the target node can provide the configurations for the applicable functionality directly in the HO configuration.

Proposal 4: For handover case, as a baseline, applicable functionality reporting can be sent to the target cell via UAI right after the handover is completed based on UAI configuration provided in the HO command by the target cell.

Proposal 5: If the service continuity of the AI/ML should be supported, RAN2 should focus on solutions allowing for proper AIML configuration to be provided already in HO command, e.g. the source node can help collect the applicable functionality information for the target node.

## Applicability status changes

This clause discusses the details of the applicable functionality reporting. In the email discussion [2], we have one open issue:

- whether the UE needs to report the non-applicability of the functionality

When the UE initiates the first applicable functionality reporting upon receiving the UAI in proactive reporting or initiates the applicable functionality reporting when configured by the network in reactive reporting, the UE only needs to report the applicable functionalities since NW can consider the other functionalities are non-applicable.

It seems a common understanding that a functionality as indicated by a UE may not remain applicable or non-applicable all the time. UE internal conditions and/or NW side conditions and/or the availability of the functionality can impact the status of the functionality. The status of a functionality may change from “non-applicable” to “applicable” or from “applicable” to “non-applicable”. Once an applicable functionality is activated, the functionality has to be deactivated if it becomes non-applicable, otherwise system performance will degrade. Also, when applicable functionality reporting was requested by the NW before, the UE should report the functionality once it becomes applicable. Therefore, in the proactive reporting, when the functionality has been previously reported as applicable or non-applicable and its status changes, such information needs to be delivered to the network.

Proposal 6a: When the UE initiates applicable functionality reporting for the first time in a cell, the UE only needs to report the applicable functionalities.

Proposal 6b: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, such information needs to be delivered to the network.

However, it should be noted that as opposed to the changes of NW side conditions, UE internal conditions are unseen and unpredicted from NW perspective. Even if UE could have the flexibility to timely indicate the status changes of the applicable functionality due to UE internal conditions, signaling overhead is introduced, especially for the cases where the changes among different functionality statuses are frequent. On the other hand, if status updates frequency is limited by the network (e.g. using a prohibit timer), then there may be a delay in UE reporting a functionality as non-applicable. In this case, the NW may need to configure more resources for performance monitoring, but this will introduce additional resource overhead.

Observation 3: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

From network perspective, it is preferential to choose such functionalities for inference which are likely to be used for a longer time and which are not prone to frequent status changes, e.g. due to dynamic changes of UE-side conditions. To help the network to choose proper configurations, it would be useful if on top of bare applicability information, the UE also provided also some kind of “stability of applicability” indication. For example, if the UE knows a certain inference operation is likely to exceed its processing power or that the model is only applicable for the current UE speed, the network should be aware of this. Such indication does not have to mention an explicit reason why the functionality status may change, but instead the UE could for example indicate the time during which the UE commits to keep the applicability of the functionality. Such information would allow the network to achieve a better tradeoff between signaling/resource overhead and system performance guarantee and would be helpful for performing appropriate functionality management. For example, if the applicable status can be maintained for a relatively long time, less resources for functionality monitoring are needed. If the applicable status cannot be maintained for a relative long time or cannot be determined reliably, the NW may decide not to activate the functionality, even though it is applicable for the time being, or the NW may still choose to activate the functionality, but configure more resources for performance monitoring.

Observation 4: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 7: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated.

## Performance monitoring

For the UE-side AI/ML model for BM-Case1 and BM-Case2, RAN1 had the following progress for performance monitoring.

RAN1 117 meeting progress Agreement For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report - FFS on other contents - The report is at least configured/triggered by NW - Note: this may or may not have additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring RAN1 118 meeting progress Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resources for monitoring - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resources for monitoring - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability information of the predicted beam(s) to be the Top 1 or Top K beam - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

RAN1 is still discussing the requirements and solutions for monitoring. In option 1, RAN1 has agreed measurement results from resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report, which can be in the L1 signaling. For Option 2, the UE-assisted performance metric and how to report are still FFSs. In addition, the requirements for data collection for monitoring are still under RAN1 discussions, and if it is time critical, L1 signalling is suitable for such reporting and configuration parts can be further discussed by RAN2.

In summary, we think that RAN2 can wait for RAN1 for more progress, and if RAN1 would like RAN2 to check some parts of the solution, RAN2 can have further discussions in future.

Observation 5: RAN1 is discussing the solutions for monitoring, and agreed that the L1-RSRP and/or RS index can be reported to the NW in option 1. It can be left to RAN1 to decide whether L1 signaing can be reused.

Observation 6: In option 2, the UE-assisted performance metric and how to report are FFSs, RAN 2 can wait for RAN1 to decide the content to report.

Proposal 8: RAN2 waits for RAN1 progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.

# Conclusion

In this paper, our observations and proposals are summarized as follows:

Applicable functionality determination

Proposal 1: For applicable functionalities determination, both option 1 (joint decision by UE and NW) and option 2 (UE decision) should be supported in the following way:

- In case the network does not provide the UE with associated IDs, the UE performs applicable functionality filtering without considering NW-side additional conditions and reports the (UE-considered) applicable functionalities together with their associated IDs, if available. (Option 1)

- In case the network provides the UE with associated IDs, the UE performs applicable functionality filtering considering them and only reports the functionalities meeting the NW-side additional condition. (Option 2)

Applicable functionality reporting procedure

Observation 1: UAI can be used to support both proactive and reactive applicable functionality reporting.

Observation 2: Usage of RRCReconfigurationComplete message for applicable functionality reporting does not improve the continuity of AIML feature.

Proposal 2: As the associated ID is not globally unique, it is proposed RAN2 to discuss how to guarantee the continuity of the AI/ML operation during mobility between the cells.

Proposal 3: Applicable functionality reporting via RRCReconfigurationComplete is not supported.

Proposal 4: For handover case, as a baseline, applicable functionality reporting can be sent to the target cell via UAI right after the handover is completed based on UAI configuration provided in the HO command by the target cell.

Proposal 5: If the service continuity of the AI/ML should be supported, RAN2 should focus on solutions allowing for proper AIML configuration to be provided already in HO command, e.g. the source node can help collect the applicable functionality information for the target node.

Applicability status changes

Observation 3: Changes among different functionality statuses have an impact on functionality management. Frequent changes may result in frequent activation/deactivation or may require more resources for performance monitoring, which will introduce signaling/resource overhead.

Observation 4: Frequent functionality applicability changes/updates may have a detrimental impact on the signaling overhead, system performance and complicate AIML functionality management by the network.

Proposal 6a: When the UE initiates applicable functionality reporting for the first time in a cell, the UE only needs to report the applicable functionalities.

Proposal 6b: When the status of the functionality which has been previously requested by the network changes from applicable to non-applicable or vice-versa, such information needs to be delivered to the network.

Proposal 7: RAN2 should investigate the means to avoid the need of frequent applicability status updates and ensure the stability of inference operation once it is activated.

Performance monitoring

Observation 5: RAN1 is discussing the solutions for monitoring, and agreed that the L1-RSRP and/or RS index can be reported to the NW in option 1. It can be left to RAN1 to decide whether L1 signaing can be reused.

Observation 6: In option 2, the UE-assisted performance metric and how to report are FFSs, RAN 2 can wait for RAN1 to decide the content to report.

Proposal 8: RAN2 waits for RAN1 progress related to performance monitoring for the UE-side AI/ML model for BM-Case1 and BM-Case2.

# References

[1] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)

[2] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model