3GPP TSG-RAN WG2 Meeting #127bis					           R2-2408737

Hefei, China, October 14th – 18th, 2024

Agenda Item:		8.3.3

Source:			Huawei, HiSilicon

Title:			Discussion on simulations for measurement event prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#125bis meeting [1], the agreements on measurement event prediction were reached as follows:

Agreements: 1. At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed. 2. Clarifications on what is being as input should be provided with results 3. Start with A3 as a baseline. 4. Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

And in RAN2#127 meeting [2], further agreements on RLF prediction were reached as follows:

Agreements - Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations Agreements on KPI For indirect - SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction. Direct - missed RLF detection, false RLF detection, F1 score. - Whether the RLF will happen based on the following methodology. Above a probability threshold we assume that RLF will happen. This is then compared with true RLF. - For the time being we don’t need HO procedure simulation in RLF simulation - FFS full buffer and assumption that all the cells are fully loaded. We will not simulate traffic. - Simulation results are not expected before February

In this contribution, we will further discuss the simulations for measurement event prediction.

# 2   Discussion

## 2.1 Model input and output

It had been agreed that indirect measurement event prediction (i.e., based on RRM measurement prediction) will be studied, and direct measurement event prediction is also allowed.

Whether the measurement event conditions are met is judged by the UE based on RRM measurements. For indirect prediction, the input should be RRM measurement and the output should be predicted RRM measurement. Then the time instance that the measurement event occurs can be determined based on the predicted RRM measurement without further AI/ML model involvement. Since we used RSRP as the measurement quantity for RRM measurements prediction use case, we can also reuse the RSRP for measurement event prediction.

Proposal 1: For indirect measurement event prediction, the input and output are measured RSRP and predicted RSRP respectively, based on which the time instance when the measurement event occurs can be determined without further AI/ML model involvement.

Regarding direct measurement event prediction, similar to direct RLF prediction, the output should be the probability of measurement event within a time window. While for the input, it may be RRM measurement and/or historical measurement event, it can be up to each company to provide what inputs they are using for direct measurement event prediction.

Proposal 2: For direct measurement event prediction, the output is the probability of measurement event to occur within a time window and companies should indicate what inputs they are using for the simulation (e.g. RSRP or historical measurement event occurrences).

## 2.2 Simulation scenarios and assumptions

In RAN2#126 meeting [3], the priority of each use cases of RRM measurement prediction were reached as follows:

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

Regarding the purpose of AI aided mobility, the first goal is measurement overhead reduction, and the second goal is HO performance improvement. For measurement event prediction, it had been agreed that A3 will be used as the baseline (see TR 38.744 v0.0.3). The accuracy of RRM measurement prediction depends on several factors such as, e.g., UE speed, observation window length, prediction window length, or measurement reduction rate. In order to minimize the extra workload needed for measurement event prediction simulations, we think we should select only a part of use cases that we simulated for RRM measurement prediction. As use case#2 is evaluated for 1st study goal using intra-frequency intra-cell temporal domain case B prediction by predicting a sub set of measurement instances in temporal domain of the same cell and use case#4 is evaluated for the 2nd study goal using intra-frequency intra-cell temporal domain case A prediction by predicting measurement result(s) in prediction window based on actual measurement result(s) in observation window of the same cell without reducing any measurement in observation window, use case#2 and use case#4 can be the representative case for study goal 1 and study goal 2. Furthermore, measurement event prediction use case has many similarities to system level simulations of RRM measurement prediction as it requires an alignment on the same aspects. The main difference will be in the KPIs that are evaluated for each task, e.g. HOF rate for SLS and measurement event prediction accuracy for the other task. Based on this we propose the following:

Proposal 3: To limit the extra simulation workload, for measurement event prediction use case, RAN2 should reuse the same scenarios/settings as for system level simulations for RRM measurement prediction use case.

The details depend on RAN2 agreements on SLS, but we copy here our views, as presented in more detail in our companion Tdoc in [4]:

|  | Case 2: FR1 to FR1 intra-frequency temporal domain case B | Case 4: FR2 to FR2 intra-frequency temporal domain case A |
| --- | --- | --- |
| Goal | Measurement overhead reduction | HO performance improvement |
| UE speeds | 30, 60, 90 km/h | 60, 90, 120 km/h |
| OW/PW | 1000/200 ms 1000/600 ms | 400/400 ms (RRM sub use case 2) 800/400 ms (RRM sub use case 2) 160/160 ms (RRM sub use case 1) 320/160 ms (RRM sub use case 1) |
| MRRT | 50% for target frequency, no reduction on serving frequency | N/A |
| Frequency layers | Inter-frequency HO: - Serving frequency: 2 GHz - Target Frequency: 4 GHz | Intra-frequency HO: Freq=30 GHz |
| A3 event settings | - TTT: 40, 80, 160, 480 ms - a3-offset: -1, 0, 1, 2, 3 dB |  |

Proposal 4: RAN2 to consider two cases for measurement event prediction, i.e., case#2 (FR1 to FR1 intra-frequency temporal domain case B) and case#4 (FR2 to FR2 intra-frequency temporal domain case A) from Table 5.2.1-1 in the TR.

Proposal 5: The following assumptions and settings can be used for measurement event prediction simulation.

|  | Case 2: FR1 to FR1 intra-frequency temporal domain case B | Case 4: FR2 to FR2 intra-frequency temporal domain case A |
| --- | --- | --- |
| Goal | Measurement overhead reduction | HO performance improvement |
| UE speeds | 30, 60, 90 km/h | 60, 90, 120 km/h |
| OW/PW | 1000/200 ms 1000/600 ms | 400/400 ms (RRM sub use case 2) 800/400 ms (RRM sub use case 2) 160/160 ms (RRM sub use case 1) 320/160 ms (RRM sub use case 1) |
| MRRT | 50% for target frequency, no reduction on serving frequency | N/A |
| Frequency layers | Inter-frequency HO: - Serving frequency: 2 GHz - Target Frequency: 4 GHz | Intra-frequency HO: Freq=30 GHz |
| A3 event settings | - TTT: 40, 80, 160, 480 ms - a3-offset: -1, 0, 1, 2, 3 dB |  |

## 2.3 KPIs

Similar to RLF prediction, the KPIs for measurement event prediction should be evaluated separately for indirect method and direct method. Since both RLF and measurement event prediction are about predicting a certain event to happen, the KPIs agreed for RLF can be reused for measurement event prediction as well. For indirect measurement prediction, the KPIs can be RSRP measurement difference, missed measurement event prediction, false measurement event prediction, F1 score, time difference of true time measurement event and predicted measurement event, true measurement event prediction.

Proposal 6: For indirect measurement event prediction, the KPIs are RSRP measurement difference, missed measurement event prediction, false measurement event prediction, F1 score, time difference of true time measurement event and predicted measurement event, true measurement event prediction.

Regarding direct measurement event prediction, the KPIs can be missed measurement event prediction, false measurement event detection, F1 score, whether the measurement event will happen based on the corresponding methodology (i.e., above a probability threshold we assume that measurement event will happen, this is then compared with true measurement event).

Proposal 7: For direct measurement event prediction, the KPIs are as follows:

- missed measurement event prediction, false measurement event detection, F1 score.

- whether the measurement event will happen based on the corresponding methodology: above a probability threshold we assume that measurement event will happen, this is then compared with true measurement event

It should be noted that the details of these KPIs for RLF are still discussed as part of “[POST127][029][AI mobility] RLF” e-mail discussion. Once it is concluded, RAN2 can verify whether/how any updates in the KPIs definitions should be applied to measurement event use case.

Proposal 8: After concluding on further details of RLF prediction KPIs discussed as part of “[POST127][029][AI mobility] RLF”, RAN2 should verify whether/how any updates in the KPIs definitions should be applied to measurement event use case.

# 3   Conclusions

In this contribution, we discussed the simulations for measurement event prediction, and provide following observation and proposals:

Scenarios and simulation assumptions

Proposal 1: For indirect measurement event prediction, the input and output are measured RSRP and predicted RSRP respectively, based on which the time instance when the measurement event occurs can be determined without further AI/ML model involvement.

Proposal 2: For direct measurement event prediction, the output is the probability of measurement event to occur within a time window and companies should indicate what inputs they are using for the simulation (e.g. RSRP or historical measurement event occurrences).

Proposal 3: To limit the extra simulation workload, for measurement event prediction use case, RAN2 should reuse the same scenarios/settings as for system level simulations for RRM measurement prediction use case.

Proposal 4: RAN2 to consider two cases for measurement event prediction, i.e., case#2 (FR1 to FR1 intra-frequency temporal domain case B) and case#4 (FR2 to FR2 intra-frequency temporal domain case A) from Table 5.2.1-1 in the TR.

Proposal 5: The following assumptions and settings can be used for measurement event prediction simulation.

|  | Case 2: FR1 to FR1 intra-frequency temporal domain case B | Case 4: FR2 to FR2 intra-frequency temporal domain case A |
| --- | --- | --- |
| Goal | Measurement overhead reduction | HO performance improvement |
| UE speeds | 30, 60, 90 km/h | 60, 90, 120 km/h |
| OW/PW | 1000/200 ms 1000/600 ms | 400/400 ms (RRM sub use case 2) 800/400 ms (RRM sub use case 2) 160/160 ms (RRM sub use case 1) 320/160 ms (RRM sub use case 1) |
| MRRT | 50% for target frequency, no reduction on serving frequency | N/A |
| Frequency layers | Inter-frequency HO: - Serving frequency: 2 GHz - Target Frequency: 4 GHz | Intra-frequency HO: Freq=30 GHz |
| A3 event settings | - TTT: 40, 80, 160, 480 ms - a3-offset: -1, 0, 1, 2, 3 dB |  |

KPIs

Proposal 6: For indirect measurement event prediction, the KPIs are RSRP measurement difference, missed measurement event prediction, false measurement event prediction, F1 score, time difference of true time measurement event and predicted measurement event, true measurement event prediction.

Proposal 7: For direct measurement event prediction, the KPIs are as follows:

- missed measurement event prediction, false measurement event detection, F1 score.

- whether the measurement event will happen based on the corresponding methodology: above a probability threshold we assume that measurement event will happen, this is then compared with true measurement event

Proposal 8: After concluding on further details of RLF prediction KPIs discussed as part of “[POST127][029][AI mobility] RLF”, RAN2 should verify whether/how any updates in the KPIs definitions should be applied to measurement event use case.

# References

- R2-2404102	Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China, 15 - 19 April, 2024

- Draft Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

- Draft Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 20 - 24 May, 2024

- R2-2408736	Simulation results and discussion on RRM measurement prediction	Huawei, HiSilicon