3GPP TSG-RAN WG2 Meeting #127b		R2-2408740

Hefei, China, October 14th – 18th, 2024

Agenda Item	: 8.3.2

Source	: Fraunhofer HHI, Fraunhofer IIS

Title	: AI/ML RRM measurement prediction

Document for	: Discussion & Decision

# Introduction

A new study item on AI/ML for mobility in NR, approved at RAN#102, aims to achieve the following objectives for RRM measurement prediction [1]:

... - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] ...

In the RAN2#125-bis discussion, the following agreements were made for RRM measurement prediction sub-agenda item:

- For cell level measurement prediction model, at least consider the following cases: - Case 1: To predict beam level results, then generate cell level results based on the predicted beam results. - Case 2: To directly predict cell level results based on cell level results. - Case 3: To directly predict cell level results based on beam level results. - Consider intra-frequency intra- and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. - The baseline study will focus on pure temporal prediction. - The following items can be considered as a baseline for the prediction accuracy of the cell-level measurement prediction: - Spatial-domain prediction: RSRP difference to the actual measurement. - Temporal prediction: RSRP difference to the actual measurement and measurement reduction rate. - As a first step we will focus on measurement prediction accuracy. - FFS whether and what system level performance evaluation is needed.

In this contribution, we provide discussion regarding on beam level RRM measurement prediction for AI/ML for mobility and we conclude with several proposals.

# Discussion

## Prediction Metrics

In general, the decision for handover based on L3 is determined by the cell-level measurement report obtained from beam-level measurements in L1. At the L3, filtering is applied to RSRP measurements to reduce noise fluctuations and remove outliers. However, when AI/ML models or functionalities are utilized, the predicted L1-RSRP values of neighboring cells may already be smoothed, meaning they do not contain unnecessary noise fluctuations. Consequently, AI/ML predictions of L1-RSRP measurements can be effectively employed in the handover process. Furthermore, leveraging AI/ML predicted L1-RSRP measurements can lead to reduced processing delays compared to traditional L3 filtering.

Proposal 1: Consider L1 beam level results alongside L3 level results as AI/ML model output.

## Measurement and Prediction Configuration

As outlined in reference [1], the study on AI/ML for mobility in NR aims to enhance mobility in RRC_CONNECTED mode by following the existing mobility framework, i.e., the handover decision is always made on the network side. Given that the handover decision is determined by the network, the UE has to be configured to report measurement and prediction results to the gNB. The beam management use case in the Rel-18 AI/ML for NR Air Interface SI [2] supports both temporal and spatial beam prediction. Two sets of beams, designated as Set A and Set B, are defined for the beam management sub-use cases. Set A comprises predicted beams derived from Set B, which contains measured beams. Similar to the AI/ML for NR Air Interface SI, the UE should be configured with two distinct beam sets, referred to as Set A and Set B, for the purpose of RRM measurement prediction.

Proposal 2: Configure UEs with beam sets A (predicted beams) and B (measured beams) for RRM measurement predictions.

As outlined in the agreement from RAN2#125-bis, it has been determined that at least three alternatives are under consideration for the AI/ML model aimed at predicting cell-level measurements. Case 1 focuses on predicting cell-level measurements based on beam-level measurement predictions. In this scenario, the input consists of beam-level measurements, with the AI/ML model providing predicted beam measurements. The cell-level measurements are then derived from these predicted beam measurements.

There are three sub-cases—Case 1, Case 2, and Case 3—which can vary in terms of model input and output for cell-level measurement prediction. Regardless of the model's input and/or output, the UE will be configured with one or more reference signals to perform AI/ML models.

Proposal 3: Configure UEs with at least two reference signals.

For measurement and/or prediction reports, the UE is configured by the gNB to measure reference signals. For instance, the measurement of the Synchronization Signal Block (SSB) reference signal serves as a basis for channel estimation and is utilized for cell selection and handover decisions. Conversely, beam level measurement prediction is conducted based on the Channel State Information Reference Signal (CSI-RS). Therefore, to enhance measurements for cell-level prediction, we propose the use of both SSB and CSI-RS as reference signals.

Proposal 4: Utilize both SSB and CSI-RS as reference signals for cell-level measurement prediction.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: Consider L1 beam level results alongside L3 level results as AI/ML model output.

Proposal 2: Configure UEs with beam sets A (predicted beams) and B (measured beams) for RRM measurement predictions.

Proposal 3: Configure UEs with at least two reference signals.

Proposal 4: Utilize both SSB and CSI-RS as reference signals for cell-level measurement prediction.

# References

- 3GPP RP-234055, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”, 3GPP TSG-RAN #102, Edinburg, Scotland, Dec 11th – Dec 15th, 2023.

- 3GPP TR38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, version 18.0.0.