3GPP TSG-RAN WG2 #127bis								         R2-2408746
Hefei, China, October 14-18, 2024

Agenda Item:	7.14.2 Corrections

Source:	Ericsson, Nokia, Nokia Shanghai Bell

Title:	Discussion on IRAT handover from LTE

Document for:	Discussion

# 1	Introduction

The release 18 work item for QoE entitled “Enhancement on NR QoE management and optimizations for diverse services”, which aims at introducing enhanced QoE support for diverse services[1], is completed. However, there is still one Editor’s Note in TS 38.331 which hasn’t been resolved:

Editor's Note: FFS whether to capture the agreement that all LTE QoE configurations are released at handover to NR in LTE specification or NR specification.

# 2	Discussion

In RAN2#124, the following agreements were made:

For HO from LTE/5GC to NR, UE should release all LTE QoE configurations and apply NR QoE configuration if received. How UE releases old QoE configurations can be handled in CR phase.

For HO from NR to LTE/5GC, UE should release all NR QoE configurations and apply LTE QoE configuration if received. How UE releases old QoE configurations can be handled in CR phase.

The agreement for HO from NR to LTE/5GC was captured in TS 38.331 at successful completion of the IRAT handover in chapter 5.4.3.4:

#### 5.4.3.4	Successful completion of the mobility from NR

Upon successfully completing the handover, at the source side the UE shall:

1>	if T316 was not running at the time of receiving MobilityFromNRCommand and if the UE was configured with successHO-Config when connected to the source PCell and the targetRAT-Type is set to eutra:

2>	perform the actions for the successful handover report determination for inter-RAT handover as specified in clause 5.7.10.6.

1>	reset MAC;

1>	stop all timers that are running except T325, T330 and T400;

1>	release ran-NotificationAreaInfo, if stored;

1>	release the AS security context including the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key, if stored;

1>	release all radio resources, including release of the RLC entity and the MAC configuration;

1>	release the associated PDCP entity and SDAP entity for all established RBs;

NOTE :	PDCP and SDAP configured by the source RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure.

1>	if the UE is configured with any application layer measurement configuration:

2>	inform upper layers about the release of all application layer measurement configurations;

2>	release all application layer measurement configurations including their fields in UE variables VarAppLayerIdleConfig and VarAppLayerPLMN-ListConfig;

2>	discard any application layer measurement reports which were not yet fully submitted to lower layers for transmission;

2>	consider itself not to be configured to send application layer measurement reports;

1>	if the targetRAT-Type is set to eutra and the nas-SecurityParamFromNR is included: or

1>	if the targetRAT-Type is set to utra-fdd:

2>	indicate the release of the RRC connection to upper layers together with the release cause 'other'.

------------------------------------------------------------------------------------------------------------------------------------------------

The most straight forward and consistent solution would be to capture the agreement for HO from LTE/5GC to NR correspondingly in the LTE specification, as e.g. proposed in CR R2-2407090:

#### 5.4.3.4	Successful completion of the mobility from E-UTRA

Upon successfully completing the handover, the cell change order or enhanced 1xRTT CS fallback, the UE shall:

1>	if the targetRAT-Type in the received MobilityFromEUTRACommand is set to eutra (intra-E-UTRA inter-system HO):

2>	indicate to the upper layers associated to the source system the release of the RRC connection together with the release cause 'other';

2>	the procedure ends;

1>	else if the UE was connected to 5GC prior to the reception of the MobilityFromEUTRACommand and the targetRAT-Type in the received MobilityFromEUTRACommand is set to nr:

2>	reset MAC;

2>	stop all timers that are running except T325, T330;

2>	release ran-NotificationAreaInfo, if stored;

2>	release the AS security context including the KRRCenc key, the KRRCint, the KUPint key and the KUPenc key, if stored;

2>	release all radio resources, including release of the RLC entity, the MAC configuration and the associated PDCP entity and SDAP entity for all established RBs;

NOTE 1:	PDCP and SDAP configured by the source configurations RAT prior to the handover that are reconfigured and re-used by target RAT when delta signalling (i.e., during inter-RAT intra-system handover when fullConfig is not present) is used, are not released as part of this procedure.

2> if a serviceType is stored in the current UE configuration:

3>	release the stored serviceType:

3>	inform upper layers to clear the stored application layer measurement configuration;

3>	discard received application layer measurement report information from upper layers;

3>	consider itself not to be configured to send application layer measurement report;

1>	else:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

NOTE 2:	If the UE performs enhanced 1xRTT CS fallback along with concurrent mobility to CDMA2000 HRPD and the connection to either CDMA2000 1xRTT or CDMA2000 HRPD succeeds, then the mobility from E-UTRA is considered successful.

------------------------------------------------------------------------------------------------------------------------------------------------

However, there have been concerns about such a solution. One concern is that it may impact legacy behaviour. The behaviour would be added for rel-18 and the release of QoE configurations does not have any impact on other functionality, so there is no legacy impact. For other features, a target NR node can trigger fullConfig if there is some functionality from the source node which the target doesn’t support and the UE and the network will then clear the configurations respectively. In NR for QoE, only the RRC part is cleared at fullConfig and the configuration in the application layer is kept unless the target node releases them or does not reconfigure the QoE RRC part in the handover command (in particular for the case that the target node doesn’t support QoE). However, the release in the NR specification relates to NR AT-command and is not compatible with LTE QoE configurations.

Therefore, fullConfig cannot be used to release QoE configurations. A target NR node cannot release an LTE QoE configuration and it is also not possible for the source LTE node to release the QoE configuration in MobilityFromEUTRACommand. Moreover, for inter-RAT handover from LTE/5GC to NR, delta configuration is also allowed where PDCP and SDAP configurations can be reused after inter-RAT handover (as described in Note1 of Clause 5.4.3.4). It needs to be stated clearly what should be the QoE configuration handling for delta configuration during inter-RAT mobility. Hence for both of fullConfig and delta configuration, the conclusion is that it is necessary to capture in the specification that the UE shall release a possible QoE configuration at IRAT handover from LTE/5GC to NR, otherwise the configurations may remain hanging in the application layer.

One proposal has been to capture the release of QoE configurations in a Note instead of in the procedure text or in Stage 2. However, that would be very inconsistent as the release of QoE configurations is captured in the procedure text in all other places in the specification. Notes are only informative and are only supposed to explain the normative part of the specification. Since there is no normative wording mandating that the UE AS shall indicate release of QoE configuration to upper layers, it is not suitable to rely on a Note.

Adding the release of QoE configurations at IRAT handover from LTE/5GC to NR in rel-18 does not have any legacy impact.

Adding the release of QoE configurations at IRAT handover from LTE/5GC to NR in a Note is inconsistent with existing specifications.

Therefore, the following is proposed:

Capture the release of LTE NR QoE configurations at IRAT handover from LTE/5GC in normative text in “5.4.3.4 Successful completion of the mobility from E-UTRA” in TS 36.331 according to CR R2-2408833 [2].

# Conclusion

In the previous sections we made the following observations:

Observation 1	Adding the release of QoE configurations at IRAT handover from LTE/5GC to NR in rel-18 does not have any legacy impact.

Observation 2	Adding the release of QoE configurations at IRAT handover from LTE/5GC to NR in a Note is inconsistent with existing specifications.

Based on the discussion in the previous sections the following is proposed:

Proposal 1	Capture the release of LTE NR QoE configurations at IRAT handover from LTE/5GC in normative text in “5.4.3.4 Successful completion of the mobility from E-UTRA” in TS 36.331 according to CR R2-2408833 [2].

# References

RP-221803, Enhancement on NR QoE management and optimizations for diverse services, China Unicom

R2-2408833, Correction on QoE measurements release at successful handover from LTE to NR,  Nokia, Nokia Shanghai Bell, Ericsson