3GPP TSG-RAN WG2 #127bis	R2-2408747

Hefei, China, 14th – 18th October 2024

Agenda Item:	8.3.2.1

Source:	Ericsson

Title:	Simulation results for temporal and inter-frequency RRM measurement predictions

Document for:	Discussion

# 1	Introduction

A new study item on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR was approved in RAN plenary #102 with the following objectives [1].

In RAN2#126 meeting, priorities for RRM measurement prediction were agreed to well-proceed the simulation in the next meetings [3].

Case A is an urban micro-scenario, and Case B is an urban macro-scenario, more details in the annex.

| Case number | Prioritization | Evaluation scenario combination | Target study goal |
| --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 2nd goal |

In this contribution, the simulation results for those cases highlighted as high priority are discussed.

# 2	Discussion

## 2.1	Simulation scenarios

In RAN2#125bis meeting, it was agreed that following cases are considered for cell-level measurement prediction model,

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results.

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results.

Considering the importance of the cell level measurement predictions for L3 mobility (which is the main focus of the study item) in this contribution we focus on Case 2 and Case 3, wherein the L3 filtered cell-level measurements are directly predicted by AI/ML model. For AI/ML model, we use cell-level measurement (Point C in Figure 1) as both output and input of model in Case 2 (for different cells).

Besides, how to perform layer1 filtering has impact on the prediction although it’s not standardized (currently, it is a UE implementation). In this contribution, we use the already existing L3 filtering formula in TS 38.331 section 5.5.3.2 with specific alpha 0.2, i.e., Fn = (1 – a)*Fn-1 + a*Mn, where the parameters are defined in the mentioned TS 38.331. For layer1 filtering, we used the existing L3 filtering mentioned in TS 38.331 section 5.5.3.2 with specific alpha 20%.

Figure 1. Measurement model illustrated in TS 38.300

In this contribution, Case 2 and Case 3 are studied for evaluation of cell level measurement predictions.

For Case 2, L3 filtered cell-level measurements is used as model input to predict the L3 filtered cell level measurements.

For Case 3, L1 filtered beam-level results is used as model input to predict the L3 cell-level measurements.

For layer1 filtering, the existing L3 filtering mentioned in 38.331 section 5.5.3.2 with specific alpha 20% is reused.

For both Case 2 and Case 3, the single-cell prediction approach is used i.e., the number of cells for which their measurements are used as input and the number of cells for which the time domain/frequency predictions are inferred as output are set to 1 during the training and inference.

For evaluation of Case 2 and Case 3, the single-cell prediction approach is used.

## 2.2	Temporal RRM measurement predictions

In post email discussion phase 1 of RAN2#127 meeting, two L1/L3 filtering options were agreed [4]. Option 1 shown in Figure 2 is considered in this contribution which is the legacy approach specified in 5.5.3.2 in TS 38.331. I.e., a new L3 filtered value would be calculated every time receiving measurement results (L1 filtered value).

Figure 2 Sliding L1/L3 filtering

Option 1, i.e., sliding L1/L3 filtering is considered.

### 2.2.1	Evaluation methodology

Given that the first goal of the study is to study the RRM measurements for the sake of reducing the measurement overhead, in the following we first present the evaluation of the AI/ML performance targeting to reduce the measurement overhead. Later we present the evaluation of the AI/ML performance targeting to enhance the handover performance.

In the following, we refer to Case B from RAN1 AI/ML PHY TR 38.843, as the measurement to prediction pattern used to reduce the measurement overhead. In addition, we use Case A from RAN1 AI/ML PHY TR 38.843, as the measurement to prediction pattern used to enhance the mobility performance.

In general, we evaluate the following cases:

- Case 2 – Case B: prediction cell level measurements from cell level measurements targeting to reduce measurement overhead

- Case 3- Case B: prediction cell level measurements from beam level measurements targeting to reduce measurement overhead

- Case 2- Case A: prediction cell level measurements from cell level measurements targeting to mobility performance improvement

- Case 3- Case A: prediction cell level measurements from beam level measurements targeting to mobility performance improvement

### 2.2.2	First goal: Prediction to reduce measurement overhead (Case B)

Two prediction patterns were discussed [5]. Prediction pattern example 2 shown in the figure below is used in this contribution.

Figure 3. Temporal domain case example 2

Measurement overhead reduction rate calculation is related to the number of samples, not the length of observation window and prediction window. Thus, the reduced measurement overhead using time domain prediction is calculated by:

MRRT= num of skipped measurementsnum of all measurements

“Skipped measurements” include measurements skipped in observation window (Gray squares in the figure above) and measurement predicted in the prediction window (green squares). “All measurements” include skipped measurements and sampled measurements (yellow squares).

To maintain a certain measurement overhead reduction rate for a prediction pattern, OW/PW window should slide with a proper offset to ensure the predicted samples will not be measured during future inference. For example, in Figure 3, 50% measurement overhead reduction rate is achieved in first inference (first row) because there are 3 sampled measurements and 3 skipped measurements. To maintain 50% overhead reduction rate for the following inference, OW/PW window should slide with offset “2” just to make the skipped measurement in previous inference (first row) will not be measured in future inferences (second row and so on). That is, gray squares should never overlap with yellow squares.

Cell level measurement prediction based on cell level measurements (Case 2)

As mentioned above in case 2, we use L3 filtered cell level measurements for both model input and the label/output. For example, in the 50% measurement reduction rate case illustrated in Figure 4, we use 2 cell-level RSRP measurement sampled at time instance T0 (current time) and T0-2 as model input to predict cell-level results at time instance T0+1 which is 40ms later from time instance T0. As the 66.67% measurement reduction rate case illustrated in Figure 4, model input is 2 cell-level RSRP measurements sampled at time instance T0 and T0-3 respectively, and model output is cell-level results at time instance T0+1 and T0+2 which are 40ms and 80ms later from time instance T0 respectively.

Figure 4.  L3 filtered cell level measurement predictions from L3 filtered cell level measurement results; Case 2 and Case B. Cell level measurement sampling rate is 40ms. Prediction window is varying from 40ms to 160ms. Observation window is varying from 120ms to 240ms.

By increasing the observation window length to skip more cell-level results and increasing the prediction window length to predict more cell-level results, we evaluate the prediction accuracy with different measurement overhead reduction rates. We evaluated the RSRP difference (prediction error compared to the ground through information) for various scenarios targeting to reduce the measurement overhead in the range of 50%~80%. The evaluation results are summarized in Table 1.

Table 1. Case 2 – Case B: RSRP difference (prediction error) and measurement overhead reduction rate for Cell-level measurement prediction from cell level measurement results. RSRP difference values are in dB.

| Measurement overhead reduction rate |  | 50% | 66.67% | 75% | 80% |
| --- | --- | --- | --- | --- | --- |
| UE moving speed | 30km/h | 0.117 | 0.145 | 0.17 | 0.236 |
|  | 120km/h | 0.135 | 0.193 | 0.285 | 0.401 |

Table 1 summaries the AIML evaluations for the scenario Case 2 – Case B. As shown, both in low mobility scenarios (e.g., 30km/h speed) and high mobility scenarios (120km/h), the AIML model can predict the RSRP with RSRP difference (so-called prediction error) of less than 0.5 dB while reducing the measurement overheads up to 80%.

For Case 2-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 0.5 dB while reducing the measurement overheads up to 80%.

For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 0.5 dB while reducing the measurement overheads up to 80%.

Cell level measurement prediction based on beam level measurements – (Case 3)

As mentioned above in Case 3, we use beam-level measurements for model input and cell-level results for the label/output. Similar prediction pattern as used in Case 2 is considered here, but the difference from case 2 is, we considered larger observation window length to sample more beam-level measurements (e.g., beam-level results of 8 time instances, e.g., T0,T2, …,T0-14 are sampled according to Figure 5, MRRT 50% case.) to make it have sufficient time domain information in each data sample for training and inference.

For example, as the 50% measurement reduction rate case illustrated in Figure 5, we use 2 beam-level measurements as model input within the observation window. Same as case 2, we predict cell-level results at time instance T0+1 which is 40ms later from time instance T0.

By increasing the observation window length to skip more beam-level results and increasing the prediction window length to predict more cell-level results, we simulate the prediction accuracy with different measurement overhead reduction rate. The simulation results are provided in the following.

Figure 5.  L3 filtered cell level measurement predictions from L1 filtered beam level measurement results; Case 3 and Case B. Cell level measurement sampling rate is 40ms and beam level measurement sampling rate is varying from 80ms to 240ms. Prediction window is varying from 40ms to 160ms. Observation window is varying from 120ms to 840ms

We evaluated the RSRP difference (so called prediction error) with 50 %~80 % measurement overhead reduction rate. Simulation results is summarized in Table 2.

Table 2. Case 3 – Case B: RSRP difference (prediction error) and measurement overhead reduction rate for Cell-level measurement prediction from cell level measurement results.

| Measurement overhead reduction rate |  | 50% | 66.7% | 75% | 80% |
| --- | --- | --- | --- | --- | --- |
| UE moving speed | 60km/h | 0.651 | 0.515 | 0.556 | 0.607 |
|  | 120km/h | 0.761 | 0.568 | 0.62 | 0.785 |

In general, we observe that similar to Case 2, in Case 3 and both in low mobility scenarios (e.g., 60 km/h speed) and high mobility scenarios (e.g., 120 km/h) the AIML model can predict the RSRP with RSRP difference (so-called prediction error) of less than 1 dB when the reduction rate is up to 80 %. Similarly, the RSRP difference is slightly higher in high mobility scenarios (120 km/h).

For Case 3-Case B: in low mobility scenarios (e.g., 60km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 80%.

For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with a good accuracy, i.e., RSRP difference is less than 1 dB while reducing the measurement overheads up to 80%.

### 2.2.3	Second goal: Prediction to improve handover performance (Case A)

Temporal prediction with measurement to prediction pattern denoted as Case A aims to improve handover performance. In post email discussion phase1 in RAN2#127 meeting, some proposals were made to proceed the simulation of case A:

Proposal 3: The FR2 sampling period is 80ms. Proposal 4: For temporal domain case A, Company can report the observation window and prediction window with following limitation: 1, The ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5} 2, The window length should be multiple times of sampling period or measurement period of corresponding frequency range.

Cell level measurement prediction based on cell level measurements (Case 2)

As mentioned above in Case 2, we use cell-level measurement results for both model input and the output/label. Specifically, we use cell-level results sampled every 80ms at each time instance depicted with {T0-4, …, T0-1, T0} in Figure 6. As shown the observation window fixed to 5x80ms to predict the cell-level results which is (N x 80 ms) after the end of the observation window T0 wherein N  {1, 2, 3, 4, 5}. For example, as the case of prediction ratio 5, prediction step 80ms illustrated in Figure 6, we use 5 cell-level RSRP measurement sampled at time instance T0~T0-4 as model input to predict cell-level measurements at time instance T0+1 which is 80ms after time instance T0. As the case of ratio 1, prediction step 80ms illustrated in Figure 6, we predict cell-level results at time instance T0+1 ~T0+5 by using 5 cell-level RSRP measurement sampled at time instance T0~T0-4 as model input.

Figure 6.  L3 filtered cell level measurement predictions from L3 filtered cell level measurement results; Case 2 and Case A. Cell level measurement sampling rate is 80ms. For ratio 5, prediction window is 80ms and prediction step size is N*80ms wherein N  {1, 2, 3, 4, 5}. For ratio 1, prediction window is 400ms and prediction step fix to 80ms.

We evaluate the prediction accuracy (measured as RSRP difference) with different prediction window step size N leading to prediction window size between 80ms to 400s. The evaluation results are provided in the following.

Table 3. Case 2 – Case A: RSRP difference (prediction error) for Cell-level measurement prediction from cell level measurement results.

| Ratio 5 | Averaged RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Prediction step(ms) | 80 | 160 | 240 | 320 | 400 |
| UE moving speed 60km/h | 0.217 | 0.467 | 0.767 | 1.139 | 1.497 |
| UE moving speed 120km/h | 0.269 | 0.571 | 0.971 | 1.356 | 1.732 |

| Ratio 1 | Average RSRP differnece(db) | RSRP difference (dB) for each position in prediction window |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction step(ms) | 80 | 1st | 2nd | 3rd | 4th | 5th |
| UE moving speed 60km/h | 0.817 | 0.222 | 0.458 | 0.78 | 1.13 | 1.494 |
| UE moving speed 120km/h | 0.997 | 0.294 | 0.603 | 0.973 | 1.362 | 1.754 |

The evaluation shows that in a scenario with UE mobility set to 60 km/h the AIML model can predict the RSRP value with up to 1dB difference compared to the ground truth value when prediction window size is less than 320ms. In a higher mobility scenario (i.e., when UE speed is 120 km/h) the RSRP difference is slightly higher.

Comparing ratio 5 and ratio 1 case, similar prediction accuracy is obtained for a certain prediction step. However, in ratio 1 case, there are 5 different models trained for different prediction step while there are 1 model trained for predicting 5 steps. Thus, in ratio 1 case, prediction accuracy is achieved with a high cost of model size (i.e., UE need to store 5 models.) and low cost of model complexity (i.e., FLOPs). For ratio 2 case, prediction accuracy is achieved with low cost of model size (i.e., only 1 model is stored) and high model complexity in terms of FLOPs.

Therefore, we have the following observations:

One-shot training and inference (e.g., ratio 1) has similar performance as multi-model training and inference (e.g., ratio 5 with different prediction steps)

Good prediction accuracy either requires large model size or larger model complexity.

Maximum prediction step is below 320ms for UE moving speed 60km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Maximum prediction step is below 320ms for UE moving speed 120km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Prediction accuracy on temporal prediction case A - Case 2 should be further improved by e.g., complex model structure, cluster approach, assist information etc.

Cell level measurement prediction based on beam level measurements (Case 3)

As mentioned above in Case 3, we use beam-level measurements for model input and cell-level measurements is the label/output. Difference from case 2 is, we considered larger observation window length to sample more beam-level measurements (e.g., beam-level results of 20 time instances, e.g., T0-19~T0 are sampled according to Figure 7) to make it have sufficient time domain information in each sample in training and inference.

Specifically, we use beam-level results sampled every 80ms in observation window fixed to 20*80ms to predict the cell-level measurements which is N*80ms later from the end of the observation window (i.e., T0 (current time) + N*80ms). The prediction window size fix to 320ms to keep it “ratio 5” as agreed in last meeting. For example, as the case of prediction step 80ms illustrated in Figure 7, we use 20 beam-level RSRP measurements sampled in the observation window as model input to predict cell-level results at time instance T0+1~T0+4 which is 80ms~320ms later from time instance T0.

Figure 7.  L3 filtered cell level measurement predictions from L1 filtered beam level measurement results; Case 3 and Case A. beam level measurement sampling rate is 80ms. For ratio 5, prediction window is fixed to 320ms and prediction step size is N*80ms wherein N  {1, 2, 3, 4, 5}.

We simulate the prediction accuracy with different length of prediction step. The simulation results are provided in the following. For temporal prediction case A – Case 3 we evaluated the RSRP difference with prediction step 80ms~400ms. Simulation results is summarized in Table 4.

Table 4. Case 3 - Case A: L3 filtered cell level measurement predictions from L1 filtered cell level measurement results; Case 3 and Case A. beam level measurement sampling rate is 80ms. prediction window is 320ms and prediction step size is N*80ms wherein N  {1, 2, 3, 4, 5}.

| Ratio 5 |  | Averaged RSRP difference (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Prediction step(ms) |  | 80 | 160 | 240 | 320 | 400 |
| UE moving speed 60km/h | AIML, 8 beams as input | 0.939 | 1.096 | 1.227 | 1.593 | 1.831 |
| UE moving speed 120km/h | AIML, 8 beams as input | 1.166 | 1.32 | 1.668 | 1.885 | 2.182 |

| Ratio 5 | Prediction steps(ms) | Averaged RSP difference(dB) | Separate RSRP difference(db) for each position in prediction window |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1st | 2nd | 3rd | 4th |
| UE moving speed 60km/h | 80 | 0.939 | 0.82 | 0.823 | 0.952 | 1.15 |
|  | 160 | 1.096 | 0.871 | 0.982 | 1.149 | 1.382 |
|  | 240 | 1.227 | 0.97 | 1.134 | 1.364 | 1.643 |
|  | 320 | 1.593 | 1.23 | 1.46 | 1.717 | 1.965 |
|  | 400 | 1.831 | 1.427 | 1.683 | 1.958 | 2.255 |
| UE moving speed 120km/h | 80 | 1.166 | 0.972 | 1.025 | 1.194 | 1.471 |
|  | 160 | 1.32 | 1.019 | 1.163 | 1.397 | 1.700 |
|  | 240 | 1.668 | 1.199 | 1.469 | 1.809 | 2.196 |
|  | 320 | 1.885 | 1.463 | 1.734 | 2.02 | 2.322 |
|  | 400 | 2.182 | 1.728 | 2.041 | 2.333 | 2.626 |

Unlike the good prediction performance of Case 2- Case A, in scenario of Case 3 – Case A, the RSRP differences are close or above 1 dB for all prediction steps.

Some remarks:

Given that the results in case of Case 3 – Case A shows a substantial performance degradation compared to the results in Case 2- Case A, some remarks are in order.

For example, we believe RAN2 should study which set of beams should be included as input to the model for the Case 3 – Case A. In our simulation, we used the measurement results of the 8 beams of the cells as input to the model, while the actual cell level RSRPs are derived by linear combination of the 3 best beams of each cell (i.e., 5 beams are filtered as part of beam consolidation procedure). This might have been a reason to get such degradation in the model performance in that case.

In addition, beams are spatially different (transmitted to non-orthogonal directions) and such phenomenon might create uncorrelated behaviour among the input that finally affects the performance of the AIML model. Therefore, we propose the following.

RAN2 investigate the impact of the set of beams used as input in the Case 3. i.e., which set of beams to use as input to the model.

Finally, we would like to request RAN2 to consider the results in the Annex to be included in the TR of the study item.

RAN2 include the results in the Annex A of the paper in the TR of the study item.

## 2.3	Inter-frequency RRM measurement predictions

In this section, we show the prediction of 4 GHz RSRP with 2 GHz as input (hereafter referred to as low-to-high frequency prediction), and prediction of 2 GHz RSRP from 4 GHz as input (hereafter referred to as high-to-low frequency prediction).

In the below analysis, the following terms are defined as:

- ML RSRP difference (ML-Loss): is the average of the difference between measured L3 filtered Cell-RSRP (ground truth) and the predicted L3 filtered Cell-RSRP, measured in dB.

- Baseline: the output (measurement of the target cell in neighbouring frequency is calculated by averaging the input cells’ RSRPs. This is a non-AI/ML based algorithm for comparison with AIML based prediction

- Baseline RSRP difference (BL-Loss): is defined as the average of the difference between measured L3 filtered Cell-RSRP (ground truth) and the Baseline RSRP, measured in dB.

- Gain: is defined as the difference between prediction plus baseline loss divided by baseline loss, i.e., (ML-Loss - BL-Loss) / BL-Loss.

- Gain: is defined as the difference between prediction plus baseline loss divided by baseline loss, i.e., (ML-Loss - BL-Loss) / BL-Loss.

Further detailed system and AI parameters are shown in Table B-2.

It is worth mentioning the 2 GHz is an FDD band whereas the 4 GHz is a TDD band, hence we aligned our system with such setup.

Additionally, we would like to remark that in our previous contribution R2-2407092 we discussed the definition of frequency distance concept, that is a metric indicating the correlation between frequencies. We showed that the larger the distance between frequencies, the lower the correlation between frequencies. We have also concluded that the smaller the frequency distance the better ML prediction. More precisely in Table 3 of R2-2407092 we showed that, when using 2.6 GHz as input to predict 1.8 GHz (noted as 2.6-to-1.8GHz) the RSRP difference (ML to ground truth) is 0.48 dB. However, when we increase frequency distance to predict the case of 1.8-to-3.5GHz the RSRP difference is 1.77 dB. Furthermore, if we increased the frequency distance to the case of 0.8-to-3.5Ghz, ML model achieves even worse performance, i.e., 2.3 dB.

Inter-Frequency prediction highly depends on the frequency distance (inversely proportion to correlation) between input/output frequencies, i.e., the higher the frequency distance the worse the prediction performance.

### 2.3.1	Simulation Insights for Inter-frequency prediction Case-2

In this section, we focus our evaluation on only 2 GHz and 4 GHz, as shown in Table 5, in which we evaluate the model performance of L3 Cell-Level measurement as input and L3 Cell-Level measurement on another frequency as output.

Table 5 shows that 2-to-4 GHz prediction achieves about 1.4 dB RSRP difference, in row-1. This result is higher than results of 1.8-to-2.6 GHz obtained in previous contribution R2-2407092. Such degradation in performance is expected (and aligned with Observation 16) since the frequency distance of 2-to-4 GHz is much higher than that of 1.8-to-2.6 GHz. In row-2, Table 5 shows that 4-to-2 GHz prediction outperforms 2-to-4 GHz with a loss (RSRP difference) of 1.2 dB.

Moreover, we observe that the inter-frequency prediction accuracy for predicting the RSRP for a cell operating in lower frequency from higher frequency RSRP measurements slightly out-performs the prediction accuracy for predicting the RSRP for a cell operating in higher frequency from lower frequency RSRP measurements.

Table 5. Case-2 Inter-frequency prediction evaluation without any assistant Information (merely based on RSRP of serving frequency as input) for 2 and 4 GHz frequencies.

| Scenario | Model input | Model output | ML L3-RSRP difference (dB) | Baseline RSRP different (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | RSRP@2GHz | RSRP@4GHz | 1.4 | 9.7 | 85.5 |
| 2 | RSRP@4GHz | RSRP@2GHz | 1.2 | 9.5 | 87.36 |

The Inter-frequency prediction error for predictions without any assistance information is slightly above 1 dB.

Higher-to-lower frequency prediction slightly outperforms lower-to-higher frequency prediction (on average 0.2 dB less error is observed when predicting a cell in 2GHz frequency).

Given the above simulation results we kindly ask RAN2 to include the simulation results concerning inter-frequency prediction evaluation in the TR.

RAN2 includes the inter-frequency Case-2 prediction results in the TR.

Impact of assistance/additional information as input to the AIML model

In order to improve the prediction accuracy, we have also included UE-Location as assistant information to the model inputs (evaluated in Table 6). Such assistant information reduced 2-to-4 GHz prediction error from 1.4 dB to 0.89 dB (as shown in row-1 in Table 6). In addition, it reduces prediction errors for 4-to-2 GHz scenario from 1.2 dB to 0.82 dB (as show in row-2 in Table 6).

Table 6. Case-2 Inter-frequency prediction evaluation with assistant Information for 2 and 4 GHz frequences. In these set of simulations the location information of the UE, beside the RSRP measurement of the serving cell in the serving frequency is used as mode

| Scenario | Model input | Model output/label | ML L3-RSRP difference (dB) | Baseline RSRP different (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | - RSRP@2GHz - UE-Location | RSRP@4GHz | 0.89 | 9.7 | 90.8 |
| 2 | - RSRP@4GHz - UE-Location | RSRP@2GHz | 0.82 | 9.5 | 91.4 |

Based on the above discussions, we have the following observations and proposals.

Assistance information (e.g., location information) improves inter-frequency prediction performance (reducing the error below 1 dB).

In light of the above we propose the following:

RAN2 study the impact of assistance information (e.g., location information, etc.) as model input in improving the inter-frequency prediction accuracy.

### 2.3.2	Simulation Insights for Inter-frequency prediction Case-3

In this section, we focus our evaluation on Case-3, where beam-level measurements (of e.g., s serving cell) is input and cell-level measurement of a neighboring cell in a different frequency is the ,model output of the model, for only 2 GHz and 4 GHz, as shown in Table 7 and Table 8, in which we evaluate the model performance of L3 Beam-Level measurement as input and L3 Cell-Level measurement on another frequency as output.

Table 7. Case-3 Inter-frequency prediction evaluation without any assistant Information (merely based on beam level RSRP of serving frequency as input) for 2 and 4 GHz frequencies.

| Scenario | Model input | Model output | ML L3-RSRP difference (dB) | Baseline RSRP different (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | Beam-RSRP@2GHz | Cell-RSRP@4GHz | 2 | 3.7 | 45.95 |
| 2 | Beam-RSRP@4GHz | Cell-RSRP@2GHz | 1.9 | 15 | 87.33 |

Comparing Case-3 to Case-2, where beams vs cells are used as input to the model:

Table 7 illustrate evaluation of the Case-3 prediction, we observed for Case-2, lower to higher frequency prediction (2 dB) performed worse than higher to lower frequency prediction (1.9 dB), but the difference is negligible.

The main insight from Table 7 is that the performance of using beam-level as input is worse than performance of using cell-level as input (in Table 6). For low-to-high frequency the performance degrades from 1.4 dB (Case-2) to 2 dB (Case-3), whereas from high-to-low frequency the performance degrades from 1.2 dB (Case-2) to 1.9 dB (Case-3).

The performance of In Case-3 (beam-level as input and cell-level as output) prediction is slightly worse than that of Case-2 prediction.

Table 8. Case-3 Inter-frequency prediction evaluation with assistant Information for 2 and 4 GHz frequences. In these set of simulations the location information of the UE, beside the beam level RSRP measurement of the serving cell in the serving frequency is use

| Scenario | Model input | Model output | ML L3-RSRP difference (dB) | Baseline RSRP different (dB) | Gain (%) |
| --- | --- | --- | --- | --- | --- |
| 1 | - BeamRSRP@2GHz - UE-Location | Cell-RSRP@4GHz | 1.4 | 3.7 | 62.16 |
| 2 | - BeamRSRP@4GHz - UE-Location | Cell-RSRP@2GHz | 1.4 | 15 | 90.67 |

Impact of assistance/additional information as input to the AIML model

In order to improve the prediction accuracy, we have also included UE-Location as assistant information to the model inputs (as in Table 8). Such assistant information reduced 2-to-4 GHz prediction error from 2 dB to 1.4 dB (as shown in row-1 in Table 8). In addition, it reduces prediction errors for 4-to-2 GHz scenario from 1.9 dB to 1.4 dB (as shown in row-2 in Table 8)

In Case-3 (beam-level as input and cell-level as output), Assistance information (e.g., location information) still improves inter-frequency prediction performance.

### 2.3.3	Measurement reduction rate for inter-frequency predictions

The main goal of the inter-frequency predictions is to reduce the UE measurement overhead, as the UE can predict cell quality in another frequency instead of using measurement gaps to tune the RF and measure the cells in the neighbouring frequencies. In the meeting RAN2#126 measurement overhead reduction rate formulas have been defined for the temporal predictions (Case B) and for the spatial predictions, as following.

Definition of measurement reduction for intra-frequency scenario is defined as: Measurement reduction rate in temporal domain (MRRT) assuming same length of measurement time instances: MRRT= skipped measurement time instances / total measurement time instances Measurement reduction rate in spatial domain (MRRS): MRRS = skipped beams to be measured/ total beams to be measured

Similar to the above cases we propose to define measurement overhead reduction rate formula, specific for the inter-frequency predictions, as following.

MRRF = skipped inter-freq. measurements / (total number of measured and predicted inter-freq). measurement

Use the following formula for measurement reduction for inter-frequency scenario:
Measurement reduction rate in frequency domain (MRRF):
MRRF = skipped inter-frequency measurements / (total number of measured and predicted inter-freq. measurements)

# Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	In this contribution, Case 2 and Case 3 are studied for evaluation of cell level measurement predictions.

Observation 2	For Case 2, L3 filtered cell-level measurements is used as model input to predict the L3 filtered cell level measurements.

Observation 3	For Case 3, L1 filtered beam-level results is used as model input to predict the L3 cell-level measurements.

Observation 4	For layer1 filtering, the existing L3 filtering mentioned in 38.331 section 5.5.3.2 with specific alpha 20% is reused.

Observation 5	For evaluation of Case 2 and Case 3, the single-cell prediction approach is used.

Observation 6	Option 1, i.e., sliding L1/L3 filtering is considered.

Observation 7	For Case 2-Case B: in low mobility scenarios (e.g., 30km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 0.5 dB while reducing the measurement overheads up to 80%.

Observation 8	For Case 2-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 0.5 dB while reducing the measurement overheads up to 80%.

Observation 9	For Case 3-Case B: in low mobility scenarios (e.g., 60km/h speed) the AIML model can predict the RSRP with RSRP difference (prediction error) of less than 1 dB while reducing the measurement overheads up to 80%.

Observation 10	For Case 3-Case B: in high mobility scenarios (e.g., 120km/h speed) the AIML model can predict the RSRP with a good accuracy, i.e., RSRP difference is less than 1 dB while reducing the measurement overheads up to 80%.

Observation 11	One-shot training and inference (e.g., ratio 1) has similar performance as multi-model training and inference (e.g., ratio 5 with different prediction steps)

Observation 12	Good prediction accuracy either requires large model size or larger model complexity.

Observation 13	Maximum prediction step is below 320ms for UE moving speed 60km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Observation 14	Maximum prediction step is below 320ms for UE moving speed 120km/h if we consider 1 dB as acceptable threshold for RSRP difference.

Observation 15	Prediction accuracy on temporal prediction case A - Case 2 should be further improved by e.g., complex model structure, cluster approach, assist information etc.

Observation 16	Unlike the good prediction performance of Case 2- Case A, in scenario of Case 3 – Case A, the RSRP differences are close or above 1 dB for all prediction steps.

Observation 17	Inter-Frequency prediction highly depends on the frequency distance (inversely proportion to correlation) between input/output frequencies, i.e., the higher the frequency distance the worse the prediction performance.

Observation 18	The Inter-frequency prediction error for predictions without any assistance information is slightly above 1 dB.

Observation 19	Higher-to-lower frequency prediction slightly outperforms lower-to-higher frequency prediction (on average 0.2 dB less error is observed when predicting a cell in 2GHz frequency).

Observation 20	Assistance information (e.g., location information) improves inter-frequency prediction performance (reducing the error below 1 dB).

Observation 21	The performance of In Case-3 (beam-level as input and cell-level as output) prediction is slightly worse than that of Case-2 prediction.

Observation 22	In Case-3 (beam-level as input and cell-level as output), Assistance information (e.g., location information) still improves inter-frequency prediction performance.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 investigate the impact of the set of beams used as input in the Case 3. i.e., which set of beams to use as input to the model.

Proposal 2	RAN2 include the results in the Annex A of the paper in the TR of the study item.

Proposal 3	RAN2 includes the inter-frequency Case-2 prediction results in the TR.

Proposal 4	RAN2 study the impact of assistance information (e.g., location information, etc.) as model input in improving the inter-frequency prediction accuracy.

Proposal 5	Use the following formula for measurement reduction for inter-frequency scenario: Measurement reduction rate in frequency domain (MRRF): MRRF = skipped inter-frequency measurements / (total number of measured and predicted inter-freq. measurements)

# References

RP-234055,	SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

R2-2404955, Summay of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO).

RAN2 Chairman’s Notes, RAN2#126

R2-2407849, Summary of [POST127][030][AI mobility] RRM simulation assumptions – Phase 1(OPPO)

R2-2407885, Draft Summary of [POST127][030][AI mobility] RRM simulation assumptions-phase 2(OPPO)