3GPP TSG-RAN WG2 #127bis	R2-2408749

Hefei, China, Oct 14th – 18th, 2024

Source:		NEC

Agenda item:		8.10.2	MRO enhancements for Rel-18 mobility features

Title:	MRO enhancements for LTM

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, the following agreements was made for MRO enhancements of LTM[1]:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

In this paper, we provide our further consideration on the MRO for LTM.

- Discussion

During last RAN2 meeting, it was agreed that RAN2 include the specific access type in the RLF report, i.e., whether it is RA-based or RA-less cell switch, and FFS wither explicitly or implicitly signalled. Currently, RA information (ra-InformationCommon IE) included in RLF-report if a handover failure happens. In case of RA-based LTM, same as legacy handover procedure, the RA information should be included in RLF-report.  In case of RACH-less LTM, since RA procedure is not performed by the UE during the LTM procedure, the RA information in RLF-report should be omitted. Therefore, the omitting of RA information in RLF-report can indicate that the access type of the LTM cell switch is RACH-less cell switch, and there is no need to introduce an explicit indication.

Proposal 1. Omit RA information (ra-InformationCommon IE) in RLF-report in case of RACH-less LTM cell switch.

Proposal 2. There is no need of explicit indication for the access type and omitting RA information in RLF-report implies the RACH-less LTM cell switch.

For RACH-less LTM, there are two ways for UE to determine the TA of the target cell, i.e., by the TA value carried in LTM cell switch command, or by UE-based TA measurement. If TA command is provided in LTM cell switch command, the UE accesses to the target cell based on the TA command without performing RA procedure. If TA command is not provided in the LTM cell switch command, and if the UE is configured with TA measurement and measured TA is available, the UE accesses to the target cell based on the measured TA without performing RA procedure. To help the network to determine whether the failure of RACH-less LTM is due to bad TA information provided by the network, or due to UE-based TA measurement, related information can be reported. For example, the UE can report indication indicating whether the TA is network provided or UE measured, and/or the TA value provided or measured by the UE.

To access to the target cell without RA procedure, both dynamic grant and configured grant can be used. And the UE shall apply the TCI state indicated in the LTM cell switch command for dynamic grant reception or configured grant resource selection. To analysis whether the failure of RACH-less LTM is due to improper TCI state information, the related information can be reported, as this information is not part of UE Context and the network may not store this information.

Proposal 3: Consider the following additional information related to RACH-less LTM in RLF-report:

- Indication indicating whether TA is provided by the network or measured by the UE

- TA value provided by the network or measured by the UE

- TCI state ID in the LTM cell switch command

For RACH-based LTM, both CFRA and CBRA can be performed. And CFRA can be based on either CFRA configuration in RRC configuration, or CFRA resource in LTM cell switch command. During RAN2 #125bis meeting, it was agreed that RAN2 considers RA report for MCG-LTM. However, it is not clear yet, whether the RA-report can be supported for a RA procedure for early sync purpose.

To support RACH-less based LTM, early synchronization can be performed, during which the UE can be triggered by PDCCH order from the network to perform RA to the LTM candidate cell. More specifically, UE would transmit preamble to the LTM candidate cell, and the RA procedure is considered as completed. No RAR would be received by the UE, instead, the TA information would be transmitted from the candidate DU to the source DU via CU, and the TA value is sent together in the LTM cell switch command to the UE by the source DU. The UE is not aware of whether the RA procedure for early UL sync is successful or not, and the retransmission of the preamble is indicated by the source cell. The RA resource is allocated per candidate cell per gNB-DU by the candidate DU, so it is beneficial to record RA information for better usage of RA resource for early sync, however, whether there is the worth to work on the related function, and whether this is done by the UE or network needs further discussion.

Observation: The UE is not aware of whether the random access for early UL sync is successful or not.

Proposal 4: Discusses whether to support RA-report enhancement for early UL sync procedure.

# 3. Conclusion

In this paper, we discuss on further consideration on the MRO for LTM, and have the following proposals:

Proposal 1. Omit RA information (ra-InformationCommon IE) in RLF-report in case of RACH-less LTM cell switch.

Proposal 2. There is no need of explicit indication for the access type and omitting RA information in RLF-report implies the RACH-less LTM cell switch.

Proposal 3: Consider the following additional information related to RACH-less LTM in RLF-report:

- Indication indicating whether TA is provided by the network or measured by the UE

- TA value provided by the network or measured by the UE

- TCI state ID in the LTM cell switch command

Observation: The UE is not aware of whether the random access for early UL sync is successful or not.

Proposal 4: Discusses whether to support RA-report enhancement for early UL sync procedure.

# Reference

[1] Meeting report of RAN2 #126