3GPP TSG-RAN WG2 Meeting #127bis	R2-2408752

Hefei, China, 14 – 18 October 2024

Agenda item:	8.6.2

Source:	Nokia

Title:	On inter-CU aspects for LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the baseline procedure for inter-CU LTM support and identify the related key issues.

# Discussion

We analyze the following key issues related to Inter-CU LTM and provide our proposals on way forward on these issues in this paper.

Issue #1: CSI resource and reporting configuration

Issue #2: Signaling overhead for supporting CSI-RS resources in LTM CSI Resource configuration

Issue #3:  Remaining issues for DC-LTM

Issue #4: Scope for Reference configuration

Issue #5: SN Security key Regeneration for Inter MN MCG-LTM

Issue #6: Support of configured grant-based RACH-less LTM

## LTM CSI Resource and Reporting configuration

RAN2 has agreed the following for the CSI RS Resource configuration and report configuration in Inter-CU LTM scenario. We discuss further changes needed for Inter-CU scenarios with this agreement.

RAN2#127 agreements: CSI resource and report configuration - The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration. - The source CU is responsible to generate the common CSI resource configuration.

In the baseline Rel-18 LTM preparation message sequence, the candidate cells provides the RS configurations (SSB configurations). The CU generates the LTM CSI resource sets that includes the list of SSB and candidate cell indices.  As the source CU only knows the RS configuration of candidate cell, the LTM-CSI-Resource configuration it generates needs to be based on the L3 measurement result(s) that triggered the preparation. The source CU is not aware of any information about the potential target cells of each candidate cell for subsequent mobility for generating candidate-cell specific resource-configuration.

Observation 1: L3 measurement result that triggers the LTM Preparation is the basis for source CU for generation of  LTM-CSI Resource Config in Rel-18.

Note that all the details discussed in this section apply to both SSBs and CSI-RSs; however, for simplicity, we only mention SSBs.

With L3 measurement results as basis, the source CU can generate the LTM CSI Resource config in either of the following means.

- O1: Source CU generates single LTM CSI resource config that includes all the SSBs of all candidate cells prepared for LTM.

- O2: Source CU generates separate LTM CSI resource config for the SSBs of each candidate cell. In this case the source CU generates N LTM CSI resource configs where N is the number of LTM candidate cells.

Observation 2: For source CU generation of LTM CSI resource config , either single resource config containing all candidate SSBs from all prepared candidate cells OR separate resource config for each candidate are the only possible options.

When the source CU provides the LTM CSI resource config to a candidate cell, the candidate CU generates a LTM report config that points to the LTM CSI resource config for the candidate cells controlled by this CU.

In case of O1 all the LTM report configs needs to point to the single LTM CSI resource config ID. This resource config is large resource set consists of all RS of all candidate cells. In this option, the report config will map to single large LTM CSI resource config. However, UE does not have any information on which candidate cells to measure when it is served by different candidate cells.  The UE may need to measure all the candidate SSB in this case similar to L3 measurements. This is not efficient considering the regular L1 measurement result that needs to provide best L1 measurements periodically.

In case of O2, the candidate CU can decide to map its each of its report config to different LTM CSI resource config based on their preference for subsequent mobility. In this option candidate CU needs to have separate report configurations and also separate UL resources for reporting measurement results of each of the target cells. For making decision to select between two target cells, the candidate CU needs to receive reports associated with two separate report configs. Because Rel-18 LTM report config can only be linked to one LTM CSI resource config.

In case of subsequent mobility, the LTM measurements and reporting needs to be determined based on the current serving cell and its preference for outward mobility. Hence, in both these options of source CU generation of LTM CSI resource config further information needed from candidate CU to  modify the LTM measurements and reporting for subsequent mobility.

Observation 3: With current RAN2 agreements on LTM CSI resource config and re-use of Rel-18 LTM report config, inter-CU subsequent LTM measurements and reporting is not efficient from signalling and UE processing perspective.

Above problems can be mitigated in following ways.

- For O1, when the source CU generates a single LTM CSI resource config consists of all candidate beams, there can be two simple ways to solve the above issues:

  - Alternative 1 – Each candidate cell can provide additional information selecting a subset of cells/resources from the LTM CSI resource config (e.g., a bitmap of the cell or resource indices) to be considered for LTM measurements when it becomes serving cell. This information can be included within the LTM report config as additional information in addition to a pointer to LTM CSI resource config.

  - Alternative 2 – Each candidate cell should allow to dynamically select a subset of cells/SSBs form the LTM CSI resource config associated with a report configuration, and provide it to the UE, e.g., via MAC-CE. This approach may have additional benefit, as appropriate resources can be selected dynamically as the UE moves.

- For O2, when the multiple LTM CSI resource configs are generated, each containing the SSBs of only one candidate cell, the report configuration can be optimised if the LTM report config can be mapped to more than one LTM CSI resource configs.

Proposal 1: In order to provide flexibility for each candidate cell to select its own SSBs/candidate cells for LTM measurements from a source CU generated LTM CSI resource config,  RAN2 to consider at least one of the following options:

- Alternative 1 -a LTM report config includes additional information on the candidate cells /SSBs to measure from the linked LTM CSI resource config for subsequent mobility.

- Alternative 2 - Support dynamic selection of a subset of resources for measurements from the LTM resource config associated with a report config.

Proposal 2: For the scenario when source CU generates separate LTM CSI resource configs, each containing SSBs of one candidate cell,  RAN2 to consider that a LTM report config can be linked to more than one LTM CSI resource configs..

Proposal 3: The above solutions are applicable to both SSBs and CSI-RS based LTM measurements.

## Signaling Overhead of CSI-RSs Configuration for LTM

One of the objectives for Rel-19 is to extend the LTM measurements for RS type CSI-RS. While RAN1 is working on the details on RAN1 procedure to consider CSI-RS for LTM measurements, there will be RAN2 impacts related to configuration of CSI-RS as RS for LTM measurements.  .

In the last RAN2 meeting, it was agreed that the Rel-18 signaling structure for LTM CSI resource and report configuration would be reused for inter-CU LTM, i.e., a common CSI resource configuration and cell-specific CSI report configuration. Similarly, RAN1 has agreed that explicit configuration of CSI-RS resources for candidate cells will be supported, similar to how LTM-SSB-Config is provided for each candidate cell in Rel-18.

Moreover, RAN1 has agreed to use periodic CSI-RSs from a candidate cell, with detailed configuration for each CSI-RS as outlined above. Typically, a CSI-RS configuration includes more detailed information (e.g., finer time and frequency granularity) compared to the SSB configuration. For LTM, the candidate configuration for a candidate cell will also include parameters that the UE will apply upon switching to that cell, potentially including CSI-RS configurations for beam management and CSI acquisition within the CSI-Meas-Config

As a result, configuring CSI-RS resources for LTM L1 measurements could introduce significant overhead. Mechanisms should be developed to minimize this overhead, particularly by eliminating duplicate or redundant CSI-RS configurations inside and outside of the candidate cell configuration.

Observation 4: For LTM measurements, a larger set of CSI-RSs needs to be configured in LTM-CSI-Resource Configuration compared to SSB.

Observation 5: Each CSI-RS configuration for LTM contains more detailed information, and the servingCell-Config within RRC-Configuration of candidate cell also includes same CSI-RS configurations for beam management and CSI acquisition after a cell switch.

Observation 6: Configuration of CSI-RS resources for LTM Layer 1 measurements may lead to significant overhead.

Proposal 4: RAN2 to study mechanisms for signalling efficient configuration of CSI-RS resource for LTM minimizing overhead by removing redundant CSI-RS-related configurations across LTM-CSI-RS-configuration and the CSI-RS-Configuration included within serving-cell configuration.

## Reference configuration generation for Inter-CU LTM

In RAN2#127, the following was agreed:

RAN2#127 Agreement: Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

Considering the discussions during the session, the main motivation behind the above agreement was to avoid additional signaling on the NW side associated with generating the multiple reference configurations. The reference configuration designed in Rel. 18 enjoys the following properties:

1) It is intra-CU.

2) It can be same for all LTM candidates.

3) The LTM candidates which are not given as delta configuration over the reference configuration, must be given as complete configurations.

Observation 7: The reference configuration for Rel. 18 LTM is intra-CU and same configuration is referred by  all LTM candidates for generation of delta configuration. The candidate configurations that are not given as delta over the reference, are given as complete configurations.

In Inter-CU scenario the signaling overhead related to LTM configuration can be reduced if the candidate CU that serves most candidate cells is considered for generation of reference configuration. This would be simpler than having source CU generates reference configuration for all candidate CU. The simplest approach would be to let the candidate CU that has the largest number of LTM candidates generate the reference configuration. The other LTM candidates can be given as complete configuration.

Proposal 5: The Rel. 19 LTM configuration is intra-CU and is generated by the CU which has the largest number of LTM candidates. The other LTM configurations are given as complete configurations.

## Support of configured grant-based RACH-less LTM

RAN2#126 Agreement:

Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM.

In RAN2#126, the following agreement was made on the support of the first UL transmission for RACH-less inter-CU LTM:

Compared to dynamic grants (DGs), configured grants (CGs) enable faster access to the target cell, as the UE does not need to monitor PDCCH to receive the UL grant and can directly perform its first UL transmission on one of the valid occasions of the provided CG.

Nevertheless, a known issue of the use of CGs in LTM is the potential excessive resource reservation to support a single RACH-less LTM switch. For each UE prepared for RACH-less LTM and for each candidate cell, a CG must be included in the candidate’s RRC configuration provided to the UE during the preparation phase. Therefore, the PUSCH resources of these grants are reserved for a long period of time stretching from the preparation phase to the execution of one or multiple cell switches and they may never be used, if the UE does not switch to the corresponding cell. The issue may be even more pronounced in inter-CU LTM, as it cannot be solved or mitigated with intra-gNB resource scheduling optimizations.

Observation 8: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

On the other hand, CG-based RACH-less LTM results in reduced interruption time, which is one of the main objectives of LTM. Therefore, a mechanism to remedy the above issue is needed. To this end, the probabilistic nature of the use of the provided PUSCH resources can be exploited in a similar way as it is done for CFRA resources. PUSCH resources can be shared by multiple UEs, as it is unlikely that they switch to the same candidate cell at the same time. Still, it is possible that UEs sharing the same PUSCH resource will use the same CG occasion for their first UL transmission to the candidate. Therefore, coordination of the use of shared PUSCH resources is required. As cell switch execution is controlled by the source node, shared PUSCH resources can be provided by the candidate node to the source, which in turn can coordinate their use during cell switch.

Proposal 6: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

## DC-LTM

### 2.3.1 Scenarios

RAN2 made the following agreements related Inter-CU MCG LTM in RAN2#127:

RAN2#127 agreements: Inter-CU MCG LTM: - SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration). - Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied). - For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

RAN2 also made the following agreement in RAN2#126:

RAN2#126 agreement: Clarification of DC - An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells.

Based on the above listed agreement from RAN2 #126, simultaneous configuration of Inter-CU MCG LTM and inter-CU SCG LTM is not in Rel-19 scope.

Observation 9: Simultaneous configuration of Inter-CU MCG LTM and inter-CU SCG LTM is not in Rel-19 scope.

Observation 10: Intra-CU MCG LTM and Intra-CU SCG LTM are already supported in Rel-18.

In our view, the following scenarios are in the scope of Rel-19 and must be supported:

- Simultaneous configuration and execution of Intra-CU MCG LTM and inter-CU SCG LTM

- Simultaneous configuration and execution of Inter-CU MCG LTM and Intra-CU SCG LTM

Proposal 7: RAN2 to discuss and agree to support the simultaneous configuration and execution of a) Intra-CU MCG LTM and Inter-CU SCG LTM and b) Inter-CU MCG LTM and Intra-CU SCG LTM.

### 2.3.2 Bearer Remapping impacts for DC-LTM

In the case of interworking of Intra-CU MCG LTM and inter-CU SCG LTM, it is likely that each candidate SN may prepare different SCG configuration with reference to SCG configuration at the source, this may result in bearer remapping (e.g. remapping of SCG bearers to the MCG) in some inter-CU LTM PSCell change scenarios. We think, the existing Rel-18 framework should be able to handle this.

Observation 11: The interworking of Intra-CU MCG LTM and inter-CU SCG LTM, may result in bearer remapping (e.g. remapping of SCG bearers to the MCG) in some inter-CU LTM PSCell change scenarios, which may be handled by the existing Rel-18 framework.

### 2.3.3 LTM Candidate Configurations with /without SCG for DC-LTM

As per the WI scope, an inter-CU MCG LTM candidate node preparing a different target SN other than the source SN is not supported. Thus, it may be likely that some of the inter-CU MCG LTM candidate nodes during HO preparation may prepare MCG + SCG configuration (if candidate MN could retain the source SN) or MCG only configuration (if candidate MN couldn’t retain the source SN). This may result in to a combination of inter-CU MCG LTM preparation with SCG retained and SCG released. In such a scenario, for a dual connectivity enabled UE, if the Inter-CU LTM PCell change results in PCell HO with SCG release at the target, then UE may continue with single connectivity if subsequent cell change is configured. Enabling dual connectivity again in such a scenario would require reconfiguring the UE.

Observation 12: As per the WI scope, an inter-CU MCG LTM candidate node preparing a different target SN other than the source SN is not supported.

It is likely that some of the inter-CU MCG LTM candidate nodes during HO preparation may prepare MCG + SCG configuration or MCG only configuration; resulting in a combination of inter-CU MCG LTM preparation with SCG retained and SCG released.  In the scenario described in the previous observation, an Inter-CU LTM PCell change results in PCell HO with SCG release at the target, which means dual connectivity at the UE is converted to single connectivity and may have to continue with single connectivity if configured with subsequent cell change.

Observation 13: For UE operating in dual connectivity, preparation of LTM candidate configurations with and without SCG is valid scenario for consideration.

We think the scenario described above may be handled via one of the following listed options:

Option 1: Do not include an inter-CU MCG LTM candidate that has prepared MCG only configuration, as a subsequent cell change candidate if network wants to retain dual connectivity after subsequent cell change.

Option 2: Include all the inter-CU MCG LTM candidate nodes in the subsequent cell change configuration, irrespective of whether a candidate node has prepared MCG + SCG configuration or MCG only configuration. This option may result into DC converted to SC if an Inter-CU LTM PCell change results in PCell HO with SCG release at the target.

Supporting ‘option 2’ may require some enhancements to existing signaling to enable the UE to release SCG configuration of the last serving PSCell, if the PCell HO results in SCG release the target cell to avoid inconsistent UE and Network behavior.

Proposal 8: Network implementation for preparation of candidate LTM configurations with and without SCG is supported.  RAN2 to study the signalling procedure changes needed for this option.

### 2.3.4 MCG-Failure Recovery for DC-LTM

In case of RLF detected at the MCG, the current behavior for LTM recovery is described in 3GPP specification TS 38.300 for NR and NG-RAN overall  description, Stage-2.

Section 9.2.3.1 (Mobility in RRC_CONNECTED):

“When LTM execution attempt triggered by LTM cell switch command MAC CE fails, the UE performs cell selection and if the selected cell is an LTM candidate cell and if network configured the UE to try LTM after LTM execution failure, then the UE attempts RACH-based LTM execution once, otherwise re-establishment is performed”

The drawback of this legacy LTM recovery procedure is it has longer interruption duration and the connectivity between the UE and the network is interrupted completely while the recovery process is ongoing.  This interruption can be reduced if MCG Failure recovery is configured along with MCG-LTM.

The MCG Link recovery through SCG was introduced in Rel-16 to allow the network to reconfigure the UE in RLF scenario without interrupting the current RRC connection and user-plane operation via SCG for DC connection. Under fast MCG link recovery procedure as there is no direct link between the source MN and UE due to RLF, the UE triggers MCG Failure Information towards the SN-CU upon detecting RLF at the MCG. SN-CU upon receiving the MCG Failure Information forwards the same to the MN-CU. The MN-CU upon reception of the MCG Failure Information prepares RRCReconfigurationwithSync and shares the updated configuration for re-selection to the UE via SN. This procedure enables the UE to maintain connectivity with the network over the secondary link.

When MCG-LTM is configured  the prepared LTM configurations can be considered for MCG Recovery if the measurements indicates prepared LTM cells as suitable for recovery. This recovery is faster compared to LTM recovery procedure and also have lesser interruption for user plane traffic via SCG.

Observation 14: MCG Failure recovery via SCG has shorter interruption time than LTM Recovery procedure for MCG-LTM in DC scenario

Proposal 9: RAN2 to support fast MCG link recovery procedure as recovery procedure for UE configured with MCG-LTM. Recovery via prepared LTM configuration without new RRC-Reconfiguration is also considered.

## SN Key Regeneration for Inter-CU MCG LTM with unchanged SCG

The security key generation principles described in TS33.501 indicates that whenever master key associated with MCG is changed the network and UE need to regenerate new S-KgNB and the derived keys associated with SCG bearers.

In classic handover scenario, above requirement is achieved by MN including explicit SK-counter value in the RRC Reconfiguration message that includes master-key change or NCC change.

Observation 15: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN .

In case of Inter-MN MCG LTM with same SCG, whenever inter MN LTM is executed the UE and MN can apply the new K-GNB based on NCC value provided in the MAC CE command or based on stored NCC values. The actual solution is discussed in SA3. However, how the SN keys are regenerated in this scenario is not yet discussed in RAN2. This issue is not indicated as key issue in the LS to SA3.

Observation 16: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM is not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

There can be multiple solutions to enable regeneration of SN keys without additional RRC configuration. Some of the options are as follows:

- O1: LTM Configuration includes SK-counter value for each Rel-19 ID (the identifier to differentiate Inter-MN MCG LTM). Whenever inter MN MCG LTM is executed both UE and MN make use of this SK-counter for generation of SN keys. One SK counter value is sufficient to handle multiple MN changes because the master key is changed in this case and same SK-counter value can be reused.

- O2: Without explicit SK-counter value, for inter-MN LTM with SCG, the UE and MN should regenerate new SN-keys with default SK-counter value =1. For further inter-SN change without MN change, MN should use the value starting from 2. In this case LTM with SCG unchanged the LTM candidate configuration need not include any SK-counter value.

Proposal 10:  RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM and seek SA3 views through LS.

# Conclusions

In this paper, we analyzed the remaining open issues for Inter-CU LTM and made the following observations and proposals.

LTM CSI Resource and Report Configuration changes

Observation 1: L3 measurement result that triggers the LTM Preparation is the basis for source CU for generation of  LTM-CSI Resource Config in Rel-18.

Observation 2: For source CU generation of LTM CSI resource config , either single resource config containing all candidate SSBs from all prepared candidate cells OR separate resource config for each candidate are the only possible options.

Observation 3: With current RAN2 agreements on LTM CSI resource config and re-use of Rel-18 LTM report config, inter-CU subsequent LTM measurements and reporting is not efficient from signalling and UE processing perspective.

Proposal 1: In order to provide flexibility for each candidate cell to select its own SSBs/candidate cells for LTM measurements from a source CU generated LTM CSI resource config,  RAN2 to consider at least one of the following options:

•	Alternative 1 -a LTM report config includes additional information on the candidate cells /SSBs to measure from the linked LTM CSI resource config for subsequent mobility.

•	 Alternative 2 - Support dynamic selection of a subset of resources for measurements from the LTM resource config associated with a report config.

Proposal 2: For the scenario when source CU generates separate LTM CSI resource configs, each containing SSBs of one candidate cell, RAN2 to consider that a LTM report config can be linked to more than one LTM CSI resource configs.

Proposal 3: The above solutions are applicable to both SSBs and CSI-RS based LTM measurements.

Signalling Overhead for CSI-RS Configuration

Observation 4: For LTM measurements, a larger set of CSI-RSs needs to be configured in LTM-CSI-Resource Configuration compared to SSB.

Observation 5: Each CSI-RS configuration for LTM contains more detailed information, and the servingCell-Config within RRC-Configuration of candidate cell also includes same CSI-RS configurations for beam management and CSI acquisition after a cell switch.

Observation 6: Configuration of CSI-RS resources for LTM Layer 1 measurements may lead to significant overhead.

Proposal 4: RAN2 to study mechanisms for signalling efficient configuration of CSI-RS resource for LTM minimizing overhead by removing redundant CSI-RS-related configurations across LTM-CSI-RS-configuration and the CSI-RS-Configuration included within serving-cell configuration.

Reference Configuration generation

Observation 7: The reference configuration for Rel. 18 LTM is intra-CU and same configuration is referred by all LTM candidates for generation of delta configuration. The candidate configurations that are not given as delta over the reference, are given as complete configurations.

Proposal 5: The Rel. 19 reference configuration is generated by the CU which has the largest number of LTM candidates. The other LTM configurations are given as complete configurations.

CG Based RACH-less Inter CU LTM

Observation 8: Support of CG-based inter-CU LTM may lead to excessive PUSCH resource reservation.

Proposal 6: To avoid excessive resource reservation overhead for CG-based RACH-less LTM, PUSCH resources can be shared among multiple UEs. PUSCH resource sharing among UEs should be coordinated by the source node.

DC-LTM

Scenarios

Observation 9: Simultaneous configuration of Inter-CU MCG LTM and inter-CU SCG LTM is not in Rel-19 scope.

Observation 10: Intra-CU MCG LTM and Intra-CU SCG LTM are already supported in Rel-18.

Proposal 7: RAN2 to discuss and agree to support the simultaneous configuration and execution of a) Intra-CU MCG LTM and Inter-CU SCG LTM and b) Inter-CU MCG LTM and Intra-CU SCG LTM.

Bearer Remapping

Observation 11: The interworking of Intra-CU MCG LTM and inter-CU SCG LTM, may result in bearer remapping (e.g. remapping of SCG bearers to the MCG) in some inter-CU LTM PSCell change scenarios, which may be handled by the existing Rel-18 framework.

LTM Candidate Configurations with and without SCG

Observation 12: As per the WI scope, an inter-CU MCG LTM candidate node preparing a different target SN other than the source SN is not supported.

Observation 13: For UE operating in dual connectivity, preparation of LTM candidate configurations with and without SCG is valid scenario for consideration.

Proposal 8: Network implementation for preparation of candidate LTM configurations with and without SCG is supported.  RAN2 to study the signalling procedure changes needed for this option.

LTM Recovery for DC-LTM

Observation 14: MCG Failure recovery via SCG has shorter interruption time than LTM Recovery procedure for MCG-LTM in DC scenario

Proposal 9: RAN2 to support fast MCG link recovery procedure as recovery procedure for UE configured with MCG-LTM. Recovery via prepared LTM configuration without new RRC-Reconfiguration is considered for providing recovery configuration.

SN-Key Regeneration for Inter-CU LTM with unchanged SCG

Observation 15: In classic Inter-MN handover scenario without SCG change, the SK-counter value needs to be included along with master-key change configuration to enable regeneration of new security keys for SN.

Observation 16: Issues related to regeneration of SN-Keys during Inter-MN MCG LTM was not discussed within RAN2. Not included in the list of issues for SA3 to address in the previous LS to SA3.

Proposal 10:  RAN2 discuss and agree upon the potential options to trigger SN key regeneration for Inter-MN MCG-LTM and seek SA3 views through LS.

# Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Mar 2024

- R2-2402907, “Important topics for inter-CU LTM”, ‘Apple Inc.’, RAN2#125-bis