3GPP TSG-RAN WG2 Meeting #127bis	R2-2408754

HeFei, China, 14th Oct – 18th Oct 2024

Agenda item:	8.9.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Radio-Interface Impacts for IoT-NTN SF Operations

WID/SID:	IoT_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

One of the objectives for Rel-19 WID for IoT-NTN enhancements (RP-240776) is

Support of  Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

- Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4]

- At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Following are the agreements in RAN2 on the SF operation for IoT-NTN until RAN2-127.

RAN2-126 Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

RAN2-127 Agreements:

1.	RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

2.	RAN2 will consider both single satellite pass and multiple satellite pass scenarios

3.	RAN2 will consider both MO and MT data within scope

4.	UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature)

5.	RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected.

7.	RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

# 2	Discussion

## 2.1 Background

In some of the IoT-NTN deployments the NTN nodes may not have connectivity towards UE and the ground station at the same time. Such cases are applicable when the ground station connectivity is available only from central region and the moving NTN nodes need to cover multiple coverage areas.  For such scenarios, storage capability at NTN node is needed to handle the store and forward operation between UE and Ground node whenever they are available.  The scenario is depicted in the following Figure. The Figure illustrates the scenario where NTN node only contain eNB components. The NTN node need to store the uplink packets received from the IoT-NTN UE when it is serving area without coverage of the Ground node, and it need to store the downlink data received from ground station and the device location is not served by NTN node.

For store and forward operation, the NTN nodes may consist of only eNB components or some components of eNB and CN elements.  The radio interface signaling procedures and impacts to NAS procedures may vary depending on the architecture. For example, in case of eNB at NTN node, the NAS level registration and signaling procedures may not be completed in a single procedure if the CN connectivity is not available at the time of NAS procedures initiated by the UE.  In case of eNB+MME component at NTN node the registration procedure can be completed using a single procedure and security aspects will be handled by the MME related components.

## 2.2  RAN signalling procedure changes for MME Deployment Options

RAN2 127 made following agreement related to the architecture options for further RAN2 impacts

RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options)

The signalling steps for SF operation for these architecture options are illustrated below

For Split-MME on board option

For control-plane CIoT solution where CIoT packets are delivered as NAS payload, followings are signaling steps for the store and forward operation.

For NAS signalling procedure

- The uplink NAS signalling messages are received at MME on-board and MME will send information towards the UE with wait timer and the list of satellite-ID on which the UE can complete the remaining procedure.

- The NAS component at UE need to start timer for resuming the procedure later at different satellite.

For CIOT packet transmission

- UE sends uplink data transmission as NAS payload to MME on-board and MME stores the data packets when it does not have feeder link connectivity

- When feeder link is available the MME function delivers the packets to MME at Ground station which further delivers the packets to SGW in Ground station.

For Full- CN on board option

For signalling procedure

- RRC and NAS signalling are not impacted as the complete MME functionality is available in the satellite.

- Each node on board operates with separate MME-ID or PLMN ID. NAS signalling further provides details on other satellites where the UE can continue its operation.

For CIoT data Transmission (CIoT CP)

  - The CIoT-Control Plane packets received via NAS signalling is stored at SGW on board when feeder link is not available.

  - The SGW on board transfers the packets to SGW on ground station on feeder-link connectivity.

Observation 1: For CIoT-CP operation the RAN2 signalling procedure for CIOT data transmission is not impacted for both MME-Split and Full-CN Architecture options.

The store and forward deployments of PLMN is expected to operate in same architecture option across its cells. Hence all the UE subscribed for this PLMN needs to support the modified NAS signalling procedure that includes delayed procedure and reception of information on selected satellite-ID for the NAS procedure.

Observation 2: RAN2 assumes Store-forward PLMN operates with same architecture option across all NTN cells.

Proposal 1: RAN2 signalling is not needed to indicate the architecture option (Split MME or Full CN).

For the Split MME architecture, the UE need to wait for one of the listed satellites to complete the pending NAS procedure. However it is possible that the UE may find different satellite on wait timer expiry. In such case whether the UE should terminate the current procedure or attempt for recovery procedure via the new satellite needs to be addressed. Hence, we propose that RAN2 to check with SA2 on how to address the scenario where UE does not find the listed satellites on wait timer expiry to complete the procedure.

Proposal 2: RAN2 checks with SA2 to address the unknown UE mobility scenario in which the UE does not find the target SAT for completing the remaining network procedure after the waiting time but another SAT instead.

The signalling procedure considered in SA2 study only focus on the procedures for CIoT-CP solution where the CIoT packets are delivered as NAS PDU using NAS security keys.  However, there will be additional impacts on RAN2 signalling procedure and security aspects for CIoT-UP solution mainly for the MME Split Option.  We propose that RAN2 discuss on whether the SF mode operation needs to be supported for CIoT-UP operation and if yes RAN2 to identify the signalling procedure changes and user-plane operation impacts for this procedure. In such case LS to SA2 can be considered that RAN2 intend to further investigate CIoT-UP solution that may need further changes in SA2 procedures.

Observation 3: CIoT-UP solution support will require additional RAN impacts for the user plane data forwarding for MME split option.

Observation 4: SA2 Technical report only focus on CIoT-CP solution as the basis for the store-forward study.

Proposal 3:  RAN2 to discuss and conclude on the need for support of CIOT-UP solution for SF operation.

## 2.3 Awareness of SF mode operation at UE

Following is the RAN2-127 agreement on indication of SF mode operation towards UE

UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature).

During RAN2-127 the need to indicate whether the satellite node supports SF mode is also discussed in addition to the indication of operating mode via system Information.  The presence of new parameter ‘SF-Mode’ itself can be considered as the support of SF operation in the cell. Additional indication for SF capability is not needed.

Proposal 4: Separate indication about the SF capability of the NTN cell is not needed. Presence of SF-Mode parameter in System Information can be considered as indication of SF capability.

To avoid changes in system information for the mode change that is expected in periodic manner between SF mode and normal mode, it is preferred that the mode change is indicated via new parameter that indicate the absolute time of the mode transition. This parameter can be similar to T-service. In this case the NW can switch the mode without explicit system information modification procedure. The NW will also change the T-service parameter value corresponds to the new mode simultaneously.

Proposal 5:  For mode transmission new parameter similar to T-Service indicating the duration of current mode is included in system information.

When NW switches the mode between SF mode and normal mode, the UE may need to switch to the corresponding mode as well. To avoid the complexity and poor performance of switching between SF mode and normal mode at the UE side , while the operation mode switching happens in NW side, the UE may be configured to prepare for the upcoming mode switching based on the new parameter indicating the absolute time of mode transition.

Proposal 6: RAN2 to investigate the impacts of UE behaviour for mode change between SF mode and normal mode.

## 2.3 Access Control and Paging Changes for SF operation

On the access barring related to legacy UE and SF mode capable UE following agreement was made in RAN2-127

RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication).

When IoT-NTN satellite node operating in SF mode there may be needed to disallow the SF mode capable UE to the cell due to RAN overloading or in case of SF mode operation due to storage capacity limits.  It is possible to realise this access control via new parameter or by extending the SF mode operation parameter.  As the access control based on storage capacity is needed in SF mode operation, we think extension of SF mode with additional state indication is preferred than separate cellBarred indication.

Proposal 7:  Extension of Enumeration for the SF mode parameter is considered for allow/disallow SF capable UE. (i.e., New value for SF-Mode as INACTIVE).

When NTN node is serving new area, it may need to handle the new data transmissions from the coverage area and also complete the data transmission in downlink that was received and stored from CN at the time of earlier connectivity. In such cases the access control and data transmission across these two variants of users needs to be differentiated. Mainly to avoid the delivering the stored data becomes invalid or time-out at UE.  In the same way for the uplink transmissions depending on the storage available at NTN node to receive and buffer the packets the access control and scheduling of transmissions needs to be handled.

Proposal 8:  RAN2 to investigate access control and scheduling related changes for S&F operation.

When the mobile originated IoT data transmission that requires application level ack are sent to eNB operating in store and forward mode the UE may expect the corresponding downlink packets after connecting to the next eNB. In this case UE monitors the next gNB for receiving ACK and new MT packets if any stored in the eNB. The data packet available at this eNB may contain ACK for earlier MO traffic and MT traffic toward the UE. In some other cases, the next eNB may not have any downlink ACK for earlier transmission depending on its CN connection time. In different scenarios based on the network awareness of CN connectivity time and the actual stored downlink data to be delivered towards UE, it is possible to improve the paging monitoring towards UE.

Proposal 9:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

# 3	Conclusion

In this paper we analyse the radio signalling procedure impacts for store and forward operation. Following proposals are made based on the analysis.

RAN2 Impacts for SA2 Agreed MME Deployment Options

Observation 1: For CIoT-CP operation the RAN2 signalling procedure for CIOT data transmission is not impacted for both MME-Split and Full-CN Architecture options.

Observation 2: RAN2 assumes Store-forward PLMN operates with same architecture option across all NTN cells.

Proposal 1: RAN2 signalling is not needed to indicate the architecture option (Split MME or Full CN).

Proposal 2: RAN2 checks with SA2 to address the unknown UE mobility scenario in which the UE does not find the target SAT for completing the remaining network procedure after the waiting time but another SAT instead.

Observation 3: CIoT-UP solution support will require additional RAN impacts for the user plane data forwarding for MME split option.

Observation 4: SA2 Technical report only focus on CIoT-CP solution as the basis for the store-forward study.

Proposal 3:  RAN2 to discuss and conclude on the need for support of CIOT-UP solution for SF operation.

Indication of SF mode changes to UE

Proposal 4: Separate indication about the SF capability of the NTN cell is not needed. Presence of SF-Mode parameter in System Information can be considered as indication of SF capability.

Proposal 5:  For mode transmission new parameter similar to T-Service indicating the duration of current mode is included in system information.

Proposal 6: RAN2 to investigate the impacts of UE behaviour for mode change between SF mode and normal mode.

Access Control and Paging changes for SF Mode

Proposal 7:  Extension of Enumeration for the SF mode parameter is considered for allow/disallow SF capable UE. (i.e., New value for SF-Mode as INACTIVE).

Proposal 8:  RAN2 to investigate access control and scheduling related changes for S&F operation.

Proposal 9:  RAN2 to consider paging enhancements in SF mode for delivering ACK for MO Traffic and MT Traffic towards IoT-NTN UE.

# Reference

[1]	RP-240776 WID for IoT-NTN Enhancements