3GPP TSG-RAN WG2 Meeting #127bis	R2-2408757

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		7.11.2

Source:		Samsung

Title:			Corrections for MII

Document for:	Discussion & Decision

# 1 Introduction

In this contribution, we discuss corrections for MBS Interest Indication (MII) that may be addressed for Rel-17 and/or Rel-18 specifications.

# 2 Discussion

## 2.1 Use of MII for add/release DRBs/multicast MRBs

As MII may indicate the loss or change of interest for the UE, consequently it also facilitates the gNB to use this information for the addition of DRBs/multicast MRBs. However, present stage 2 specification only specifies about the usage of MII by the gNB for the release of DRBs/multicast MRBs. It seems more appropriate to generalize the description in the specification as proposed in TP1.

Proposal 1: Adopt TP1 in stage-2 spec for the use of MII by the gNB to add/release DRBs/multicast MRBs.

TP 1: (TS 38.300)

##### 16.10.6.5.2 Service Continuity in RRC_CONNECTED To ensure service continuity of MBS broadcast, the UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, consisting of the following information: - List of MBS frequencies UE is receiving or interested to receive, sorted in decreasing order of interest; - Priority between the reception of all listed MBS frequencies and the reception of any unicast bearer and multicast MRB; - List of MBS broadcast services the UE is receiving or interested to receive, in case SIB20 is provided for PCell or SCell. MBS Interest Indication information reporting can be implicitly enabled/disabled by the presence of SIB21. The gNB may use this information, together with the information about the UE's capabilities (e.g., supported band combinations), when providing an RRC configuration and/or downlink assignments to the UE or to add/release DRBs/multicast MRBs, to allow the UE to receive the MBS services the UE is interested in. MBS Interest Indication information can be exchanged between source gNB and target gNB during handover.

## 2.2 Determining ‘about to start’ sessions

In clause 5.9.4.3 and clause 5.9.4.4, NOTE in the RRC specification is not specifying that the UE may determine whether the session is ‘about to start’ from the start time indicated in the User Service Description (USD). Presently, the NOTE is only addressing the determination of whether the session is ‘ongoing’ from the start and stop time indicated in USD. To bring completeness and clarity, NOTE should be amended as proposed in TP2.

Proposal 2: Adopt TP2 for NOTE in clause 5.9.4.3 and clause 5.9.4.4 in RRC spec and also clarify that UE may determine whether session is ‘about to start’ from the start time indicated in USD.

TP 2: (TS 38.331)

#### 5.9.4.3 MBS frequencies of interest determination The UE shall: 1> consider a frequency to be part of the MBS frequencies of interest if the following conditions are met: 2> at least one MBS session the UE is receiving or interested to receive via a broadcast MRB is ongoing or about to start; and NOTE 1: The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67]. 2> for at least one of these MBS sessions, SIB21 acquired from the PCell or a non-serving cell includes mapping between the concerned frequency and one or more MBS FSAIs indicated in the USD for this session, or for at least one of these MBS sessions, the concerned frequency is not included in SIB21 but is indicated in the USD for this session; and NOTE 2: The UE considers a frequency to be part of the MBS frequencies of interest even though NG-RAN may (temporarily) not employ a broadcast MRB for the concerned session, i.e., the UE does not verify if the session is indicated on MCCH. 2> the supportedBandCombinationList the UE included in UE-NR-Capability contains at least one band combination including the concerned MBS frequency. NOTE 3: When evaluating which frequencies the UE is capable of receiving, the UE does not take into account whether they are currently configured as serving frequencies. #### 5.9.4.4 MBS services of interest determination The UE shall: 1> consider an MBS service to be part of the MBS services of interest if the following conditions are met: 2> the UE is receiving or interested to receive this service via a broadcast MRB; and 2> the session of this service is ongoing or about to start; and 2> one or more MBS FSAIs in the USD for this service is included in SIB21 acquired from the PCell for a frequency belonging to the set of MBS frequencies of interest, determined according to 5.9.4.3 or SIB21 acquired from the PCell does not provide the frequency mapping for the concerned service but that frequency is included in the USD for this service. NOTE: The UE may determine whether the session is ongoing or about to start from the start and stop time indicated in the User Service Description (USD), see TS 38.300 [2] or TS 23.247 [67].

# 3 Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: Adopt TP1 in stage-2 spec for the use of MII by the gNB to add/release DRBs/multicast MRBs.

Proposal 2: Adopt TP2 for NOTE in clause 5.9.4.3 and clause 5.9.4.4 in RRC spec and also clarify that UE may determine whether session is ‘about to start’ from the start time indicated in USD.