3GPP TSG-RAN WG2 #127-bis	R2-2408759

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.6.3

Source:	Ericsson

Title:	Event definition and MAC CE content for L1 event-triggered measurements

Document for:	Discussion

# 1	Introduction

With the agreements from RAN2#127 around how event based measurements are reported back to the network, there are some questions that need to be settled about the design of the events. This contribution discusses these issues and brings forth a number of proposals in this area.

# 2	Discussion

## 2.1	L1 events

The L1 event-triggered measurements objective is ongoing, and the following events have been agreed so far:

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

In the latest RAN2 meeting there was a discussion to support a new event, named event LTM1 which would trigger a L1 measurement report when the beam of serving cell becomes better than absolute threshold. However, RAN2 has not agreed on this event with the main justification that network may already some of the L3 A-events together with the new LTM events to figure out e.g., when to reconfigure the CSI measurement configuration or to release a LTM candidate cell.

The event LTM1 (the beam of serving cell becomes better than absolute threshold) has not been agreed with the motivation that network can combine the use of L3 and L1 events.

However, given that the L3 measurement report is sent to the gNB-CU while the L1 measurement report is sent to the gNB-DU, it would be quite complicated for the network to use both reports for deciding a configuration that is related to LTM, such as the one of the CSI measurement configurations. Also, there could be a case where network configures the UE to perform L1 measurement on an LTM candidate cell but not L3 measurements.

The use of L1 and L3 measurement reports to configure LTM-related configurations (e.g., update of the CSI measurement report) is complex for the network, as the L3 measurement report is sent to the gNB-CU while the L1 measurement report is sent to the gNB-DU.

According to this, we would like for RAN2 to reconsider the decision on not specifying the event LTM1, as this event bring evident benefits in the overall handling of LTM-related operations. Thus, we propose:

RAN2 to agree on supporting the event LTM1 (the beam of serving cell becomes better than absolute threshold).

Additional use cases that we believe should be considered for conditional LTM are the “Release LTM Candidate Configuration” and “Early Sync deactivation of LTM candidate cells”. Managing the LTM candidate cells is always a difficult task for the network, and the early synchronization procedure are probably the most important feature that we have for LTM.

Managing the LTM candidate cells is always a difficult task for the network, and the early synchronization procedure is probably the most important feature that we have for LTM.

In a scenario where UE capability on the number of activated TCI states is limited (e.g., initial versions of the LTM, UE may support limited number of TCI states like 1 or 2), it may happen that after the TCI state of the LTM candidate cell is pre-activated, the radio conditions may change before the UE receives an LTM cell switch command and, without the measurement report from event LTM6, NW may not know the radio conditions change and NW may indicate the activated TCI which may not be the best beam to use after cell switch. We think LTM6 is quite useful to keep the preactivated TCI states update to best beams in the candidate cells.

We also would like to highlight that number of LTM candidates UE can support is a UE capability. We cannot rule out a possibility that some UEs may support only one LTM candidate cell. In that scenario, it is important for the NW to maintain LTM candidate cell as per the radio conditions change. If the beam of the candidate cell becomes worse than absolute threshold, then NW may release the LTM candidate cell and add a new LTM candidate cell (report upon LTM7 event entering condition will help achieving this).

Therefore, we believe that is important to have events which can help the network to address these use cases in the most efficient way. According to this, we would like to propose:

RAN2 to consider how to release the LTM candidate configuration and the triggering or updating of the early synchronization procedures when specifying L1 events.

The following events are supported:

  - Event LTM6: Beam of candidate cell becomes offset better than pre-activated beam of same candidate cell

  - Event LTM7: Beam of candidate cell becomes worse than absolute threshold

## 2.2		Triggering for sending the measurement report

One open issue is whether a measurement report should be sent when the leaving condition is met. In L3 measurements, a UE sends a measurement report when the field reportOnLeave is configured and also when the leaving condition is met for a certain event. For L1, it would be good to align as this information is particularly useful for the network to understand which of the configured candidate cells is not good anymore. This may lead the network to release such candidate cell or to disable certain features such as the early UL or DL synchronization. Therefore, we propose:

The UE can be configured to send a L1 measurement report when the leaving condition is met for L1 event, similar to what is available for L3 measurements.

Another aspect which would be good to align with what we have for L3 is the sending of a measurement report periodically after an event criterion has been fulfilled. This is particularly useful for feature like LTM because it will guarantee the network to have very timely information from the UE before taking the decision on whether to trigger an LTM cell switch procedure or not. Thus, our proposal is:

The UE sends periodic L1 measurement reports after event-related measurement report has been sent. A report amount and report interval are specified, similar to what is available for L3 measurements.

## 2.3	Scheduling of L1 measurement report MAC CE

Once that RAN2 has agreed to use a MAC CE to report the event-triggered L1 measurements, it would be good to clarify how the UE receive the grant for transmitting such MAC CE. In general, the normal SR/BSR procedure can be used by the UE. However, it would be beneficial for the network to understand that the received SR is because an event-triggered L1 measurements MAC CE needs to be sent. This will allow the network to prioritize the scheduling of the grant and also to provide it in a faster way. According to this, we propose:

Using a special SR for requesting resource to send the event-triggered L1 measurements MAC CE would allow the network to prioritize the delivery of the grant and to schedule it in a faster way.

The legacy SR and BSR procedure are the baseline to send the event-triggered L1 measurements MAC CE.

Additionally, a special SR can be used for requesting resources to send the event-triggered L1 measurements MAC CE.

## 2.4	MAC CE design

### 2.4.1 Reporting Content

Now that RAN2 has agreed in the last meeting to use the MAC CE to report the L1 measurements to the network, something that needs to be addressed is what to include in the MAC CE. The first aspect that would be good to discuss is the number of SSBs for each LTM candidate configuration which is included in the report. In Rel-18, this is controlled by the network where a maximum of 4 SSBs per LTM candidate cell for a maximum of 4 LTM candidate cells can be include in the L1 report. In this sense, it would be good to align with what we have in Rel-18. Therefore, we propose:

The event-triggered L1 measurement report MAC CE can include up to 4 LTM candidate cells.

The event-triggered L1 measurement report MAC CE can include a maximum of 4 SSBs or CSI-RSs for each LTM candidate cell.

A further thing to discuss is that it must be clear to the network which event that triggered the reporting. It appears reasonable that all configured events are associated with an event ID and that the measurement report includes the ID of the triggering event.

Event triggered LTM measurement report should include an identifier of the event.

Another aspect to consider is that the MAC CE design should work for the case on when the reference signal that the UE measure is an SSB or a CSI-RS. According to this, since both the SSB ID space and the CSI-RS ID space have size 64 it would make sense to design the MAC CE to be agnostic to the reference signal that the UE measure. Therefore, we propose:

A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

Other information which can either be included in the MAC CE or configured at the UE, which are maybe straightforward, are i) the measurement quantity, and ii) an indication to report the serving cell. Therefore, according to this, we propose:

The MAC CE format for the event-triggered L1 measurement report includes for each reference signal reported the related measurement quantity (e.g., L1-RSRP).

The network can configure the UE to report the serving cell in the event-triggered L1 measurement report.

In Rel-18, the L1 report over UCI is basically include an index, which indicate to the network the i-th position of the lists which are included in the LTM-CSI-SSB-ResourceSet:

LTM-CSI-SSB-ResourceSet-r18 ::=     SEQUENCE {

ltm-CSI-SSB-ResourceList-r18        SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index,

ltm-CandidateIdList-r18             SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18,

...

}

By reporting this index, which is called SSBRI, the network knows the value of this index is the i-th position in the list ltm-CSI-SSB-ResourceList and the i-th position in the field ltm-CandidateIdList. At first, this solution may look likes some signalling is saved, as with one index we indicate two positions in two separate list, but when re-using this same mechanism for the design of a MAC CE, this means that 9 bits shall be always reserved for every reported measurement, as the maximum size of the lists ltm-CSI-SSB-ResourceList and ltm-CandidateIdList is 512.

Using the SSBRI in the event-triggered L1 measurement report MAC CE may not be efficient as a large number of bits are required.

As the MAC CE allows for more flexibility with respect to the UCI, it is possible to achieve the same results by including the reference signal ID and the LTM candidate configuration ID of the reported measurement quantity in an explicit way. Because of this, it would be good if RAN2 discusses whether the SSBRI is used for the event-triggered L1 measurement report MAC CE as this it may impact the MAC CE design. Thus, we propose:

RAN2 to discuss whether the event-triggered L1 measurement report MAC CE reports the SSBRI or instead the reference signal ID and the LTM candidate configuration ID explicitly.

### 2.4.2	Possible MAC CE format

#### 2.4.2.1	MAC CE Format without using the SSBRI

This first MAC CE format assumes that no SSBRI is used for reporting the reference signal and the LTM candidate configuration identifier. In this case, one octet is used to indicate which of the eight LTM candidate configurations are part in the report (L1 – L8). Further, one octet is used to indicate how many beams that are reported per candidate, for a maximum number of 4 beams per candidate configuration. Here it is assumed that we keep the number of reported LTM candidates to a maximum of four, meaning that maximum 4 candidates with maximum 4 beams each can be reported. This brings the maximum size for this MAC CE to 280 bits, excluding the header. The remaining fields of the MAC CE are as follows:

- Event ID: The event ID which triggered the report

- Li: Corresponds to the i-th LTM candidate ID. If the field is set to 1 it means that the i-th LTM candidate ID is part of the report.

- Ni: Indicated the number of reference signals which are reported of the i-th LTM candidate ID indicated by the field Lj. The i-th LTM candidate ID is considered in the order of appearance (from right to left) of the field Ni.

- RS IDi,j: Corresponds to the j-th reference signal (SSB or CSI-RS) of the i-th LTM candidate ID.

- RSRPi,j: Correspond to the RSRP value measured on the j-th reference signal of the i-th LTM candidate ID.

- F: If the value is set to 1 this means that the RS IDi,j is the measured reference signal which has fulfilled the event criteria.

Figure 1. MAC CE format without using the SSBRI.

#### 2.4.2.2	MAC CE Format using the SSBRI

For this second option, instead of LTM Candidate ID and SSB index we are using SSBRI, which is already used to identify to identify LTM Candidate SSBs as per Rel-18 UCI report format. An SSBRI uniquely identifies both an LTM candidate cell and SSB ID in an LTM CSI resource configuration. SSBRI is encoded with log2(KSSSB) bits (see TS 38.212 Table 6.3.1.1.2-6), where KSSSB is the configured number of SS/PBCH blocks in the corresponding LTM-CSI-Resource configuration. This means that SSBRI can be up to 9 bits to indicate a list index of up to 512 (8 cells with 64 beams each).

One octets are used to indicate whether or not each reported SSBRI/RSRP pair triggered the report or not. Also, for this option four candidates with four beams each is considered, bringing the total size of the MAC CE to a maximum of 264 bits. However, this MAC CE would be difficult to extend (i.e., more octets need to be added) in case more information are needed to be reported (e.g., the cell which met the leaving condition or which of the reported LTM candidate cells is the serving cell). The remaining fields of the MAC CE are as follows:

- Event ID: The event ID which triggered the report

- Triggered quantity: Indicate the RSRI value of the reference signal (SSB or CSI-RS) which triggered the measurement report.

- RSRIi: Corresponds to the i-th SSBRI or CRI index according to received LTM CSI resource configuration.

- RSRPi: Correspond to the measured RSRP value of the i-th SSBRI or CRI index according to received LTM CSI resource configuration.

Figure 2. MAC CE format using the SSBRI

Since RAN2 has still to analysis which type of MAC CE format is more convenient for reporting the L1 event-triggered measurements, our propose would be for RAN2 to take the proposed MAC CE format as baseline for the time being.

RAN2 to consider the MAC CE formats in Figure 1 and Figure 2 as the baseline for the discussions on the event-triggered L1 measurement report MAC CE.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The event LTM1 (the beam of serving cell becomes better than absolute threshold) has not been agreed with the motivation that network can combine the use of L3 and L1 events.

Observation 2	The use of L1 and L3 measurement reports to configure LTM-related configurations (e.g., update of the CSI measurement report) is complex for the network, as the L3 measurement report is sent to the gNB-CU while the L1 measurement report is sent to the gNB-DU.

Observation 3	Managing the LTM candidate cells is always a difficult task for the network, and the early synchronization procedure is probably the most important feature that we have for LTM.

Observation 4	Using a special SR for requesting resource to send the event-triggered L1 measurements MAC CE would allow the network to prioritize the delivery of the grant and to schedule it in a faster way.

Observation 5	Using the SSBRI in the event-triggered L1 measurement report MAC CE may not be efficient as a large number of bits are required.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to agree on supporting the event LTM1 (the beam of serving cell becomes better than absolute threshold).

Proposal 2	RAN2 to consider how to release the LTM candidate configuration and the triggering or updating of the early synchronization procedures when specifying L1 events.

Proposal 3	The following events are supported:

a.	Event LTM6: Beam of candidate cell becomes offset better than pre-activated beam of same candidate cell

b.	Event LTM7: Beam of candidate cell becomes worse than absolute threshold

Proposal 4	The UE can be configured to send a L1 measurement report when the leaving condition is met for L1 event, similar to what is available for L3 measurements.

Proposal 5	The UE sends periodic L1 measurement reports after event-related measurement report has been sent. A report amount and report interval are specified, similar to what is available for L3 measurements.

Proposal 6	The legacy SR and BSR procedure are the baseline to send the event-triggered L1 measurements MAC CE.

Proposal 7	Additionally, a special SR can be used for requesting resources to send the event-triggered L1 measurements MAC CE.

Proposal 8	The event-triggered L1 measurement report MAC CE can include up to 4 LTM candidate cells.

Proposal 9	The event-triggered L1 measurement report MAC CE can include a maximum of 4 SSBs or CSI-RSs for each LTM candidate cell.

Proposal 10	Event triggered LTM measurement report should include an identifier of the event.

Proposal 11	A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

Proposal 12	The MAC CE format for the event-triggered L1 measurement report includes for each reference signal reported the related measurement quantity (e.g., L1-RSRP).

Proposal 14	RAN2 to discuss whether the event-triggered L1 measurement report MAC CE reports the SSBRI or instead the reference signal ID and the LTM candidate configuration ID explicitly.

Proposal 15	RAN2 to consider the MAC CE formats in Figure 1 and Figure 2 as the baseline for the discussions on the event-triggered L1 measurement report MAC CE.

# 4	References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #103, Maastricht, Netherlands, March 18-21, 2024.