3GPP TSG-RAN WG2 #127-bis	R2-2408760

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.6.4

Source:	Ericsson

Title:	Baseline assumptions for conditional LTM

Document for:	Discussion

# 1	Introduction

After the RAN plenary in September, the following objective has been agreed in the WID in [1] for conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution we will address several aspects which are related to the objective of conditional LTM.

# 2	Discussion

## 2.1	Number of conditional LTM candidate configurations

Conditional LTM is not a new feature but is seen as an enhancement of the LTM feature specified in Rel-18. According to this, it would be useful to clarify since the beginning that RAN2 does not need to specify a feature from scratch with conditional LTM, but it should rather use as much as possible what has been done for LTM. Thus:

LTM specified in Rel-18 is the baseline for conditional LTM.

No security key change is supported with conditional Intra-CU LTM.

A further aspect to address about Conditional LTM (CLTM) is how many candidate configurations can be configured by the network. Here there are basically two options:

- Option 1:  A maximum of 8 conditional and normal LTM candidate configurations can be configured by the network. This means that the 8 LTM candidate configurations which have been specified in Rel-18 is the total number of both normal and conditional LTM candidate configurations.

- Option 2: A maximum of 8 conditional LTM candidate configurations can be configured by the network, which are independent from the 8 LTM candidate configurations.

Either option work but of course both have pro and cons. For instance, with Option 1 there is less flexibility in configuring LTM and CLTM as the number of candidate configuration will be limited. On the other side, Option 2 give more flexibility in configuring LTM and CLTM but it would require some effort (which could be anyway quite limited) in adapting the existing procedure that have been specified for LTM since Rel-18. Either way, it would be good for RAN2 to discuss this aspect first, as it will make the direction of the standardization process for CLTM much easier. Thus, we propose:

RAN2 to discuss the maximum number of conditional LTM candidate configurations that network can configure.

A maximum of 8 conditional LTM candidate configurations (plus 8 non-conditional).

A maximum of 8 conditional and normal LTM candidate configurations in total.

## 2.2	Measurements aspects for conditional LTM

The baseline LTM since Rel-18 are based according to the configuration and use of L1 measurements. While the L1 measurements have been enhanced in Rel-18, this trend has continued in Rel-19 where the L1 event-triggered measurement reporting are one of the objectives of the mobility WI. According to this, it would be good to clarify that, as for the normal LTM, also conditional LTM will be based on L1 measurements, and in particular use the events specified for the L1 event-triggered measurement reporting objective as baseline. Therefore, we propose:

Conditional LTM is based on L1 measurements (same as LTM in Rel-18) and the events introduced for L1 event-triggered measurement reporting as baseline.

## 2.3	Conditions for triggering conditional LTM

The L1 event-triggered measurements objective is ongoing, and the following events have been agreed so far:

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

The main use for specifying triggering condition for CLTM is for the UE to execute a conditional LTM cell switch procedure. According to this, as a baseline the events LTM3 and LTM5 seems to be the most suitable to be used for CLTM. Thus, we propose:

The events LTM3 and LTM5 are the baseline events for the UE to trigger a conditional LTM cell switch.

## 2.4	Early synchronization procedures for conditional LTM

Another aspect that is quite clear from the WID in [1] is that only the conditional LTM cell switch will be UE-triggered, while for the early synchronization procedures what has been specified in Rel-18 is the baseline, and thus such procedures will remain NW-triggered. It is important to support early synchronization also for conditional LTM as this allows the conditional LTM cell switch to be RACH-less, thus combining high robustness with shorter interruption.

It is important to support early synchronization also for conditional LTM as this allows the conditional LTM cell switch to be RACH-less, thus combining robustness with shorter interruption.

Nevertheless, given the limited amount of time that RAN2 has to complete the whole WI, now that RAN2 needs to work on the three objectives, reusing what has been done in Rel-18 sounds like the easiest and more straightforward option. Therefore, we propose:

The early UL and DL synchronization procedures for conditional LTM are only triggered by the network.

The early UL and DL synchronization procedures standardized in Rel-18 are the baseline for conditional LTM.

However, the procedures standardized for early synchronization cannot be re-used as they are, at least for the early UL synchronization procedure. In this case, the TA in Rel-18 is delivered via the LTM cell switch MAC CE, but in case of conditional LTM there is no LTM cell switch MAC CE, as is the UE itself that decide to trigger a conditional LTM cell switch procedure.

In conditional LTM, the TA value cannot be delivered via the LTM cell switch MAC CE.

Therefore, alternatives ways for delivering the TA value are needed. One thing that can be confirmed is that the random access procedure so that the target LTM candidate cell can estimate the TA value is triggered by a PDCCH sent from the serving cell.

The random access procedure to initiate the TA acquisition in conditional LTM is triggered by a PDCCH order sent by the serving DU.

Now the question is how to deliver the TA to the UE. According to this, in order to mimic the behaviour we have in Rel-18, a straightforward solution would be for the TA to be delivered still by the serving DU. Nevertheless, since there is no LTM cell switch MAC CE in case of conditional LTM, this TA should be delivered in a new MAC CE (or eventually it could be checked if the legacy Absolute TA MAC CE can be re-used). This will also make the work much easier for RAN3, since the signalling over F1 that has been specified in Rel-18 can be completely re-used. According to this, we propose:

In case of conditional LTM, the TA value for the early UL synchronization procedure is delivered by the serving DU via a MAC CE (FFS whether a new one or an existing one).

Finally, a last topic to be addressed is whether the UE-based TA measurements is supported for conditional LTM. In Rel-18 this feature is supported for intra-CU LTM and only in case the source and target cell are synchronized. We see no problem to support this feature also for conditional LTM, as the specification impact in this case would be very limited. Thus we propose:

The UE-based TA measurement feature is supported for conditional LTM.

## 2.5	Conditional LTM and DC scenarios

In the last RAN plenary, it was discussed whether to support conditional LTM in DC scenarios and, if this was the case, in which particular scenarios. If we go back to Rel-18, when LTM was specified RAN2 supported the coexistence of LTM with DC but only according to some restrictions. In fact, LTM with DC was supported only according to the following:

- Only intra-CU LTM is supported at the MN and SN

- LTM is configured independently at the MN and SN

- In case, of LTM at the MN, the SN can either be kept or released

- PCell change with PScell change with LTM is not possible

- No security change is supported with LTM

These are the principles that we took into account in Rel-18 and, since the scenarios which are going to be supported for conditional LTM are basically the same what we have done for the normal LTM, a straightforward way to support conditional LTM in DC scenarios would be to re-use the following principles. According to this, we propose:

Conditional LTM is supported with DC according to the following principles (same as in Rel-18 for intra-CU LTM):
a) Only intra-CU conditional LTM is supported at the MN and SN
b) Conditional LTM is configured independently at the MN and SN
c) In case of a conditional LTM cell switch at the MN, the SN can either be kept or released
d) PCell change with PSCell change is not supported with conditional LTM
e) No security change is supported with conditional LTM

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	It is important to support early synchronization also for conditional LTM as this allows the conditional LTM cell switch to be RACH-less, thus combining robustness with shorter interruption.

Observation 2	In conditional LTM, the TA value cannot be delivered via the LTM cell switch MAC CE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	LTM specified in Rel-18 is the baseline for conditional LTM.

Proposal 2	No security key change is supported with conditional Intra-CU LTM.

Proposal 3	RAN2 to discuss the maximum number of conditional LTM candidate configurations that network can configure.

Proposal 4	A maximum of 8 conditional LTM candidate configurations (plus 8 non-conditional).

Proposal 5	A maximum of 8 conditional and normal LTM candidate configurations in total.

Proposal 6	Conditional LTM is based on L1 measurements (same as LTM in Rel-18) and the events introduced for L1 event-triggered measurement reporting as baseline.

Proposal 7	The events LTM3 and LTM5 are the baseline events for the UE to trigger a conditional LTM cell switch.

Proposal 8	The early UL and DL synchronization procedures for conditional LTM are only triggered by the network.

Proposal 9	The early UL and DL synchronization procedures standardized in Rel-18 are the baseline for conditional LTM.

Proposal 10	The random access procedure to initiate the TA acquisition in conditional LTM is triggered by a PDCCH order sent by the serving DU.

Proposal 11	In case of conditional LTM, the TA value for the early UL synchronization procedure is delivered by the serving DU via a MAC CE (FFS whether a new one or an existing one).

Proposal 12	The UE-based TA measurement feature is supported for conditional LTM.

Proposal 13	Conditional LTM is supported with DC according to the following principles (same as in Rel-18 for intra-CU LTM): a) Only intra-CU conditional LTM is supported at the MN and SN b) Conditional LTM is configured independently at the MN and SN c) In case of a conditional LTM cell switch at the MN, the SN can either be kept or released d) PCell change with PSCell change is not supported with conditional LTM e) No security change is supported with conditional LTM

# 4	References

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.