3GPP TSG-RAN WG2 #127-bis	R2-2408762

# Hefei, China, 14th – 18th October 2024

Agenda Item:	8.0

Source:	Ericsson

Title:	Regarding LS from SA5 on Number of UEs in RRC_INACTIVE state with data transmission

Document for:	Discussion

# 1	Introduction

An incoming LS from SA5 was received at RAN2 [1] which tries to get information from RAN2 about feedback for “the number of SDT UEs in RRC_INACTIVE state”. This contribution shares our thoughts on this matter.

# 2	Discussion

The LS from SA5 formulates the following question:

# 1 Overall description The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission. # 2 Actions To RAN2 ACTION: SA5 kindly asks RAN2 to provide feedbacks.

It seems from this request for feedback that there is no clear question to be answered, rather SA5 asks for feedback around the stated information in the “Overall description” chapter. Let’s break down the provided information piece by piece and provide our view on each of these:

Statement 1 in LS: “SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell.”

It seems reasonable that the number of UEs in a specific cell in state RRC_INACTIVE would be useful information to the operator. However, a UE in RRC_INACTIVE mode is not known to the network on cell level. The network might have transferred a UE to RRC_INACTIVE and the UE from that moment could have moved into another cell without informing the network.

The network does not know which UEs that are in RRC_INACTIVE in a specific cell.

Statement 2 in LS: “SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.”

A UE that is capable of using SDT can do this for Mobile Originated and/or Mobile Terminated traffic, either via RA-SDT and/or CG-SDT. With any of these procedures the UE might be actively transmitting and/or receiving data, which might be relevant since statement 2 mentions “with data transmission”. The following concepts are described for SDT:

- The SDT procedure is considered to be ongoing once the conditions for initiating SDT as specified in clause 5.3.13.1b in TS 3GPP 38.331are fulfilled until the SDT procedure is completed either successfully or unsuccessfully.

- Otherwise, the SDT procedure is considered not ongoing.

It is not totally clear if SA5 means to specify the number of UEs with SDT procedure ongoing or not ongoing, but for the ongoing case this would be a very short duration of time (the SDT procedure is regulated by timer T319a which is maximum 4 seconds), so the assumption here is that SA5 asks about UEs with SDT procedure not ongoing.

Out of the possible scenarios at the UE side, it is not totally clear from the LS what cases SA5 would like to consider but one assumption is that SA5 are interested in UEs capable of SDT with SDT procedure considered as not ongoing.

When the SDT procedure is not considered ongoing, an SDT capable UE in RRC_INACTIVE has an SDT configuration, which might include any of the described cases (MO/MT and RA-SDT/CG-SDT). The number of configurations would be quite easy to extract on a gNB level, but it should be pointed out, that while this number would be known on cell level, there is no guarantee that the UE remains in that cell, only the RAN notification area.

This request for evaluation in statement 2 does not consider cell level, but it can be assumed from statement 1 in the LS that the question regards to cell level knowledge of a UE configured for SDT as well as transmitting data.

If this interpretation is correct, we would like to point out that even if the gNB would know how many UEs that was sent to RRC_INACTIVE with SDT configuration, this would not answer the question on the current number since the UE might move out of the cell area without notifying the network.

An alternative interpretation of Statement 2 above would be that SA5 asks about UEs that has SDT procedure ongoing. For this case the usefulness could also be questioned since the SDT procedure runs for a short time, up to 4 seconds from the point the UE sends RRCResumeRequest. A number that indicates UEs in this state would be short lived and it is not expected to indicate cell load.

With the above information considered the usefulness of such a counter could be questioned, given that UEs could move out of the cell area without notifying the network.

These questions seem to also concern RAN3.

RAN2 to send a reply-LS to SA5 with CC to RAN3; stating; “RAN2 think it would be possible to specify for the RAN to indicate the number of users which are sent to RRC_INACTIVE with SDT configuration for a specific cell but would like to point out that UEs might not remain in the cell, and thus it is not expected as a good indication of cell load.”

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The network does not know which UEs that are in RRC_INACTIVE in a specific cell.

Observation 2	Out of the possible scenarios at the UE side, it is not totally clear from the LS what cases SA5 would like to consider but one assumption is that SA5 are interested in UEs capable of SDT with SDT procedure considered as not ongoing.

Observation 3	With the above information considered the usefulness of such a counter could be questioned, given that UEs could move out of the cell area without notifying the network.

Observation 4	These questions seem to also concern RAN3.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to send a reply-LS to SA5 with CC to RAN3; stating; “RAN2 think it would be possible to specify for the RAN to indicate the number of users which are sent to RRC_INACTIVE with SDT configuration for a specific cell but would like to point out that UEs might not remain in the cell, and thus it is not expected as a good indication of cell load.”

# 4	References

- S5-245138, LS on Number of UEs in RRC_INACTIVE state with data transmission