3GPP TSG-RAN WG2 Meeting #127bis	R2-2408765

# Hefei, China, Oct 14th – 18th, 2024

Source:                Qualcomm Incorporated

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# Discussion

## 2.1 Serving cell measurement relaxation

### 2.1.1 Serving cell measurement relaxation with offloading to LP-WUS

RAN2 made agreements in the last meeting that MR and LR measurement can be considered for serving cell measurement relaxation with offloading to LP-WUS as follows:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

However, the UE behavior is still unclear on how to consider the MR/LR measurement when determining whether to offload to LP-WUS. UE can be configured a threshold to determine whether to offload measurement to LR. Considering MR and LR may have different coverage, it is more safe that UE starts offloading measurements to LR. That means when MR measurement is above a threshold and the LR measurement is above a threshold, UE starts offloading measurements to LR. There could be case that MR measurement is above the threshold but the LR measurement is below a threshold, it can be left to UE implementation to determine when to start LR measurement.

- For MR serving cell offloading to LR, the entry conditions for serving cell measurement offloading UE determines the offloading start and stop as following:

- When the MR measurement is above a threshold and the LR measurement is above a threshold, UE starts totally offloading measurement to LR (i.e, stop MR measurements). It is up to UE implementation to determine when to start LR measurement.

- When the LR measurement is below a threshold, UE switches to MR measurements.

- When the corresponding thresholds are not configured, it is up to UE implementation to determine the conditions.

When the serving cell measurement is offloaded to the LP-WUS, it makes no sense to monitor paging in MR again. So UE should also switch to LP-WUS triggered paging monitoring.

- RAN2 assumes when serving cell measurement is offloaded to LP-WUS, paging monitoring is also switched to LP-WUS.

## 2.2	Neighbor cell measurement relaxations

Normal intra-frequency measurement trigger

Currently, for intra-frequency measurement, in order to save UE power, a serving cell quality threshold (SIntraSearchP/SIntraSearchQ) are introduced for trigger intra-frequency measurement. When the UE offloads measurement to LP-WUS, it still makes sense to define a threshold to trigger intra-frequency measurement, i.e. only when LP-WUS measurement is below the threshold UE starts intra-frequency measurement.

- For intra-frequency measurement, introduce LP-WUS measurement threshold (e.g. similar with SIntraSearchP/SIntraSearchQ) for starting intra-frequency measurement.

Intra-freqeuncy/Inter-frequency/Inter-RAT measurement relaxation

For measurement relaxation, there are current measurement relaxation mechanism defined from Rel-16, and RAN2 had achieved the following agreements for neighboring cell measurement relaxation

RAN2 will further discuss the neighbouring cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In current clause 5.2.4.9.2 of TS38.304, the relaxed measurement criterion for UE not at cell edge is fulfilled when:

-	Srxlev > SSearchThresholdP, and,

-	Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	Squal = current Squal value of the serving cell (dB).

Two parameters SSearchThresholdP, SSearchThresholdQ are introduced for ‘not at cell edge’ determination, Considering UE is using LR to measure serving cell, then LR specific parameters need to be introduced to determine the relaxed measurement criterion

- For neighbouring cell measurement relaxation, introduce LP-WUS measurement parameters (e.g. silimiar with SSearchThresholdP, SSearchThresholdQ) for determining ‘not at cell edge’.

In current clause 5.2.4.9.1 of TS38.304, the relaxed measurement criterion for UE with low mobility is fulfilled when:

-	(SrxlevRef – Srxlev) < SSearchDeltaP,

Where:

-	Srxlev = current Srxlev value of the serving cell (dB).

-	SrxlevRef = reference Srxlev value of the serving cell (dB), set as follows:

-	After selecting or reselecting a new cell, or

-	If (Srxlev - SrxlevRef) > 0, or

-	If the relaxed measurement criterion has not been met for TSearchDeltaP:

-	The UE shall set the value of SrxlevRef to the current Srxlev value of the serving cell.

For relaxed measurement criterion with low mobility, two parameters (SSearchDeltaP/SrxlevRef) are introduced for MR. When the UE is using LR to measure serving cell, LR specific parameters need to be introduced to determine the relaxed measurement criterion.

- For neighbouring cell measurement relaxation, introduce LP-WUS measurement parameters (e.g. silimiar with SSearchDeltaP/SrxlevRef) for determining ‘not at cell edge’.

Additionally, for inter-frequency/Inter-RAT frequency measurement, additional parameters SnonIntraSearchP SnonIntraSearchQ are introduced for inter-frequency/Inter-RAT measurement relaxation. For LR measurement, it also needs to introduce similar parameters for UE to determine whether to perform relaxed neighbouring cell measurements

- For inter-frequency/Inter-RAT frequency measurement relaxation, introduce LP-WUS measurement parameters (e.g. silimiar with SnonIntraSearchP/SnonIntraSearchQ) for determining whether to perform relaxed neighbouring cell measurements.

# Conclusion

- For MR serving cell offloading to LR, the entry conditions for serving cell measurement offloading UE determines the offloading start and stop as following:

- When the MR measurement is above a threshold and the LR measurement is above a threshold, UE starts totally offloading measurement to LR (i.e, stop MR measurements). It is up to UE implementation to determine when to start LR measurement.

- When the LR measurement is below a threshold, UE switches to MR measurements.

- When the corresponding thresholds are not configured, it is up to UE implementation to determine the conditions.

- RAN2 assumes when serving cell measurement is offloaded to LP-WUS, paging monitoring is also switched to LP-WUS.

- For intra-frequency measurement, introduce LP-WUS measurement threshold (e.g. similar with SIntraSearchP/SIntraSearchQ) for starting intra-frequency measurement.

- For neighbouring cell measurement relaxation, introduce LP-WUS measurement parameters (e.g. silimiar with SSearchThresholdP, SSearchThresholdQ) for determining ‘not at cell edge’.

- For neighbouring cell measurement relaxation, introduce LP-WUS measurement parameters (e.g. silimiar with SSearchDeltaP/SrxlevRef) for determining ‘not at cell edge’.

- For inter-frequency/Inter-RAT frequency measurement relaxation, introduce LP-WUS measurement parameters (e.g. silimiar with SnonIntraSearchP/SnonIntraSearchQ) for determining whether to perform relaxed neighbouring cell measurements.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)