3GPP TSG-RAN WG2 Meeting #127bis			         R2-2408769

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.4.3

Source: 	LG Electronics Inc.

Title:         	RRM relaxation and RRM offloading

Document for: 	Discussion and Decision

Introduction

In this contribution we will discuss the RRM relaxation and offloading based on the agreements so far:

RAN2#126

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

- RAN2#127

RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs.

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

Discussion

- Neighbor cell measurement relaxation

In R16 measurement relaxation, two relaxed measurement criteria for neighbor cell were introduced as follows, and UE performs measurements on neighbor cells using relaxed measurement requirement, if at least one of criteria is met.

- Relaxed measurement criterion for UE with low mobility - (SrxlevRef – Srxlev) < SSearchDeltaP, Where: - Srxlev = current Srxlev value of the serving cell (dB). - SrxlevRef = reference Srxlev value of the serving cell (dB), - Relaxed measurement criterion for UE not at cell edge - Srxlev > SSearchThresholdP, and, - Squal > SSearchThresholdQ, if SSearchThresholdQ is configured,

For R19 relaxed measurement on neighbor cell for UEs capable of LP-WUS, following three options can be considered:

- Option 1. Defining a new criterion

- Option 2. Defining a new measurement requirement, i.e. further relaxed measurement requirement

- Option 3. Enhancing the existing criteria so that they can be evaluated based on the LR measurements

The first option is defining an additional criterion beyond the R16 criteria, allowing UE to perform measurements using the existing relaxed requirement defined by RAN4 in R16. However, since criteria for measurement relaxation were thoroughly discussed in R16, it seems very challenging to define an additional criterion that really help with further relaxed measurement.

Alternatively, an additional threshold can be introduced in the existing criteria, which is associated with further relaxed measurement requirement. That is, if the existing criterion with the additional threshold is met, UE is allowed to apply further relaxed measurement requirements. However, to achieve this, RAN4 should specify the further relaxed measurement requirements for intra-frequency/inter-frequency/inter-RAT cells. This is out of RAN2 scope.

Option 3 is to reuse the relaxation criteria and the relaxed measurement requirement defined in R16. The R16 relaxation criteria are based on the MR measurement of serving cell. If the serving cell measurement is fully offloaded to LR, the UE has no MR measurements for serving cell and cannot evaluate the R16 relaxation criteria. If R16 criteria are modified so that UE can evaluate them based on the LR measurements, the UE can relax the neighbor cell measurements even when the serving cell measurement is fully offloaded to LR. In general, the condition for serving cell measurement offloading to LR will be more stringent than that for relaxing neighbor cell measurement. That is, if the condition for serving cell measurement offloading to LR is met, the condition for neighbor cell measurement relaxation is also likely to be met. Therefore, the UE should be able to evaluate the relaxed measurement criteria for neighbor cell based on the LR measurements for serving cell.

The neighbor cell measurements already can be relaxed sufficiently by R16 relaxed measurement. The option 3 seems the best option RAN2 can take for further measurement relaxation related to LP-WUS/LR. Therefore, we propose followings:

Proposal 1	For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional criterion over the R16 criteria, i.e. ‘UE with low mobility’ and ‘UE not at cell edge’.

Proposal 2	For neighbor cell measurement relaxation for UEs capable of LP-WUS, enhance the R16 criteria, i.e. ‘UE with low mobility’ and ‘UE not at cell edge’, to enable UE to evaluate them based on the LR measurements when the serving cell measurement is offloaded to LR.

- Serving cell measurement offloading

In RRC_IDLE/INACTIVE state, the UE periodically performs serving cell and neighbour cell measurements for cell reselection according to measurement rules. The measurement interval for serving cell varies depending on the SMTC periodicity, DRX cycle, and UE’s power class. When the interval is shortest, the serving cell measurement should be performed at least once every 2.56 seconds. In contrast, the minimum measurement period for neighbour cells on the intra-frequency is 10.24 seconds, and for inter-frequency neighbour cells, it is 15.36 seconds. Therefore, if serving cell measurements are offloaded to LR, MR can maintain ultra-deep sleep for at least 10 seconds.

The target cell that the UE needs to measure depends on the quality of the serving cell, and the measurement cycle varies according to the frequency priority of the target cells. Hence, the duration that MR can sleep without neighbour cell measurements varies based on the quality of the serving cell.

Table 1 Expected power saving gain and impact of measurement offloading on cell reselection depending on the serving cell quality

| Serving cell quality | Measurement target | Maximum MR sleep duration | cell-ranking criterion Rs |
| --- | --- | --- | --- |
| If Srxlev > SIntraSearchP and Squal > SIntraSearchQ | Serving cell + Higher priority cells | 60 seconds | Does not need to be modified. |
| If Srxlev < SIntraSearchP or Squal < SIntraSearchQ | Serving cell + Higher priority cells + intra-frequency neighbour cell (+ inter-frequency with equal or lower priority) | 10.24~30.72 seconds | Needs to be modified to support LR measurement cell-ranking |

Even if the quality of the serving cell is excellent (i.e., if the serving cell fulfills Srxlev > SIntraSearchP and Squal > SIntraSearchQ), MR should periodically measure higher priority cells. However, once a higher priority cell has been measured but not reselected, the UE does not need to measure that cell for 60 seconds thereafter. Therefore, when serving cell quality is good enough to meet the condition for skipping intra-frequency measurement, a great power saving can be achieved through the serving cell measurement offloading to LR.

In the cell reselection to a higher priority frequency, the UE selects the highest ranked cell on the highest priority frequency by comparing the cell-ranking of higher priority cells measured by MR. Therefore, UE is not required to perform ranking using LR measurements though the serving cell measurement is offloaded to LR.

Observation 1	If the serving cell measurement is offloaded to LR when the condition for skipping intra-frequency measurement is met,

- a great power saving can be achieved by allowing MR to sleep for 60 seconds, and

- it does not affect the cell-ranking criterion Rs.

If the serving cell does not fulfil Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE should perform measurements on identified cells on the intra-frequency using MR every 10.24 to 30.72 seconds. Consequently, MR needs to wake up much more frequently than when UE is not required to perform intra-frequency measurements, and the UE power saving gain achievable through the serving measurement offloading to LR is significantly reduced.

The intra-frequency and equal priority inter-frequency cell reselection is performed based on comparing the ranking of serving cell and neighbour cells on intra-/equal priority inter-frequency. This means the UE needs to calculate ranking of serving cell using LR measurements and compare it with MR measurements based ranking of neighbour cells to select the highest ranked cell among them. If the target cell for reselection is selected based on the relatively inaccurate LR measurement results, the UE may perform incorrect cell reselection.

Observation 2	If the serving cell measurement is offloaded to LR when the condition for skipping intra-frequency measurement is not met,

- the power saving gain is significantly reduced, compared to the case when the condition for skipping intra-frequency measurement is met, and

- the highest ranked cell is selected based on LR measurements and it may lead to incorrect cell reselection.

The mobility performance should not be compromised for power saving, and the mobility decision should be made based on reliable MR measurements. Therefore, when UE initiates (or before initiating) intra-frequency measurements, the serving cell measurement offloading to LR should be halted, and MR should measure serving cell.

Proposal 3	The serving cell measurement is allowed to be offloaded to LR, only when the condition for skipping intra-frequency measurement is met.

Proposal 4	Do not support cell-ranking based on LR measurements.

Conclusion

Regarding the neighbor cell measurement relaxation

Proposal 1	For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional criterion over the R16 criteria, i.e. ‘UE with low mobility’ and ‘UE not at cell edge’.

Proposal 2	For neighbor cell measurement relaxation for UEs capable of LP-WUS, enhance the R16 criteria, i.e. ‘UE with low mobility’ and ‘UE not at cell edge’, to enable UE to evaluate them based on the LR measurements when the serving cell measurement is offloaded to LR.

Regarding the serving cell measurement offloading

Observation 1	If the serving cell measurement is offloaded to LR when the condition for skipping intra-frequency measurement is met,

- a great power saving can be achieved by allowing MR to sleep for 60 seconds, and

- it does not affect the cell-ranking criterion Rs.

Observation 2	If the serving cell measurement is offloaded to LR when the condition for skipping intra-frequency measurement is not met,

- the power saving gain is significantly reduced, compared to the case when the condition for skipping intra-frequency measurement is met, and

- the highest ranked cell is selected based on LR measurements and it may lead to incorrect cell reselection.

Proposal 3	The serving cell measurement is allowed to be offloaded to LR, only when the condition for skipping intra-frequency measurement is met.

Proposal 4	Do not support cell-ranking based on LR measurements.