3GPP TSG-RAN WG2 Meeting #127bis			         R2-2408771

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.3

Source: 	LG Electronics Inc.

Title:         	On-demand transmission of SIB1

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss all remaining issues to complete the study on on-demand SIB1, such as need of WUS configuration acquisition from NES cell, need of SIB1 validity, and whether/how to support case 3 defined in RAN1.

Discussion

- OD-SIB1 of neighbor NES cell (Case 2)

To maximize the NES gain through the on-demand SIB1 transmission, RAN2 agreed that the UE can request SIB1 only for NES cell that meets the cell reselection criteria. Based on this baseline agreed during SI phase, detailed triggering conditions for WUS transmission need to be discussed and decided for normative work.

RAN2#127

6. If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

The cell reselection criteria consist of several conditions and rules, and depend on the frequency priority of the neighbor cell. The cell reselection criteria for a cell of a higher priority are as follows:

For a cell of a higher priority, the cell reselection criteria are: - If more than 1 second has elapsed since the UE camped on the current serving cell, and - If threshServingLowQ is broadcast in system information, and if a cell of a higher priority frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT, or - If threshServingLowQ is not broadcast in system information, and if a cell of a higher priority frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT,

The cell reselection criteria include the 1-second rule to prevent UE from performing frequent cell reselection. The cell reselection criteria are reused as they are for the condition of the SIB1 request, UE can request SIB1 of NES Cell starting from 1 second after it has camped on the current cell. However, if so, the cell reselection to the NES Cell will be delayed by the sum of the time taken for MSG1 transmission, MSG2 reception, and the time for SIB1 acquisition, compared to the cell reselection to legacy cell. Even if SIB1 of an NES Cell is acquired within 1 second of the last cell reselection, the UE will not perform cell reselection to the NES Cell until 1 second has passed since the last cell reselection, according to the exiting 1-second rule in the cell reselection criteria. Therefore, UE should be able to transmit UL WUS to the NES Cell, although 1 second has not elapsed since the UE camped on the current serving cell.

Observation 1	If the 1-second rule in the cell reselection criteria is applied to the triggering condition of UL WUS transmission, it will lead to unnecessary delays in the cell reselection to OD-SIB1 cell.

Therefore, we propose to define the triggering condition of UL WUS transmission for OD-SIB1 cell of a higher priority as follows:

Proposal 1	For an OD-SIB1 cell of a higher priority UE requests SIB1, if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT, or

- If threshServingLowQ is not broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

For a cell of a lower priority, the cell reselection criteria are: - If more than 1 second has elapsed since the UE camped on the current serving cell, and - If threshServingLowQ is broadcast in system information, and if The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT, or - If threshServingLowQ is broadcast in system information, and if The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

The cell reselection criteria for a cell of a lower priority also includes the 1-second rule, and the same conclusion on it can be applied to the triggering condition of WUS transmission for OD-SIB1 cell of a lower priority.

Proposal 2	For an OD-SIB1 cell of a lower priority, UE request SIB1 if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if the serving cell fulfils Squal < ThreshServing, LowQ and an OD-SIB1 cell of a lower priority frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

- If threshServingLowQ is not broadcast in system information, and if the serving cell fulfils Srxlev < ThreshServing, LowP and an OD-SIB1 cell of a lower priority frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

In addition to the inter-frequency cell reselection criteria, there are two rules for inter-frequency cell reselection:

- Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria. - If more than one cell meets the above criteria, the UE shall reselect a cell as follows: - If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6

Above two rules also need to be applied to the triggering condition of WUS transmission to minimize the awakening of the OD-SIB1 cell.

Proposal 3	SIB1 request to a higher priority frequency shall take precedence over a lower priority frequency if multiple OD-SIB1 cells of different priorities fulfil the cell reselection criteria.

Proposal 4	If more than one OD-SIB1 cell meets the triggering condition of WUS transmission, the UE requests SIB1 of the highest ranked OD-SIB1 cell.

The cell reselection to a cell on an intra-frequency or equal priority inter-frequency is based on cell-ranking as follows:

If rangeToBestCell is not configured, and if the highest ranked cell is a NES Cell, the UE requests SIB1 of the NES cell. If rangeToBestCell is configured, and if the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell is the NES Cell, the UE request SIB1 of the NES cell. If there are multiple such NES cells, the UE requests SIB1 of the highest ranked NES cell among them.

The cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as follows:

Proposal 5	For intra-frequency or equal priority inter-frequency OD-SIB1 cell the UE requests SIB1, if following condition is met:

- If rangeToBestCell is not configured, and if the highest ranked cell is an OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell.

- If rangeToBestCell is configured, and if the cell with the highest number of beams above the threshold among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell is a OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell. If there are multiple such OD-SIB1 cells, the UE requests SIB1 of the highest ranked OD-SIB1 cell among them.

If the SIB1 request condition is met, the UE transmits MSG1 to the neighbor OD-SIB1 cell to request SIB1 transmission. According to the RAN2 agreements, the UE monitors MSG2 from the neighbor OD-SIB1 cell after transmitting MSG1. Until now, there has been no cases where UEs in RRC_IDLE/INACTIVE perform the random access procedure on the neighbor cell. UE in RRC_IDLE/INACTIVE should monitor periodically the serving cell for paging reception. If UE transmits MSG1 to a neighbor OD-SIB1 cell or monitors RAR window during the paging occasion, the UE could miss paging. Receiving paging is much more important than acquiring SIB1 of neighbor cell. The MSG1 transmission and MSG2 reception for SIB1 request should not interfere with the paging reception from serving cell.

Observation 2	The paging reception from the serving cell should be prioritized over the WUS transmission and the response reception on the neighbor OD-SIB1 cell.

Therefore, UE should select a PRACH occasion which does not overlap with the paging occasion, and the RAR window associated with the selected PRACH occasion also should not overlap with the paging occasion.

Proposal 6	UE takes into account the possible overlapping between the paging occasion and PRACH occasion/RAR window when determining the PRACH occasions of the neighbor OD-SIB1 cell for WUS transmission.

If UE is unable to acquire SIB1 from a cell, the cell is considered as barred. The conditions under which the UE determines that SIB1 reception is not possible are not specified. As it is up to UE implementation, there is no limit on the number of attempts or time restrictions for SIB1 reception. That is, if UE fails to receive SIB1 once, it may either retry to acquire SIB1 or immediately consider the cell as barred.

Observation 3	The conditions for declaring SIB1 acquisition failure is up to UE implementation in the existing cell reselection procedure.

Although the SIB1 is transmitted in on-demand manner, the same UE behaviours should be allowed. The UE may determine that SIB1 reception is not possible after a single RA failure, or it may initiate second random access to acquire SIB1. Therefore, we propose to leave the UE behaviours upon random access failure for SIB1 request up to UE implementation.

Proposal 7	When random access for WUS transmission fails, it is up to UE implementation whether to transmit WUS again or immediately consider the OD-SIB1 cell as barred.

- OD-SIB1 of serving NES Cell

In the last meeting, RAN2 discussed whether the WUS configuration needs to be transmitted from the NES cell and concluded that it needs to be transmitted for UEs camping on the NES cell.

RAN2#127

Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

However, RAN2 has not discussed the condition for SIB1 request of the serving NES Cell, i.e. when UE needs to request SIB1 after camping on the NES Cell. According to TS 38.331, UE is required to re-acquire SIB1 of the serving cell in the following cases:

- Case 1. If UE receives a Short Message with systemInfoModification bit

- Case 2. If UE receives a Short Message with etwsAndCmasIndication bit

- Case 3. When UE wants to acquire a SIB for which it does not have a valid version

Regardless of the RRC state that the UE is in, if etwsAndCmasIndication bit or systemInfoModification bit is set in the received Short Message, the UE shall re-acquire SIB1 immediately or from the start of the next modification period. Since RAN2 agreed on-demand SIB1 does not affect UEs in RRC_CONNECTED, OD-SIB1 Cell should start SIB1 transmission upon transmission of Short Message with systemInfoModification or etwsAndCmasIndication. If not, UEs in RRC_CONNECTED could fail to acquire SIB1.

RAN2#127

RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

Observation 4	If the OD-SIB1 cell transmits SIB1 based on the UE’s request after sending a Short Message with systemInfoModification or etwsAndCmasIndication, UEs in RRC_CONNECTED might not be able to receive SIB1.

Therefore, after transmitting the Short Message with systemInfoModification or etwsAndCmasIndication, the OD-SIB1 cell should start transmitting SIB1, even if it has not received UL WUS from UEs. That is, UEs in RRC_IDLE/INACTIVE don’t need to transmit UL WUS when the SIB1 acquisition is triggered by reception of Short Message with systemInfoModification or etwsAndCmasIndication.

Proposal 8	The OD-SIB1 Cell should start SIB1 transmission upon transmission of Short Message with systemInfoModification or etwsAndCmasIndication, even if it has not received UL WUS from UEs. UE in RRC_IDLE/INACTIVE is not required to transmit WUS when the SIB1 acquisition is triggered by reception of Short Message with systemInfoModification or etwsAndCmasIndication.

When the UE wants to acquire a SIB for which it does not have a valid version, if it is in RRC_IDLE/INACTIVE or if it is in RRC_CONNECTED with an active BWP configured with common search space with the field searchSpaceOtherSystemInformation, it determines whether to perform SI request or acquire the SI message containing the required SIB without SI request, based on the si-BroadcastStatus associated with the required SIB. Since the change of si-BroadcastStatus does not result in system information change notifications in Short Message, even though the UE already have a valid version of SIB1, it should re-acquire SIB1 to check the up-to-date si-BroadcastStatus of the required SIB. However, unlike case 1 and 2, the network does not know when the UE will want to receive an SIB that it does not have, so it is impossible for UEs in RRC_CONNECTED to acquire SIB1 from serving OD-SIB1 cell in case 3.

Observation 5	UEs in RRC_CONNECTED could not check up-to-date si-BroadcastStatus by re-acquiring SIB1 from OD-SIB1 cell, when it wants to acquire a SIB for which it does not have a valid version.

When UE in RRC_IDLE/INACTIVE wants to acquire a SIB for which it does not have a valid version, it can transmit the WUS to request SIB1 for the serving cell. However, considering that the reason for introducing the si-BroadcastStatus was to prevent UE performing unnecessary random access procedure, it is undesirable to make UE perform random access to acquire the si-BroadcastStatus. If required SIB is not being broadcast, the UE camping on the OD-SIB1 cell will perform random access twice to acquire it, i.e. one for SIB1 request and another for SI request.

Observation 6	If required SIB is not being broadcast, UE camping on an OD-SIB1 cell will perform random access twice to acquire it, i.e. one for SIB1 request and another for SI request.

Therefore, if a UE is connected to or camps on the OD-SIB1 cell, the UE should be allowed to perform SI request without checking the si-BroadcastStatus of the required SIB, i.e. without re-acquiring SIB1.

Proposal 9	UE which camps on or is connected to the OD-SIB1 cell is allowed to perform SI request without checking the si-BroadcastStatus of the required SIB, i.e. without re-acquiring SIB1.

If the proposal 8 and 9 are agreed, for UEs camping on the OD-SIB1 cell there is no case where the WUS needs to be transmitted to the serving OD-SIB1 cell. Then, we don’t need to consider the SIB1 request for serving OD-SIB1 cell.

Proposal 10	Do not consider the scenario where UE requests SIB1 for serving OD-SIB1 cell.

According to definition of Cell A defined by RAN1, the Cell A provide SIB1 periodically. However, after camping on a OD-SIB1 cell, the OD-SIB1 cell can act as a Cell A for neighbor OD-SIB1 cells. If the UE can receive the WUS configuration for neighbor OD-SIB1 cell from the serving OD-SIB1 cell, then the subsequent UE behaviors of sending WUS and receiving SIB1 are the same as in Case2. Therefore, after camping on the OD-SIB1 cell, the UE should be able to acquire the WUS configuration for neighbor OD-SIB1 cells from the serving OD-SIB1 cell.

Proposal 11	UE can acquire the WUS configuration for neighbor OD-SIB1 cells from the serving OD-SIB1 Cell.

- Access restriction for legacy UEs

During the SI phase, following three options were discussed as the potential solution to handle legacy UEs:

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

UEs capable of NTN, ATG, or nes-CellDTX-DRX determines if a cell is barred based on the cellBarredNTN, cellBarredATG, or cellBarredNES in SIB1, respectively. That is, even though the cellBarred in the acquired MIB is set to barred, such UEs will try to acquire SIB1 from the OD-SIB1 cell. Therefore, option 1 should be ruled out from the potential solutions.

Observation 7	cellBarred in MIB cannot make UEs capable of NTN, ATG, or nes-CellDTX-DRX consider the OD-SIB1 cell as barred.

If ssb-SubcarrierOffset indicates that SIB1 is not scheduled in the cell, the UE assumes it cannot acquire SIB1 of the cell and considers the cell as barred regardless of the UE capability. However, UEs capable of OD-SIB1 should be able to receive SIB1 from that cell after SIB1 request, hence the CORESET#0 should be configured. If ssb-SubcarrierOffset is used to indicate no SIB1, kSSB of the NES Cell cannot be configured in MIB. Therefore, kSSB of the OD-SIB1 cell needs to be provided via WUS configuration of the OD-SIB1 cell.

Proposal 12	Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

Proposal 13	kSSB of the OD-SIB1 cell is provided in the WUS configuration.

The option 1 or 2 cannot prevent legacy UEs from acquiring MIB of the OD-SIB1 cell. If the OD-SIB1 cell is indicated as excluded cell for legacy UEs, the MIB acquisition from the OD-SIB1 cell can be avoided. However, to achieve it, a separate excluded cell list needs to be defined for UEs capable of OD-SIB1. There have been some instances where legacy UE receives MIB of a cell that it cannot camp on, e.g. NTN cell, ATG cell. We can see no reason why OD-SIB1 cell should be excluded specially from cell resection of legacy UEs, and option 3 seems like overkill. Therefore, we propose not to introduce any solution to prevent legacy UEs from acquiring MIB of the OD-SIB1 Cell.

Proposal 14	Do not introduce any solution to prevent legacy UEs from acquiring MIB of the OD-SIB1 Cell.

Conclusion

Regarding OD-SIB1 of neighbor NES cell (Case 2)

Observation 1	If the 1-second rule in the cell reselection criteria is applied to the triggering condition of UL WUS transmission, it will lead to unnecessary delays in the cell reselection to OD-SIB1 cell.

Proposal 1	For an OD-SIB1 cell of a higher priority UE requests SIB1, if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT, or

- If threshServingLowQ is not broadcast in system information, and if an OD-SIB1 cell of a higher priority frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT.

Proposal 2	For an OD-SIB1 cell of a lower priority, UE request SIB1 if following conditions are met:

- If threshServingLowQ is broadcast in system information, and if the serving cell fulfils Squal < ThreshServing, LowQ and an OD-SIB1 cell of a lower priority frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

- If threshServingLowQ is not broadcast in system information, and if the serving cell fulfils Srxlev < ThreshServing, LowP and an OD-SIB1 cell of a lower priority frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT.

Proposal 3	SIB1 request to a higher priority frequency shall take precedence over a lower priority frequency if multiple OD-SIB1 cells of different priorities fulfil the cell reselection criteria.

Proposal 4	If more than one OD-SIB1 cell meets the triggering condition of WUS transmission, the UE requests SIB1 of the highest ranked OD-SIB1 cell.

Proposal 5	For intra-frequency or equal priority inter-frequency OD-SIB1 cell the UE requests SIB1, if following condition is met:

- If rangeToBestCell is not configured, and if the highest ranked cell is an OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell.

- If rangeToBestCell is configured, and if the cell with the highest number of beams above the threshold among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell is a OD-SIB1 cell, the UE requests SIB1 of the OD-SIB1 cell. If there are multiple such OD-SIB1 cells, the UE requests SIB1 of the highest ranked OD-SIB1 cell among them.

Observation 2	The paging reception from the serving cell should be prioritized over the WUS transmission and the response reception on the neighbor OD-SIB1 cell.

Proposal 6	UE takes into account the possible overlapping between the paging occasion and PRACH occasion/RAR window when determining the PRACH occasions of the neighbor OD-SIB1 cell for WUS transmission.

Observation 3	The conditions for declaring SIB1 acquisition failure is up to UE implementation in the existing cell reselection procedure.

Proposal 7	When random access for WUS transmission fails, it is up to UE implementation whether to transmit WUS again or immediately consider the OD-SIB1 cell as barred.

OD-SIB1 of serving NES Cell

Observation 4	If the OD-SIB1 cell transmits SIB1 based on the UE’s request after sending a Short Message with systemInfoModification or etwsAndCmasIndication, UEs in RRC_CONNECTED might not be able to receive SIB1.

Proposal 8	The OD-SIB1 Cell should start SIB1 transmission upon transmission of Short Message with systemInfoModification or etwsAndCmasIndication, even if it has not received UL WUS from UEs. UE in RRC_IDLE/INACTIVE is not required to transmit WUS when the SIB1 acquisition is triggered by reception of Short Message with systemInfoModification or etwsAndCmasIndication.

Observation 5	UEs in RRC_CONNECTED could not check up-to-date si-BroadcastStatus by re-acquiring SIB1 from OD-SIB1 cell, when it wants to acquire a SIB for which it does not have a valid version.

Observation 6	If required SIB is not being broadcast, UE camping on an OD-SIB1 cell will perform random access twice to acquire it, i.e. one for SIB1 request and another for SI request.

Proposal 9	UE which camps on or is connected to the OD-SIB1 cell is allowed to perform SI request without checking the si-BroadcastStatus of the required SIB, i.e. without re-acquiring SIB1.

Proposal 10	Do not consider the scenario where UE requests SIB1 for serving OD-SIB1 cell.

Proposal 11	UE can acquire the WUS configuration for neighbor OD-SIB1 cells from the serving OD-SIB1 Cell.

Access restriction for legacy UEs

Observation 7	cellBarred in MIB cannot make UEs capable of NTN, ATG, or nes-CellDTX-DRX consider the OD-SIB1 cell as barred.

Proposal 12	Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

Proposal 13	kSSB of the OD-SIB1 cell is provided in the WUS configuration.

Proposal 14	Do not introduce any solution to prevent legacy UEs from acquiring MIB of the OD-SIB1 Cell.