3GPP TSG-RAN WG2 Meeting #127bis	R2-2408773

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.7.6

Source:	OPPO

Title:	Discussion on uplink congestion signalling

Document for:	Discussion, Decision

Introduction

The revised WID of Rel-19 XR approves a new objective as follows [1]:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

This contribution shares our opinions on this matter.

Discussion

Recently, there have been some discussions in RAN plenary meetings on RAN-aware rate control for UL. The method can adjust the UL XR application codec bit rate and encoding parameters dynamically to ensure a seamless XR user experience when congestion happens. The adjustment is based on monitoring the DL signalling from the network and adapting the application's behaviour accordingly.

RAN#105 approves a new Rel-19 XR objective related to this issue and has finalized the use of DL MAC CE to inform the congestion.

During the Rel-18 XR discussion, RAN2 confirms in the reply LS[2] that the NG-RAN is capable of estimating congestion for uplink.

- It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in downlink and uplink directions. - It is feasible for RAN to estimate the congestion information per-QoS flow and per-DRB in uplink without UE impacts.

Accordingly, when congestion occurs, the NG-RAN is capable of notifying the UE about the congestion situation, either for each DRB or QoS flow. The question is whether the congestion indication should be provided at the level of QoS flow or DRB.

It could be argued that providing more precise congestion indication would help the APP layer to more accurately adjust the frame rate of a specific QoS flow. However, from the perspective of congestion mitigation, a per-DRB indication is simple, sufficient and effective. In the current spec, data transmission via the Uu interface is typically handled per LCH/DRB. This means that if there is available data buffered for a single DRB, the UE always uses the allocated UL grant to transmit it, without distinguishing which data is associated with which QoS flow belonging to this DRB. If congestion occurs, reducing the data rate for the entire DRB would efficiently and promptly control/alleviate the congestion. Therefore, per-DRB MAC CE signalling from the network is preferred to enable uplink rate adaption for congestion.

Per-DRB MAC CE signalling from the gNB is used to enable uplink rate adaption.

If Proposal 1 is accepted, we prefer to use the existing Recommended Bit Rate MAC CE format for uplink rate adaption because

- The recommended bit rate reflects the congestion situation. The worse the congestion situation/status, the lower the recommended bit rate.

- Usually, one RLC/logical channel is associated with one DRB. The UE can easily identify which DRB is experiencing congestion by the indication of LCID of that logical channel.

- Avoid the effort of discussing/designing the format for the new MAC CE. However, it is still necessary to introduce the usage description and the new LCID for this new MAC CE in the MAC/Stage-2 spec accordingly.

RAN2 aims at using the format of the existing Recommended Bit Rate MAC CE for uplink rate adaption (but using a new LCID).

Conclusion

We have the following proposals:

Proposal 1	Per-DRB MAC CE signalling from the gNB is used to enable uplink rate adaption.

Proposal 2	RAN2 aims at using the format of the existing Recommended Bit Rate MAC CE for uplink rate adaption (but using a new LCID).

Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

[2] R2-2213226 Reply LS to SA2 on XR, vivo, Huawei